3GPP SA WG2 Meeting #170	S2-2506404

Gothenburg, Sweden, 25-29 August 2025

# Title:	[Draft] LS on support of multiple IMS voice calls over NB-IoT NTN

# Response to:	-

# Release:	Release 20

# Work Item:	FS_5GSAT_Ph4_ARC

Source:	SA2

To:	SA1

Cc:	-

Contact Person:

Name:	Haris Zisimopoulos

Tel. Number:

E-mail Address:	harisz AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

In IMS it is possible for the UE to support more than one IMS voice calls simultaneously using the communication HOLD service that enables a user to suspend media within a session, and resume that media at a later time. Another similar example is 3PTY CONFerence. These are defined based on service requirements in TS 22.173 and TS 22.228 and they are mandated to be supported in GSMA IR.92 and therefore all commercial IMS UEs. SA2 is currently studying support for IMS voice over NB-IoT NTN this requirement will have an effect on the overall overhead since if more than one voice media flows need to be support, UDP needs to have destination and source port numbers headers to distinguish them. Alternatively, if specifically for support of IMS voice over NB-IoT NTN only one IMS voice call can be supported related procedures need to be defined in IMS “network” and UE in order to supress an additional IMS voice call if the user is already active in one.

Since this is a service requirement, SA2 would like a clarification from SA1 whether it is required for the UE to support more than one IMS voice calls simultaneously using the communication HOLD service or 3PTY CONFerence when IMS client is operating over NB-IoT NTN.

2. Actions:

To SA1:

ACTION: 	SA2 asks SA1 whether it is required for the UE to support more than one IMS voice calls simultaneously using the communication HOLD service or 3PTY CONFerence when IMS client is operating over NB-IoT NTN.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #171		13-17 October 2025	Wuhan, PRC

TSG-SA2 Meeting #172		17-21 November 2025	Dallas, TX, USA