3GPP TSG-WG2 Meeting #170	S2-2506413

# Goteborg , SE, 25 Aug – 29 Aug 2025

Source:	Ericsson

Title:	KI#3 Evaluation and agreements

Document for:	Approval

Agenda item:	20.4.1

Spec:	3GPP TR 23.700-67

Version:	0.2.0

Work Item:	FS_EnergySys_Ph2

Introduction

All major solutions proposed for KI#3 have been folded into Solution #20. Hence the evaluation and proposed agreements for KI#3 focus on solution #20

Solution #20: Energy aware NF and path selection in 3GPP TR 23.700-67 V0.2.0 (2025-05) proposes:

This solution proposes the following principles to be agreed:

1.	The NF profile is augmented with Energy Priority Information.

2.	The NF profile is augmented with a Schedule of Energy Priority Information to enable providing a schedule of Energy Priority information, if one exists, instead of requiring additional updates at a time when the Energy Priority information is known to change.

3.	The NF profile is augmented with Schedule of EnergySaving/not EnergySaving State (See TS 28.310 [9]) and the EnergySaving/Not EnergySaving state attributes to enable the NF service consumer and NRF to know when the NF may be taken out of service according to a schedule or to enable the NRF to know a NF is temporarily out of service to save energy.

4.	The UPFs in the UP path are selected by the SMF taking into account the Attributes above in the NF profile, if any are present, based on a UP Path Selection policy.

Evaluation:

1.	Usage of Energy Priority Information
This parameter should be mostly affected by availability of renewable energy source, enabling the selection of the NF with the most usage of renewable energy, increasing network energy efficiency. Energy Priority information may be integrated and influence the value of the priority parameter in NF profile.

Considering the benefits that Energy Priority Information can bring related to energy efficiency, Energy Priority Information may be considered for the normative phase.

2.	Usage of Schedule Energy Priority Information
Even though renewable energy source availability has some patterns; it is hard to have a full prediction. Considering that all NFs maintain a heartbeat with NRF (which have a periodicity varying from several seconds to several minutes) that can be used to send Energy Priority information in real time. Since the availability of renewable energies has a variation of several minutes, even a less frequent heartbeat would timely inform about any variation of renewable energy availability.
Therefore, it does not make sense to limit flexibility with a strict schedule.

Considering this, the usage of scheduling for Energy Priority Information should not be considered for normative phase.

3.	Usage of Schedule EnergySaving/not EnergySaving

The NF producer keeps sending a heartbeat with a frequency that changes with configurations but can vary from a few seconds to several minutes, the time is defined by the network operator. This heartbeat informs NRF that NF producer is working, additionally these heartbeats also inform about NF status and load information when load reaches certain criteria.
If an NF did not leave EnergySaving state when it should be available according to the schedule. Who will update the schedule?
If the NF fails when schedule says it is available? NRF needs to detect the failure using mechanisms like the heartbeat, and then, NRF needs to update the schedules in all affected NFs.
After an NF comes back from offline how does it discover its own schedule? Does it make a new schedule which then is propagated to all NFs affected?

The use of scheduling would not provide any major benefits and would limit the flexibility of NFs to change their status. Additionally, even with the implementation of scheduling, it would still be necessary to rely on existing mechanisms, such as heartbeat, to ensure that the NF is working properly when it is in “not EnergySaving” state. Therefore, existing mechanisms need to be maintained.
In summary, implementing scheduling would increase complexity, introduce several issues, and limit flexibility without significantly reducing energy consumption; it might even increase energy consumption.

Therefore, schedule of EnergySaving/not EnergySaving state should not be considered for normative phase.

4.	Selection of UPFs based on Energy Priority Information
UPF is a fundamental component of communication, and since Energy Priority Information enables the selection of the most energy efficient UPF.

The selection of UPF based on Energy Priority Information may be considered for the normative phase.

Proposed Changes

* * * First Change * * * *

### 7.1.3	Agreed Principles for KI#3

-	The following aspects from Solution#20 may be considered for the normative phase according to discussion in S2-2506413:

-	Energy Priority information.

-	Selection of UPF based on Energy Priority Information.

-	The following aspects from Solution#20 should not be considered for the normative phase according to discussion in S2-2506413:

-	Scheduling of EnergySaving/not EnergySaving state.

-	Scheduling of Energy Priority Information.

* * * End of Changes * * * *