SA WG2 Meeting #170	S2-2506420

25 – 29 Aug, 2025, Goteborg , SE

Source:	InterDigital Inc.

Title:	[WT#5, Study data framework for all aspects related to efficient and scalable data handling] New Data Framework

Document for:	Approval

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This p-CR proposes to extent the scope of WT#5 to enable concreate details of what needs to be study, beyond a shell definition.

# 1. Justifications

There are so many solutions today to transfer data, making it difficult for MNO to coordinate and manage Data Collection mechanism in their networks.

The following are examples of the plethora of options the 5GS offers:

- Location LTE Positioning Protocol (LPP) is used between the UE to the LMF to provide the LMF with UE positioning measurements. Location Services transfer data over both CP and UP. For user plane transfer, URSP rules are used to establish a PDU Session specifically set up for the transfer of positioning information with a DNN/S-NSSAI combination. The UE requests connection information from the LMF via NAS.

- Performance Measurement Function Protocol (PMFP) is used between the UE and the UPF to enable the UE to measure Round Trip Time, exchange packet loss information and send information about Access Network Availability to support ATSSS functionality.  To enable use of the protocol, a UP connection is set up between the PMF logic in the UE and the PMF function in the UPF using the PDU Session Establishment Accept message

- QoE measurements may be requested by Management Subsystem (MnS) consumer (at the MNO OAM system) by configuring QoE Data Collection in the UDM. When QoE data collection configuration is set up or updated, the UDM informs the AMF, and the AMF triggers the activation of QoE measurements through the gNB. When the measurement collection is complete, the UE sends a measurement report to the gNB, which in turn sends it to a QoE Measurement Collection entity. To handle QMC (QoE Measurements Collection), QoE is configured from OAM via gNB and AT Commands and then the QoE info is sent from the RAN to a collection point (Measurement Collection Entity).  In addition, MDT (Minimization of Drive Test) measurement collection works in a similar way.

- For AIML, new data collection solutions have been proposed as part of the current CN AIML Ph2 Rel.20 study, including an updated EVEX proposal and multiple other solution that relay on a central data collection control entity. In addition, many of these solutions address User Consent handling to certain degree.

When looking at the large number data transfer variants (e.g., which have mostly been developed independently) one could see potential benefits that unified Data Framework may bring.

It is therefore important that the MNO may have access to single mechanism able to support all data transfer, configuration, storage and distributions needs, and yet while providing the MNO with full visibility, controllability and the possibility to protect sensitive data traversing their networks, while ensuring compliance with privacy and regulatory requirements.

Architectural Motivation:

To be able to tackle the multiplicity of mechanisms handling the transfer of data across the MNO network, a data framework that can handle data transfer, in a coordinated, scalable and efficient way is required. There are several Use Cases related to data transfer, beyond those already described (i.e., PMF, QMC and LPP) that can benefit from a unified data collection framework fully defined in Mobile Network Operator domain, to transfer data such as measurement reports, e.g., used by AIML and Sensing applications, or computing task execution data over a data session capable of transferring data intended for multiple and distinct services.

Unlike the current mechanisms that use both CP and UP to handle the transfer of data, a unified framework should consider a single data transfer mechanism, that includes handling of data collection, data distribution, data processing and data storage in unified manner, where data to satisfy the needs of multiple services can be delivered using such mechanism, as opposed to the disjointed collection of solutions present in the current system.

Gap Analysis:

The disjointed silo-like data transfer options in the current system make it difficult for an MNO to manage an efficient data transfer mechanism, as these mechanisms rely on multiple transfer methods and they are configured and triggered by different network entities.

A 6G unified data framework is expected to provide architecture enhancements and mechanisms to enable handling of all required data transfer needs via a single unified controlling entity, that enables full visibility and controllability. A unified controlling entity does not necessarily mean centralized vs distributed, but rather that a controlling entity might address requests from service providers that have multiple data transfer needs.

Technical Impact:

Architectural impacts and new unified procedures are expected to enable the coordination, configuration and handling of data transfer, including processing, distribution and storage to satisfy the needs of all services that require transfer of data across the MNO network. Some of these impacts may include:

- Procedures to configure data transfer, including authorization and coordination of involved network entities

- Procedures to handle initiation, maintenance and termination of data transfer

- Procedures to handle data distribution, and data storage

- System level mechanism to provide secure data transfer and distribution.

**** First Change ****

# Annex A.X. WT#5 Study data framework for all aspects related to efficient and scalable data handling

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

The scope of this work task includes the following:

- Whether and how to specify a unified data collection and reporting framework to enable efficient and secure data collocation in a way that fulfils all of the data collection requirements of the 6G system.

- How to enable MNO controllability, and MNO visibility on standardized data collection process, including data collected at the AN, or any other network entity.

- How to enable service-differentiated data transfer, processing and storage mechanisms within and beyond the MNO domain, e.g., also serving request for data transfer from third party AFs.

- Whether and how system level security mechanisms need to be in placed to enable MNO-controlled data access and exposure.

NOTE:	Details of the required security mechanism are under the remit of SA3.

**** Second Change ****

# 5.X. Key Issue #X: Key Issue Title

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

**** End of Changes ****