SA WG2 Meeting #170	S2-2506421

25 – 29 Aug, 2025, Goteborg, SE

Source:	OPPO

Title:	[WT#2] Migration and Interworking

Document for:	Agreement

Agenda Item:	20.6.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes WT update and Key Issue description for WT#2 in TR 23.801 for SA2 6G Study.

1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

Motivation：

In order to serve the legacy UEs, EPS and 5GS have to exist in market for another decades, as a consequence, in some areas of an operator’s deployment, there may be no 6GS coverage for a couple of years.

In the early stage of 6G RAN deployment, limited 6G radio coverage may result in service interruption and degraded user experience. In order to ensure the session/service continuity and improve network resource utilization, the 5GS/EPS communications infrastructure can be utilized to provide legacy or roaming UE support.

As operators deploy 6G RAN alongside the legacy 3GPP RAN (i.e., E-UTRAN, NG-RAN), ensuring service continuity during UE mobility between heterogeneous RANs is critical. Without interworking between 6GS and legacy systems:

- Service disruptions degrade user experience (e.g., dropped calls during inter-RAT handovers).

- Network resources are not fully utilized.

Besides, different services demand tailored handling:

- Voice and other real-time services require near-zero interruption (e.g., <50 ms).

- uRLLC applications, e.g., industrial automation, demand ultra-high reliability during inter-RAT mobility.

- mMTC services tolerate brief interruptions but need session persistence.

In addition, a 6G-eabled UE will get at least EPC/5GC services where E-UTRAN/NG-RAN exists, if the UE supports backward compatibility with legacy networks. On top of that, the 6G-eabled UE can also be able to access any NG RAN in the VPLMN that the roaming agreement allows. The roaming scenarios can be categorized into the following Cases:

- EPC/5GC in VPLMN, CN to support 6G-RAN in HPLMN,

- CN to support 6G-RAN in VPLMN, EPC/5GC in HPLMN.

When 6G-enabled UEs roam to EGS/5PS-only networks, interworking between 6GS and EPS can avoid “islands” of 6G services that reduce user experience.

Technical Impact：

In order to support the interworking between 6GS and 5GS/EPS, the RM\CM\MM\SM related functionalities and procedures are required to be enhanced.

**** First Change ****

Annex A.X. WT#X Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#2: Study the architecture and system impact to support migration and interworking for both roaming and non-roaming scenarios, including

-	How to support migration to 6GS

-	How to support interworking with 5GS

-	Whether and how to support interworking with EPS

How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 2:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 3:  The detailed migration study scope will be coordinated and aligned with RAN.

**** Second Change ****

5.X. Key Issue #X:  Migration and Interworking

In order to serve the legacy UEs, EPS and 5GS have to exist in market for another decades, as a consequence, in some areas of an operator’s deployment, there may be no 6GS coverage for a couple of years. As such, the interworking between 6GS and 5GS/EPS is deemed necessary.

To support interworking between 6GS and 5GS/EPS, following aspects need to be investigated:

- The scenarios for interworking/migration between 6GS and 5GS, and between 6GS and EPS.

- The interworking or migration architecture for both non-roaming and roaming scenarios.

- The system impacts and the corresponding procedures (e.g., Mobility Management, Session Management, Connection Management, etc.), including how to guarantee the Session Continuity and/or Service Continuity.

**** End of Changes ****