SA WG2 Meeting #170	S2-2506422

25 – 29 Aug, 2025, Goteborg, SE

Source:	OPPO

Title:	[WT#3, AI for Network] AI capable UE and network interaction

Document for:	Agreement

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes the updates to WT and new KI on the support of AI capable UE and network interaction in 6G system.

1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

Backgrounds & Motivations:

An AI agent is an intelligent system that autonomously perceives, reasons, and acts within a given environment to achieve specific goals. It makes context-aware decisions using Large Language Models (LLMs) based on other assist information. The agent then executes actions such as generating responses, controlling devices, or performing tasks ranging from simple automation (e.g., calling) to complex problem-solving (e.g., autonomous driving). Advanced AI agents utilize memory (e.g., vector databases) to learn from past interactions, adapt to dynamic conditions, and collaborate with other agents in multi-agent systems.

The rapid development of AI technology has led to the commercial deployment of AI-capable device (e.g., AI agent). These devices leverage LLMs for natural language understanding and generation and support the following cases:

- Supports voice interaction, enabling operations through voice commands

- Accesses device sensors (GPS, accelerometer) to sense user environment

- Handles basic tasks: setting reminders, checking weather, playing music, etc.

- Provides personalized services: recommends content based on user habits (e.g., schedule management, shopping suggestions)

However, the services offered by AI capable device still significantly limited. The AI capable device can invoke 3rd party application APIs based on the user requirements, but they cannot access 3GPP related services (e.g., connection service, location services, data service, AI service, sensing service, etc.). This restricts advanced use cases and impedes user satisfaction.

For example, when a user sends the request to the AI capable device “I want to play a game from xx time to xx time”. The AI capable device can know which application the user wants to interact with, but the AI capable device cannot leverage the operator connection service to improve the QoS performance in order to let user has a good gaming experience. Another example is when a user requests "Driving a car to arrive at the office on time," the AI capable device can invoke a navigation app API but cannot leverage operator sensing services to dynamically generate 3D maps for precise lane-level navigation.

To resolve this, intent-based UE and network interaction is essential for 6G’s evolution. By enabling UEs to express high-level intents instead of technical requests, this paradigm:

- Enables AI capable UE to securely access 3GPP services (e.g., location, sensing, etc.) via policy-based controls without requiring knowledge of operator-specific service implementations.

- Reduces signaling overhead by replaces fragmented API invoke or control plane signaling with single intent declaration.

- Enables fast deployment and commercialization of new services.

Figure 1 The architecture for the AI capable device interact with the 6GC

To enable AI capable devices to leverage 3GPP services for user requirements, Figure 1 illustrates the interaction architecture with the 6GC. The AI agent application on the UE first receives natural language user inputs. It then determines whether fulfilling the request requires non-3GPP services or 3GPP services. For 3GPP-related requirements, the application forwards them to the 3GPP Intent Client. This client transmits a structured 3GPP intent to an AI Capable NF in the 6GC. The NF interprets the intent using LLMs and orchestrates the necessary 3GPP services (e.g., connectivity, location) to fulfill the user’s request.

This architecture abstracts low-level service invocation from the UE: the AI-capable device only needs to express what it needs (via intents), not how to achieve it. Consequently, UE complexity and UE-6GC signaling overhead are significantly reduced.

Observation: Standardization of an AI-native framework, interfaces, and protocols is essential to enable the intent-based architecture (Fig. 1). By abstracting low-level service invocation into structured intents processed by 6GC AI functions, UE complexity and signalling overhead are drastically reduced. This ensures reliable, efficient AI capable UE access to 3GPP services (e.g., location/connectivity) while maintaining network control.

Proposal: Standardization of new interaction, including an AI-native framework, dedicated interfaces, and protocols, is required to enable efficient and reliable communication between AI capable UEs and 6GC.

**** First Change ****

Annex A.X. WT#3 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#3: Study how to support and enable use of AI in 6G (e.g. AI agent, framework) including how to support the interaction between the AI capable UE and the 6GC.

NOTE 4: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

**** Second Change (all new text) ****

5.X Key Issue #X: Support AI capable UE interaction with the 6GC

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

In order to support the interaction between an AI capable UE and the 6GC to provide intent interaction friendly user experience, the following aspects need to be studied:

- Overall architecture to support the efficient and reliable interaction between the AI capable UE and the 6GC.

- How to discover the AI capable UE and AI capable NF with the different capabilities under the control of the 6G core network.

- How and what configuration parameters are configured between the AI capable UE and the 6GC.

- Whether and how the AI capable UE interacts with the 6GC over structured 3GPP intent.

**** End of Changes ****