SA WG2 Meeting #170			S2-2506428

25 - 29 August 2025, Goteborg, Sweden

Source:	ETRI

Title:	KI#2 Solution#29 Updates

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: This solution defines a practical methodology for KI#2, enabling AI/ML-assisted user plane traffic pattern and behavior analysis to support dynamic and efficient performance control in the user plane.

1. Introduction/Discussion

In clause 5.3 and 5.3.1 of TR 23.700-04 v0.1.0, the key issue (KI#2: AI/ML-assisted user plane traffic pattern and behaviour analysis to support efficient performance of the User Plane) identifies the need to study:

-	How to provide analytics (e.g. enhancing existing in TS 23.288 [5] or introducing new) including the aspects of required inputs/outputs to improve performance of user plan.

-	Which consumers can subscribe/request the analytics and how the consumers use the analytics to take actions to improve user plane performance.

-	Whether and how to train ML models and perform the inference for the user plane analytics, considering solutions that minimize the load on Control Plane NFs.

This contribution proposes a solution for FS_AIML_CN_Ph2 addressing KI#2.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-04

# * * * * Start of changes * * * *

6.29	Solution #29: NWDAF-assisted Analytics for Efficient User Plane Performance

6.29.1	High level principles

The high-level principles of the solution are:

- Bursty or irregular traffic (e.g., IoT, live video, encrypted flows) not handled well by static QER rules can be analysed or predicted for policy adjustment.

- DLDR (Downlink Data Report, as defined in TS 29.244) or BAR settings can be updated based on observed traffic patterns.

- For encrypted traffic with limited PDI classification, use observable metadata (e.g., packet size, IAT, PPS/BPS) for QoS match evaluation, anomaly detection, and efficient UPF resource use.

- UPF exports traffic indicators to NWDAF for AI/ML or statistical analysis.

- Analysis results help identify flows that may affect UPF performance and guide SMF/PCF adjustments in line with operator policy.

6.29.2 Description

This solution supports user plane traffic pattern analysis to support UPF performance optimization and anomaly detection, including for encrypted packet flows.

The NWDAF collects traffic indicators from the UPF, SMF, and OAM via DCCF, optionally mediated by MFAF. The input may include flow-level indicators such as DL/UL packet information (e.g., 5-tuple, packet size distribution, inter-arrival time (IAT), packets per second (PPS), and bits per second (BPS), flow duration), drop cause, PDR information, and observable protocol-specific handshake characteristics (e.g., initial packet length for QUIC).

The NWDAF processes the collected data using AI/ML models or statistical algorithms and generates output analytics structured per time slot. Each time slot may include per-UPF analytics results for multiple UEs and application flows. These results contain flow stability indicators, protocol-level insights, predicted idle behaviour, QoS match level, abnormal packet indicators, and associated confidence levels. For encrypted flows, anomaly detection is performed using observable header metadata and statistical indicators, without payload inspection.

The analytics output is structured as shown in Table 6.x.1.2-1 and can be used by consumer NFs (PCF, SMF) as follows:

- Re-evaluate or adjust QER, 5QI assignment, shaping profiles, or DLDR/BAR configurations via PCC rules based on traffic analysis or anomaly flags.

- In cases of burst prediction, idle prediction, or QoS profile mismatches (e.g., 5QI misalignment), modify FAR, re-evaluate forwarding behaviour, or update UP path configurations.

NWDAF does not directly control policy or forwarding logic. The consumer NFs interpret and apply the analytics in accordance with operator policies.

NOTE1: The signalling load from UPF metric reporting is driven mainly by operator policy and system implementation.

6.29.1.1	Input Data

To support AI/ML-based traffic analysis, NWDAF can collect data from network functions (NFs) and OAM.

Table 6.29.1.1-1: NWDAF Input Data for UP Traffic Pattern Analysis

| Information | Source | Description |
| --- | --- | --- |
| UE IP Address | SMF, UPF | IP address assigned to the UE, used to associate traffic flows. |
| SUPI (within PDI) | SMF, UPF | Subscription Permanent Identifier in the Packet Detection Information, uniquely identifying the UE. |
| DL TEID | UPF | Downlink Tunnel Endpoint Identifier used in GTP-U for packet forwarding. |
| Packet Information (DL/UL) | UPF, SMF | Flow-level packet header and statistical information collected from UPF for both downlink and uplink traffic, used to support flow classification, QoS alignment, and traffic pattern analysis. |
| > 5-tuple | UPF, SMF | Source/destination IP address, source/destination port, and protocol identifier of the flow. |
| > Relevant header fields | UPF, SMF | Selected L3/L4 header fields observable without decryption, e.g., DSCP, protocol type. |
| > Observable encrypted-flow header characteristics | UPF, SMF | Non-payload header parameters from encrypted flows, such as connection/session ID length, protocol version, and initial packet length; supports anomaly detection for encrypted flows without payload inspection. |
| > Packet size | UPF, SMF | Size of each packet in bytes. |
| > Inter-arrival time (IAT) | UPF, SMF | Time between consecutive packets in the same flow, in milliseconds. |
| > Packets per second (PPS) | UPF, SMF | Average packet rate over the observation interval. |
| > Bits per second (BPS) | UPF, SMF | Average bit rate over the observation interval. |
| > Duration | UPF, SMF | Total observed flow duration, in seconds. |
| > N-second PPS/BPS average | UPF, SMF | PPS and BPS averaged over an N-second sliding window. |
| Destination network info (ASN/GeoIP) | UPF, SMF | Optional operator-specific observable metadata (ASN, GeoIP) derived from UPF 5-tuple; may support classification, QoS match evaluation, or anomaly detection; no payload inspection. |
| Packet Count (UL/DL, N3/N6) | UPF, OAM | Number of packets observed on N3 and N6 interfaces, for uplink and downlink. |
| QoS Flow Info (5QI, QFI, ARP) | SMF, UPF | Parameters used to define QoS characteristics of each flow, used for QoS match evaluation in analytics. |
| QER Info | UPF | QoS Enforcement Rule settings such as guaranteed bit rate, shaping behavior, discard handling, and buffer use, used for QoS match evaluation in analytics. |
| BAR / FAR | UPF, SMF | PDR used to forward or buffer packets as per session context. |
| DL Usage Threshold Crossing | SMF, UPF | Triggered when downlink usage exceeds a threshold; may help detect bursty flows. |
| Flow End | SMF, UPF | Marks the termination of a flow; used for resource release or analytics. |
| Congestion Notification | SMF, UPF | Indicates congestion in the UPF or RAN. May be inferred from queue depth or packet drops. |
| UL/DL Volume, Duration | UPF, OAM | Total traffic volume and session duration per direction. |
| UE CM/Reachability Information | AMF | Indicates the UE connection management (CM) state and UE reachability for data as provided by the AMF via event exposure. Used to support evaluation of DLDR and BAR handling conditions. |
| Packet drop cause | UPF, SMF | Indicates why packets were dropped (e.g., invalid header, unknown dest) |
| UPF load | UPF, SMF, OAM, NRF | CPU usage, buffer depth, or throughput metrics indicating load |
| Malformed packet count | OAM, UPF | Number of packets dropped or flagged due to protocol violations (e.g., invalid headers, checksum errors). |

NOTE2: Encrypted traffic (e.g., QUIC, TLS) is classified based on observable metadata such as 5QI, port, packet size, and timing—not by inspecting payload.

6.29.1.2 Output Analytics

Table 6.29.1.2-1: Output Analytics of UP Traffic Pattern (statistics and prediction)

| Information | Description |
| --- | --- |
| Time slot entry (1..max) | List of time slots during the Analytics evaluation period |
| > Time slot start | Start of the slot |
| > Duration | Duration of the slot |
| > Per-UPF analytics (1..max) | Analytics generated per UPF per time slot |
| >> UPF ID | Identified UPF (NF Instance ID or IP) |
| >> Application info | Application ID, FQDN (e.g., video_server.example.com), or IP |
| >> UE ID | SUPI/GPSI associated with flow |
| >> Transport protocol | Transport-layer protocol observed in the flow, e.g., TCP, UDP, QUIC. |
| >> Abnormal packet indicator | Indicates whether unclassified, malformed, or suspicious packet flows were detected during the slot, including for encrypted flows using observable metadata (e.g., packet size, IAT, PPS/BPS). Values: Yes or No |
| >> Traffic burst index | Categorized burstiness level of the traffic pattern: High / Medium / Low. |
| >> Flow stability score | Indicates how consistent the traffic is over time: Stable / Medium / Unstable. |
| >> Idle state prediction | Likelihood that the session may enter an idle state, useful for DLDR tuning: High / Medium / Low. |
| >> QoS suitability indicator | Indicates whether the currently assigned QoS parameters (e.g., 5QI, QFI, QER, ARP) appear suitable for the flow (i.e., QoS match evaluation), based on observable metrics such as packet size, inter-arrival time, and burst level. This estimation does not rely on PDI/DPI and is applicable to encrypted flows. Values: Suitable / Uncertain / Unsuitable. |
| >> Policy-related field | Relevant QoS control field that may be reviewed by the consumer NF (e.g., QER, DLDR, BAR, FAR). |
| >> Timestamp of observation | The time at which the traffic observation or prediction applies |
| >> Confidence | Confidence level (if prediction applied) |

Table 6.29.1.2-2: Example Use of NWDAF Analytics by Consumer NFs

| Consumer NF | Typical Use of NWDAF Analytics |
| --- | --- |
| SMF | May use traffic behaviour analytics (e.g., burstiness, idle prediction, abnormal flow indicators) to optimize UPF handling, such as adjusting DLDR timers or BAR settings, updating FAR configurations, or triggering path reassessment based on operator policy. |
| PCF | May interpret traffic indicators to revise PCC rules (including QER parameters) and provide updated policy decisions to SMF, enabling shaping adjustments or relative flow prioritization when traffic does not align with QoS expectations. Policy-driven mitigation may be applied for abnormal or unclassified traffic. |

6.29.2 Procedures

Figure 6.29.2.1-1: Data Collection and Analytics for UP Traffic Pattern using NWDAF

- . Consumer NF (e.g., SMF or PCF) sends a Nnwdaf_AnalyticsSubscription_Subscribe request to the NWDAF to subscribe to UP traffic pattern analytics (e.g., burst level, flow stability, idle prediction).

- Upon receiving the subscription, the NWDAF determines the required data and subscribes to DCCF using Ndccf_DataManagement_Subscribe. This message instructs the DCCF to coordinate data collection from appropriate source NFs.

- The DCCF then sends subscription requests to the relevant data-producing NFs such as UPF, SMF, and OAM using either: Nupf_EventExposure_Subscribe, Nsmf_EventExposure_Subscribe depending on the nature of the data (e.g., flow stats, session info, KPIs).

- (Optional) If direct SBI subscription is not feasible (e.g., due to protocol mismatch or legacy systems), the DCCF may utilize the Messaging Framework Adaptor Function (MFAF) to intermediate data subscription using Nmfaf_3daDataManagement_Subscribe.

- The subscribed source NFs, OAM begin sending their reports to the DCCF using: Nupf_EventExposure_Notify, Nsmf_EventExposure_Notify.

- (Optional) Alternatively, if MFAF was used, it forwards the received data to DCCF via Nmfaf_3daDataManagement_Notify.

- DCCF aggregates or filters the collected data as needed and sends it to NWDAF via Ndccf_DataManagement_Notify.

- NWDAF processes the input data, possibly invoking AI/ML models, and derives per-UPF, time-slot-based traffic pattern analytics (e.g., traffic burst index, idle probability, QoS suitability indicator), including anomaly detection for encrypted flows using observable metadata and statistical indicators (e.g., packet size distribution, inter-packet arrival time, PPS/BPS, initial packet length).

- The analytics output can be optionally stored in the ADRF (Analytics Data Repository Function) via Nadrf_AnalyticsDataManagement_Store for reuse or historical analysis.

- Finally, NWDAF delivers the structured analytics output to the subscribing Consumer NF.

NOTE3: OAM data is obtained via DCCF using implementation-specific interfaces, as defined in TS 28.552. No standard SBI exists for direct NWDAF–OAM interaction; an intermediary function such as MFAF may be used if needed.

6.29.3	Impact on services, entities and interfaces

NWDAF

- Collects user plane traffic-related indicators from UPF, SMF, and OAM.

- Analyzes user plane traffic pattern using AI/ML models or statistical methods.

- Generates analytics per UPF, UE, and application flow, structured per time slot. Analytics may include: burstiness, flow stability, idle state prediction, QoS suitability, and abnormal/unknown traffic indicators, including for encrypted flows using observable metadata (e.g., packet size, IAT, PPS/BPS).

- Exposes analytics to Consumer NFs via Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo service operations.

Consumer NFs (e.g., SMF, PCF)

- SMF: Subscribes to or requests traffic pattern analytics from NWDAF, interprets the results, and determines adjustments to DLDR, BAR, FAR, or QER parameters. Applies updated configurations to UPF via PFCP, in accordance with operator policy.

- PCF: Subscribes to or requests traffic pattern analytics from NWDAF, interprets the results to revise PCC rules (including QER-related policy), and provides updated policy decisions to SMF via Npcf_SMPolicyControl. May initiate policy-driven mitigation for abnormal or unclassified traffic.

UPF

- Acts as both a data source (exporting indicators such as burst count, drop cause, abnormal packet statistics) and a policy enforcement point.

- Implements updated PDR, FAR, QER and BAR configurations received from SMF via PFCP.

# * * * * End of changes * * * *