SA WG2 Meeting #170	S2-2506433

Stor-Göteborg, Sweden, August 25 – 29th, 2025

Source:	Nokia

# Title:	Architectural Assumptions for FS_SMS2EC_ARC

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC _ARC / Rel-20

Abstract of the contribution: This contribution proposes architectural assumptions for FS_SMS2EC_ARC.

1. Introduction

It is proposed to include the architectural assumptions for FS_SMS2EC_ARC.

2. Proposal

It is proposed to agree on the following changes to 3GPP TR 23.700-65.

# * * * * First change * * * *

# 4	Architectural and Service Assumptions

### 4.1	Architectural Assumptions

The following architecture assumptions are applied to the study:

- The solutions must support Short Message Service to the Emergency Response Centre in both 5GS and EPS.

- Architecture option for both EPS and 5GS, 3GPP access (including TN and NTN) and non-3GPP access is considered in this study.

- UE in the limited service state and unauthenticated UE will not be in the scope of this study.

- Both short message over NAS and short message over IP are considered for this study.

- Short Messages can be routed to different Emergency Response Centers, depending on the type of emergency service,.

- The architecture assumes that Short Messages initiated from the Emergency Response Center to the UE in response to an Emergency Short Message are treated as regular Short Messages.

- The architectural principles for SMS over NAS (see TS 23.272, TS 23.401, TS 23.501, TS 23.502) shall be used as a basis for this study.

- The architectural principles for SMS over IP (see TS 23.204) and the architectural principles of Emergency Services (see TS 23.167) shall be used as a basis for this study.

# * * * * End of changes * * *