SA WG2 Meeting #170	S2-2506434

Stor-Göteborg, Sweden, August 25 – 29th, 2025

Source:	Nokia

# Title:	Key Issues for Short Message Service to Emergency Response Centre

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This contribution proposes new key issues for FS_SMS2EC_ARC.

# 1. Discussion

There are three WTs captured in the SID objective of FS_SMS2EC_ARC to support Short Message Service to Emergency Response Centre in both EPS and 5GS, based on the requirements specified in TS 22.101:

-	WT-1: How to identify a Short Message to Emergency Response Center based on the use of emergency numbers. Address aspects when serving PLMN does not support this feature, but UE supports it.

-	WT-2: Whether and how to identify the type of emergency service (police, ambulance, fire brigade, etc.) using the same identification mechanism as for emergency calls.

-	WT-3: How to route a Short Message to Emergency Response Center (i.e. PSAP) serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location).

This PCR contribution proposes new key Issues corresponding to these three work tasks. As the study defines, the solutions should be based on the architectures for SMS over NAS (see TS 23.272, TS 23.401, TS 23.501, TS 23.502) and SMS over IP (see TS 23.204), and the architectural principles of Emergency Services (see TS 23.167) will be used as a basis. The following key issues are aimed to solve the WTs requirements based on these two architectural aspects.

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-65.

# * * * * First change * * * *

## 5.x	Key Issue #1: Support of Short Message Service to an Emergency Response Center via SMS over NAS

### 5.x.1	Description

This key issue aims to address the following aspects:

-	How to exchange capabilities between the UE and the network for support of Short Message to an Emergency Response Center via SMS over NAS.

-	How to support Short Message to an Emergency Response Center via SMS over NAS in UE detectable scenarios.

-	Whether and how to support Short Message to an Emergency Response Center over NAS in UE non-detectable scenarios.

-	How to identify the type of emergency service (police, ambulance, fire brigade, etc.) using the same identification mechanism as for emergency calls.

-	How to route a Short Message to the Emergency Response Center (i.e. PSAP) serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location).

# * * * * Second change * * * *

## 5.y	Key Issue #2: Support of Short Message Service to an Emergency Response Center via SMS over IP

### 5.y.2	Description

This key issue aims to address the following aspects:

-	How to exchange capabilities between the UE and the network for support of Short Message to an Emergency Response Center via SMS over IP.

-	How to support Short Message to an Emergency Response Center via SMS over IP in UE detectable scenarios.

-	Whether and how to support Short Message to an Emergency Response Center over NAS in UE non-detectable scenarios.

-	How to identify the type of emergency service (police, ambulance, fire brigade, etc.) using the same identification mechanism as for emergency calls.

-	How to route a Short Message to the Emergency Response Center (i.e. PSAP) serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location).

# * * * * End of changes * * *