SA WG2 Meeting #170	S2-2506435

Stor-Göteborg, Sweden, August 25 – 29th, 2025

Source:	Nokia

# Title:	New Solution for Support of Short Message to Emergency Response Centre via SMS over IP

# Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for FS_SMS2EC_ARC.

# 1. Discussion

There are three WTs captured in the SID objective of FS_SMS2EC_ARC to support Short Message to Emergency Response Centre in both EPS and 5GS, based on the requirements specified in TS 22.101:

-	WT-1: How to identify a Short Message to Emergency Response Center based on the use of emergency numbers. Address aspects when serving PLMN does not support this feature, but UE supports it.

-	WT-2: Whether and how to identify the type of emergency service (police, ambulance, fire brigade, etc.) using the same identification mechanism as for emergency calls.

-	WT-3: How to route a Short Message to Emergency Response Center (i.e. PSAP) serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location).

This PCR contribution proposes a new solution addressing the key issue of "Support of Short Message Service to an Emergency Response Center via SMS over IP", corresponding to these three work tasks.

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-65.

# * * * * First change * * * *

## 6.X	Solution #1: Support of Short Message Service to Emergency Response Center via SMS over IP

### 6.X.0	High-level solution Principles

This solution aims to resolve Key Issue #2, i.e. "Support of Short Message Service to Emergency Response Center via SMS over IP". The solution principles shall follow the architectural assumptions as described in clause 4.

To address the issue described in WT-1, the following solutions are proposed:

-	UE sends a new indication during Attach/Registration procedure with EPS/5GS, whether the UE supports emergency SMS over IP and/or emergency SMS over NAS.

-	When the serving network receives the above indication from the UE, the serving network sends an indication in response to Attach/Registration procedure whether the serving PLMN supports emergency SMS over IP and/or emergency SMS over NAS.

-	A UE receiving an indication that emergency SMS over IP is supported in a serving PLMN but failed to send the SMS over IP attempts emergency SMS over NAS if configured in the UE, unless emergency SMS over IP is supported over Non-3GPP access. In this case, the UE attempts to send the emergency SMS over IP via Non-3GPP access first, prior to using emergency SMS over NAS.

-	If both emergency SMS over IP and emergency SMS over NAS are not supported in the serving network, it is up to the UE implementation how to proceed. The UE can, for example try to choose another network to send the emergency SMS over IP or SMS over NAS. The UE can also try to establish an emergency call or indicate to the user that emergency SMS over IP or SMS over NAS is not possible.

To address the issue described in WT-2, the following solutions are proposed:

-	The UE uses the same mechanisms (local configuration and handling of emergency Number/emergency service and sub-service type received over NAS during attach/registration procedure).

The UE detects emergency SMS over IP and/or SMS over NAS based on used emergency numbers. Provisioning of emergency numbers and associated types or URN information as specified in clause 4.1 bullets 2a, 2b) and 2c) in 3GPP TS 23.167 applies.

To address the emergency SMS over IP described in WT-3, the following solutions are proposed:

-	When UE detects the sos emergency number and it does not have emergency IMS registration for SMS then UE shall complete emergency IMS registration for SMS. After completing the emergency IMS registration procedure, the UE sends an SMS as a SIP MESSAGE containing emergency SMS in the payload to the P-CSCF in the serving PLMN. The P-CSCF forwards the SIP MESSAGE to an E-CSCF. The E-CSCF may optionally query the LRF to obtain the UE’s location information. The E-CSCF then forwards the SIP MESSAGE, including the emergency short message, to an IP-SM-GW capable of handling emergency SMS. The IP-SM-GW selects an appropriate SMS-SC and forwards the emergency SMS to it. The SMS-SC selects an appropriate PSAP and delivers the emergency SMS to the selected PSAP based on the emergency service type.

-	Any SMS sent back by the PSAP to the UE is handled as a regular SMS.

### 6.X.1	Description

This solution addresses Key Issue #2.

-	UE performs emergency IMS registration and sends SIP MESSAGE containing an R-URI set to the emergency number or sos-URN. The payload of the SIP MESSAGE contains the emergency SM.

-	If E-CSCF queries the LRF and forwards the SIP MESSAGE to an IP-SM-GW able to handle emergency SMS.

### 6.X.2	Procedures

#### 6.X.2.1	Short Message routing to Emergency Response Centre via SMS over IP

During EPS/5GS registration, the UE indicates its capability to support SMS over IP and/or SMS over NAS. The serving network also indicates its capability to support these services in the registration response.

If the serving network has provided the above mentioned SMS over IP indication, then UE invokes the following procedure to send a Short Message to an Emergency Response Centre.

Figure 1: Procedure for Short Message to Emergency Response Centre via SMS over IP

- UE emergency IMS registration follows the procedure as described in clause 7.2 of 3GPP TS 23.167.

- If the UE decides to send SMS over IP, then it sends a SIP MESSAGE to the P-CSCF in the serving network containing an R-URI set to SOS URN or an emergency number and shall include the location of the UE. The SIP MESSAGE contains the emergency short message.

- P-CSCF forwards the SIP MESSAGE to an E-CSCF that is capable of handling emergency SMS over IP.

- E-CSCF optionally queries the LRF to get user location information and selects a proper PSAP.

- E-CSCF includes the PSAP address in the SIP MESSAGE and forwards it to the IP-SM-GW configured in the E-CSCF.

- IP-SM-GW forwards the emergency short message to the PSAP address received in step-4.

- PSAP may send acknowledgement to the IP-SM GW.

  - The successful response code 202 Accepted is sent from IP-SM-GW to E-CSCF -> P-CSCF -> UE.

### 6.X.3	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

UE:

- The UE needs to support sending and receiving feature support flag for emergency SMS over IP and/or SMS over NAS.

P/S/E-CSCF:

- P-CSCF needs to select E-CSCF during IMS emergency registration and route emergency SMS.

IP-SM-GW:

- IP-SM-GW needs to have the capability to forward emergency SMS to the local SMS-SC based on the addressee field.

# * * * * End of change * * * *