SA WG2 Meeting #170	S2-2506436

Stor-Göteborg, Sweden, August 25 – 29th, 2025

Source:	Nokia

# Title:	New solutions for Support of a Short Message to the Emergency Response Centre via SMS over NAS

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This contribution proposes new solution for FS_SMS2EC_ARC.

# 1. Discussion

There are three WTs captured in the SID objective of FS_SMS2EC_ARC to support Short Message to Emergency Response Centre in both EPS and 5GS, based on the requirements specified in TS 22.101:

-	WT-1: How to identify a Short Message to the Emergency Response Center based on the use of emergency numbers. Address aspects when serving PLMN does not support this feature, but the UE supports it.

-	WT-2: Whether and how to identify the type of emergency service (police, ambulance, fire brigade, etc.) using the same identification mechanism as for emergency calls.

-	WT-3: How to route a Short Message to the Emergency Response Center (i.e. PSAP) serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location).

This contribution proposes a new solution addressing the key issue of "Support of Short Message Service to an Emergency Response Center via SMS over NAS", corresponding to these three work tasks.

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-65.

# * * * * First change (all new text) * * * *

## 6.X	Solution #X: Support of Short Message to Emergency Response Centre via SMS over NAS

### 6.X.0	High-level solution Principles

This solution aims to resolve Key Issue #1, i.e. "Support of Short Message to Emergency Response Centre via SMS over NAS". The solution principles shall follow the architectural assumptions described in clause 4.

To address the issue described in WT-1, the following solutions are proposed:

-	UE sends a new indication during the Attach/Registration procedure with EPS/5GS, whether the UE supports emergency SMS over NAS and/or emergency SMS over IP.

-	When the serving network receives the above indication from the UE, the serving network sends an indication in response to Attach/Registration procedure whether the serving PLMN supports emergency SMS over IP and/or emergency SMS over NAS.

-	A UE receiving an indication that emergency SMS over NAS is supported in a serving PLMN but failed to send the SMS over NAS attempts emergency SMS over IP if configured in the UE, unless emergency SMS over NAS is supported over Non-3GPP access. In this case, the UE attempts to send the emergency SMS over NAS via Non-3GPP access first, prior to using emergency SMS over IP.

-	If both emergency SMS over NAS and emergency SMS over IP are not supported in the serving network, it is up to the UE implementation how to proceed. The UE can, for example, try to choose another network to send the emergency SMS over IP or SMS over NAS. The UE can also try to establish an emergency call or indicate to the user that emergency SMS over IP or SMS over NAS is not possible.

To address the issue described in WT-2, the following solutions are proposed:

-	The UE uses the same mechanisms (local configuration and handling of emergency Number/emergency service and sub-service type received over NAS during attach/registration procedure).

The UE detects emergency SMS over IP and/or SMS over NAS based on used emergency numbers. Provisioning of emergency numbers and associated types or URN information as specified in clause 4.1 bullets 2a, 2b) and 2c) in 3GPP TS 23.167 applies.

To address the issue described in WT-2, the following solutions are proposed:

-	When UE detects the sos emergency number and it does not have emergency registration for SMS, then UE shall complete emergency registration for SMS. During UE registration, when UE sends a registration request to the 5GC, the SMSF sends back the indication to the UE about emergency SMS service availability feature in the serving PLMN to the AMF and then AMF forwards the same indication (i.e. emergency SMS over NAS and SMS over IMS are allowed) to the UE in the registration accept.

-	In case of SMS over NAS in EPS during combined attach, the MSC/VLR can inform the MME on the support of emergency SMS routing, or based on MME’s local configuration (e.g. SGd interface case). The MME informs the UE about the emergency SMS routing in the serving PLMN.

-	Any SMS sent back by the PSAP to the UE is handled as a regular SMS.

### 6.X.1	Description

This solution addresses Key Issue #1.

-	The SMSF, if configured with local emergency support, sends the EM availability indication during the registration procedure. AMF informs the UE about the EM SMS feature.

-	UE needs to do access control for a new upper layer event (e.g. EM SMS) to trigger a service request with the indication of emergency SMS.

-	The AMF informs the SMSF that the SMS payload is for an emergency. The AMF also prioritises the NAS procedure based on the emergency indication.

-	The SMSF do not need to use subscription data for emergency SMS cases and routes the SMS to local SMSC/PSAP. The SMSF can replace the RP-DA with a local SMSC address. It may also set a higher transport priority (e.g. the DRMP).

### 6.X.2	Procedures

During EPS/5GS registration, the UE indicates its capability to support SMS over NAS and/or SMS over IP. The serving network also indicates its capability to support these services in the registration response.

If the serving network has provided the above mentioned SMS over NAS indication, then UE invokes the following procedures to send a Short Message to an Emergency Response Centre.

-	Route a Short Message to the Emergency Response Centre via SMS over NAS via 5GS.

-	Route a Short Message to the Emergency Response Centre via SMS over NAS via EPS (via MSC/VLR).

-	Route a Short Message to the Emergency Response Centre via SMS over NAS via EPS (via SGd interface).

#### 6.X.2.1	Short Message routing to the Emergency Response Centre via SMS over NAS via 5GS

Figure 1: Short Message to Emergency Response Centre via SMS over NAS via 5GS

- UE registration to the 5GS follows clause 4.13.3.1 of 3GPP TS 23.502 with the additional indication as described in clause 6.X.2. This additional indication can be sent by AMF.

- If the UE is in the CM-IDLE state, then the UE follows the same procedure defined in clause 4.13.3.3 of 3GPP TS 23.502. UE indicates the emergency cause indicator in the service request.

- If the UE is in the CM-CONNECTED state, it follows the same procedure defined in clause 4.13.3.3 of 3GPP TS 23.502, and it sends an uplink SMS message then, the UE needs to do access control for a new upper layer event (EM SMS in the SMS body of the NAS payload) to trigger a service request.

- The AMF forwards this message to the SMSF. The AMF provides the UE location and informs the SMSF that the SMS payload is for EM.

- The SMSF does not lookup subscription data and routes the SMS to the local SMSC/PSAP. The SMSF can replace the RP-DA with a local SMSC address. It may also set a higher transport priority (e.g. the DRMP).

- If in step 2 the UE includes a short message service centre address in the RP-DA instead of the NAS payload type as “EM SMS", then the SMSF routes to the serving PLMN SMSC.

The SMSC/PSAP forwards the SMS /Submit report to the SMSF.

- The SMSF forwards to the AMF.

- The AMF forwards the SMS submit report to the UE.

#### 6.X.2.2	Short Message routing to the Emergency Response Centre via SMS over NAS via EPS (via MSC/VLR)

Figure 2: Short Message to Emergency Response Centre via SMS over NAS via EPS (via MSC/VLR)

- The attach procedure follows the procedure as specified in clause 5.2 of 3GPP TS 23.272 with the additional indication in the Attach Request/Response as described in clause 6.X.2. This indication can be sent by either MME or MSC/VLR.

- The UE in idle-Mode or active-Mode follows the same procedure as defined in 8.2.2 of 3GPP TS 23.272 with additionally sending Emergency SMS as a different payload type in UL NAS transport.

- The MME includes the location information (cell ID, tracking area, location area), and the payload type is EM SMS, and forwards to the MSC/VLR in an Uplink Unidata message.

- For roaming UEs, the MME does not send the EM SMS payload to the H-PLMN MSC/VLR; instead, it sends it to the local SMS-IWMSC.

- If in step 3 the MME includes the short message service centre address in the RP-DA instead of the NAS payload type as "EM SMS", then the MSC/VLR routes it to the serving PLMN SMSC.

- Alternatively, the MME can send the serving PLMN support of EM SMS based on local configuration without depending on the MSC/VLR. The MME forwards the EM SMS to the local SMSC for a roaming subscriber.

#### 6.X.2.2	Route a Short Message to Emergency Response Centre via SMS over NAS via EPS (via SGd interface)

Figure 3: Short Message to Emergency Response Centre via SMS over NAS via EPS (via SGd interface)

- The attach procedure follows the procedure as specified in clause 5.3.2.1 of 3GPP TS 23.401 with the additional indication in the Attach Request/Response as described in clause 6.X.2. This indication can be sent by MME.

- The UE in idle-mode or active-mode follows the same procedure as defined in 5.3.4.1 of 3GPP TS 23.401 with additionally sending Emergency SMS as a different payload type in UL NAS transport.

- The MME mandatorily includes the location info (cell ID, tracking area, location area), and the payload type is EM SMS, towards the SMSC.

- If in step 2 the UE includes the short message service centre address in the RP-DA instead of the NAS payload type as "EM SMS", then the SMSF routes to the serving PLMN SMSC.

- For roaming UEs, the MME does not send the Emergency SMS payload to the H-PLMN SMSC, but rather sends it to a local SMSC.

- The local SMSC identifies the EM service type and forwards it to the appropriate SMS-IWMSC/PSAP.

### 6.X.3	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

UE: Need to support network feature indication from EPS and 5G core on the availability of Emergency Messaging.

AMF:

- Need to support a new NAS payload type for EM SMS.

- Need to support feature indication to indicate serving PLMN’s support for EM SMS

MME:

- Need to support a new NAS payload type for EM SMS.

- Need to support feature indication to indicate serving PLMN’s support for EM SMS

SMSF:

- Replacement of RP-DA.

- Need to route to local SMSC for EM SMS.

SMS-IWMSC:

- Need routing support to local PSAP

# * * * * End of change * * * *