SA WG2 Meeting #170	S2-2506441

25 – 29 Aug, 2025, Goteborg, SE

Source:	Samsung

Title:	[WT#6] Work Task clarification

Document for:	Agreement

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This pCR proposes clarification and scope for WT#6.

1. Justifications

The following text for WT#5 is described in the SID for 6G:

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

NOTE 7: Application layer mechanism and exposure framework may require coordination with SA6.

Further Clarifications Required

Further clarification on the “support of computing for UE, core network and application server” is needed. The current wording “computing for X” is not clear. The possible clarification on this part can be given as follows:

- Computing for UE: It can be interpreted as follows. The app client in the UE can delegate its (some part of) workflow to its corresponding server. When broken down into finer detail, this implies the followings: (i) the app client itself is designed or implemented to support such flexible operation, (ii) a given app server (corresponding to service client in the UE) will be hosted on a given service hosting environment with a reserved resource such as memory, CPU, GPU, etc. (iii) the core network can support, for a given UE, discovery of the app server and connection establishment can be controlled by the core network, (iv) the service hosting environment can be managed via 3GPP-defined entity.

- Computing for application server: The app server outside the core network (maybe in the DN) can delegate its (some part of) workflow to its corresponding server. When broken down into finer detail, this implies the followings: (i) the app server itself is designed or implemented to support such flexible operation, (ii) a given app server (corresponding to the server which need support of computing) will be hosted on a given service hosting environment with a reserved resource such as memory, CPU, GPU, etc. (iii) the core network can support, for a given app server, discovery of the app server and connection establishment can be controlled by the core network, (iv) the service hosting environment can be managed via 3GPP-defined entity

- Computing for core network: It is not clear what type of and how computing related support/service can be offered to core network functions.

Further clarification on the computing service is also required. As per discussion in SA1, TR 22.870 describes the terminology “6G Computing Service: A service provided by 6G network utilizing computing resources in Service Hosting Environment, which can be used by a subscriber (via UE)/3rd party”. From SA2 architecture perspective, the following aspects should be clarified:

Location of Service Hosting Environment (SHE): There would be different understanding of the location of SHE, e.g., within the CN, Data Network, or both.

- If located in the CN, the computing service provider would be limited to the MNO. The current 5G system cannot support such architecture.

- If located in the DN, it is very similar to edge computing service. MNO or any 3rd party can be a computing service provider. 5G system already provides various functionalities, which are related to 5G edge computing features.

Clarification on computing resource: In SA1 discussion in 22.870, some examples were described as follows: data centre, edge node, edge computing server, something distributed across cloud server/network/device edge (e.g., CPE), xPU, etc. Considering these examples, the following two types of computing resource can be considered: (i) node or server can be referred as computing resource, or (ii) finer level of virtual resource of hosting environment can be a type of resource, e.g., xPU, storage. The related computing resource has been specified in SA5 and SA6 as below:

| SA6 specified resource information for EAS (TS 23.558) |  | SA5 specified virtual resource attribute (TS 28.538) |  |
| --- | --- | --- | --- |
| Available Compute | The maximum compute resource available for the AC. | virtualResource | This parameter defines the virtual resource requirements of an EAS. |
| Available Graphical Compute | The maximum graphical compute resource available for the AC. | virtualMemory | It indicates the minimum virtual memory size requirements for EAS in megabytes. (see clause 7.1.9.3.2.2 in ETSI NFV IFA-011 [7]). |
| Available Memory | The maximum memory resource available for the AC. | virtualDisk | It indicates the minimum virtual disk storage requirement for the EAS (see clause 7.1.9.4.3.2 in ETSI NFV IFA-011 [7]). |
| Available Storage | The maximum storage resource available for the AC. | virtualCPU | It indicates the virtual CPU requirement for the EAS (see clause 7.1.9.2.3.2 in ETSI NFV IFA-011 [7]). |

Relevance with 5G edge computing: For a computing service to subscriber (via UE), it should be clarified what is the difference from the edge computing. 5G system provides the EAS discovery and UP path configuration for a given EAS. Considering the above examples of computing resource, if computing resource includes edge node or edge computing server and it is located in the DN, then the functionalities of 5G edge computing can be reused or enhanced. If computing resource is something within the core network or belongs to the core network, then 5G edge computing features cannot be re-used as it is.

Further clarification on “coordination between UE, core network and application server” is required. “Coordination” needs clarification. Interaction among UE, CN, and application server would be mainly based on the following functionalities provided by the CN to UE and application server: e.g., discovery of the computing resource on operator’s SHE and authorization for accessing the computing resource on operator’s SHE toward UE or AF. If not clarified, this pCR proposes to delete the term, coordination.

Considering the above discussion, for clarity in task definition, this pCR proposes to categorizing 6G computing service into conventional edge computing service and a new operator-provided computing service.

|  | Edge computing as 6G computing service | Operator service as 6G computing service |
| --- | --- | --- |
| Location of SHE | Data Network (outside to core network) | Core Network (or operator’s network) |
| Computing resource / consumer of resource | Edge Application Server / Application in the UE | FFS (Application in the UE and/or 3rd party application server) |
| Service Path between UE/AF and SHE | Over UP (for between UE and SHE); No path required for between AF and SHE other than network capability exposure | FFS (UP or new dedicated plane for between UE/AF and SHE) |
| Controllability of computing resource | Provider of hosting environment (MNO or 3rd party) | MNO |
| Target service (main purpose) | Low latency service | Service requiring heavy computation; Service implemented with a specific app logic of separating and delegating its workflow to other (mainly providing sufficient resource for computation, may not be for low latency service) |

Architectural Motivation & Gap Analysis

- Further optimization/simplification of edge computing features/functionalities: In 5G system, various functionalities/features for support edge computing have been specified, including: session breakout with ULCL/BP, EAS (re)discovery, Edge Relocation, Edge-specific network exposure with Local NEF, edge specific information provisioning via PCO (for SA6 EDGEAPP support), AF Guidance to PCF Determination of Proper URSP Rules, edge service during roaming (HR-SBO), N6 delay measurement for EAS/L-UPF (re)selection, Local Offloading Management with locally-deployed I-SMF. The above can be revisited in terms of the following two aspects: (i) how much we can simplify or optimize them for lean and streamlined standards for 6G edge computing as part of computing service, (ii) which functionality/feature for edge computing can be reused for the 6G computing service. For example, for a case where the computing node is located in the DN, the EAS discovery in 5G system can be considered as a basis in 6G. However, in 5G, there are four options of EAS discovery as follows:
 Option A: EASDF include/replace EDNS Client Subnet (ECS) option into the DNS Query message
 Option B: EASDF sends the DNS Query message to a Local DNS server
 Option C: without EASDF, SMF configures the local DNS server to the UE as new DNS server.
 Option D: without EASDF, ULCL steering is based on L4 information (DNS port number) and DNS Queries for a                                                      			   given FQDN (range) will be locally routed to a Local DNS server.

In 6G, it is required to simplify the discovery mechanism and reduce the options for the 6G computing service. Additionally, in order to support mobility, a mechanism for the relocation of computing node is required. In the context of re-design based on the existing 5G edge relocation, it is envisioned that 6G computing node could be also deployed within the operator’s network or combined within the CN, which would allow for a procedure with reduced complexity and less configuration.

- Low level of controllability of core network/operator for computing resource: In 5G system, computing resource (i.e., EAS) is located in the DN. For such a limited deployment case, 5G system provides just UP path configuration and its related capability exposure services. This implies that operator or CN has low controllability on the resource. Also, in general, it is up to management system of the edge hosting environment. When it comes to operator-provided SHE, more controllability can be given to the operator via standardized way. Note that this aspect is overlapped and entangled with SA5 scope. Especially,

Technical Impact:

- Overall system architecture and procedure will be investigated for optimizing and simplifying the edge node (re)discovery/relocation, UP path configuration, etc.

- Overall system architecture and procedure to support SHE within the core network and provide operator-controllability on the computing resource over SHE. Especially, for identified computing resource on the operator’s SHE, the followings are expected:

  - How to support the discovery of the computing resource on operator’s SHE

  - How to support the authorization for accessing the computing resource on operator’s SHE

  - How to provide controllability (to MNO) on the computing resource, including the enablement of computing resource on the operator’s SHE, support for monitoring resource usage.

  - Whether and how to support network capability exposure for the computing resource on operator’s SHE

  - Whether and how the functionalities defined for 5G edge computing can be reused or enhanced

Further discussion aspects for this WT:

- Dependency with other WTs (WT#1.1)

Dependency with WT#1.1: For the aspects of interaction between UE and the CN, the dependency is identified in terms of how to exchange singling related to computing service data between UE and CN.

- Dependency with other WGs

Dependency with SA5: Coordination is need in terms of how to specify and control the computing resource provided by the core network.

Dependency with SA6: Coordination is need in terms application layer mechanism and exposure framework.

**** First Change ****

Annex A.X. WT#5 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#6: Study aspects on how to support computing service for UE and application server in 6G (e.g., exposure of computing service in the core network). 6G computing service can be categorized into the conventional edge computing service and a new operator-provided computing service.

|  | Edge computing as 6G computing service | Operator service as 6G computing service |
| --- | --- | --- |
| Location of SHE | Data Network (outside to core network) | Core Network (or operator’s network) |
| Computing resource / consumer of resource | Edge Application Server / Application in the UE | FFS (Application in the UE and/or 3rd party application server) |
| Service Path between UE/AF and SHE | Over UP (for between UE and SHE); No path required for between AF and SHE other than network capability exposure | FFS (UP or new dedicated plane for between UE/AF and SHE) |
| Controllability of computing resource | Provider of hosting environment (MNO or 3rd party) | MNO |
| Target service (main purpose) | Low latency service | Service requiring heavy computation; Service implemented with a specific app logic of separating and delegating its workflow to other (mainly providing sufficient resource for computation, may not be for low latency service) |

- For edge computing service, how to simplify and optimize the 5G edge computing features with the reduced options

- For a new operator-provided computing service, the followings will be investigated:

  - Identify and specify the computing resource on the operator’s SHE that can be controlled and managed via the core network

  - How to support the discovery of the computing resource on operator’s SHE

  - How to support the authorization for accessing the computing resource on operator’s SHE

  - How to support the authorization for accessing the computing resource on operator’s SHE

  - How to provide controllability (to MNO) on the computing resource, including the enablement of computing resource on the operator’s SHE, support for monitoring resource usage.

NOTE 1:	Coordination with SA5 is need in terms of how to specify and control the computing resource provided by the core network.

  - Whether and how to support network capability exposure for the computing resource on operator’s SHE

NOTE 2:	Coordination with SA6 is need in terms application layer mechanism and exposure framework.

  - Whether and how the functionalities defined for 5G edge computing can be reused or enhanced

**** End of Changes ****