SA WG2 Meeting #170	S2-2506455

Goteborg, Sweden, August 25 – 29, 2025

Source:	NTT DOCOMO

Title:	KI#3: New Sol: Energy policy based NF discovery

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes a solution for KI#3 where multiple energy priorities can be configured in the NF profile and the consume select one of the as the criteria in NF discovery.

# 1	Discussion

Different instances of a network function (e.g., different UPFs deployed in the operator network) may have different energy-related characteristics (e.g., some UPFs may powered mainly by renewable energy, some UPFs may be more energy efficient than the others), hence, considering energy-related characteristics of NF instances helps to achieve operator’s energy policies goals. On the other hand, the operator may have different energy policies for different categories of users for example the operator may maximize energy efficiency for a group of UEs (or a specific network slice) while trying to minimize the energy consumption for another group of UEs (e.g., another network slice).

In the previous meeting, it was proposed to formulate the operator’s energy policy as the energy priority stored in the NF profile in the NRF. Having only one energy priority cannot address the cases that operator needs to implement multiple energy policy at the same time for different groups of users. For this purpose, it is proposed to extend the concept of “energy priority” to multiple values where each value is corresponding to an energy policy, and also the energy policy for the group of UEs (or a network slice) is configured in the UDR. During the NF discovery and selection process, the consumer NF or the NRF uses the configured policies in the UDR to select the corresponding energy priority in the NRF profile.

# 2 Proposal

It is proposed to update TR 23.700-67 according to the following text.

********** First Change **********

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |
| 1 | X |  |  |
| 2 | X | X |  |
| 3 |  | X |  |
| 4 |  | X |  |
| 5 |  | X |  |
| 6 |  | X |  |
| 7 |  | X |  |
| 8 | X |  |  |
| 9 | X |  |  |
| 10 | X |  |  |
| 11 | X |  |  |
| 12 | X |  |  |
| 13 |  | X |  |
| 14 |  | X |  |
| 15 |  | X |  |
| 16 |  | X |  |
| 17 |  | X |  |
| 18 |  | X |  |
| 19 |  |  | X |
| 20 |  |  | X |
| 21 |  |  | X |
| 22 | X |  | X |
| 23 |  |  | X |
| X |  |  | X |

********** Second Change (All new text) **********

## 6.x	Solution #X: Energy policy based NF discovery

### 6.x.0	High-level solution principles

The high-level principles of the proposed solution are as follows:

-	In the profile of the NF, an array of energy priority may be stored. The array is indexed by the operator's energy policy (i.e., one priority value per energy policy).

-	To apply different energy policy, a group of UEs (or a network slice) is configured in the UDR. The configuration includes the energy policy differentiation information.

-	In the NF discovery process, either the consumer NF or the NRF retrieves the energy policy information from the UDR and then selects the corresponding index in the array of the energy priorities in the NF profile.

### 6.x.1	Description

The proposed solution enables NF discovery (and selection) based on different energy policies configured for different groups of UEs (or network slice). For this purpose, the operator determines the priority of the NF per energy policy, for example for a given UPF, the priority of the NF for the energy-policy#1 would be 100 while for the energy-policy#2 it would be 30. Note that these numerical values indicate the priority rather than specific energy parameter. The second configuration made by the operator is the configuration of group of UE (or network slice) that should be treated by different energy policies (e.g., for a group of UE that energy consumption should be minimized and for another group the ratio of renewable energy needs to be maximized). The configuration, which is stored in the UDR, includes the energy policy differentiation information that indicates the energy policy that should be applied for group of UEs or network slice during the NF selection and discovery for the corresponding UEs. The information can be used either by the consumer NF or the NRF to select the corresponding priority.

In the first option, the consumer NF queries the energy policy differentiation information from the UDR and includes it in the NF discovery request. The NRF uses this information to select one of the entries in the energy priority array (which corresponds to the energy policy information provided by the consumer) and includes it in the response.

In the second option, the consumer NF includes the ID of the group of UEs (or network slice) in the discovery request and NRF uses this information to query the energy policy differentiation information configured or this group of UEs (network slice) from the UDR. And then uses this information to select the corresponding energy priority from the NRF profile and returns it to the consumer.

Figure 6.x.1-1 show the overall concept of the first option. The consumer NF uses the group ID or S-NSSAI to query the configured energy policy for the group/slice as Energy Policy differentiate information (EPDI). This EPDI is included in the NF discovery request to the NRF and then the NRF uses it to select one of the configured priorities in the NRF profile.

Figure 6.x.1-1: Overview of the proposed solution – First option

### 6.x.2	Procedures

The figure 6.x.2-1 shows the first option of the procedure of NF discovery in the case of multiple energy priority configured per NF. In this option, the consumer NF queries the UDR to identify the configured energy policy for the group of UEs or network slice.

Figure 6.x.2-1: Procedure of NF discovery in the case of multiple energy priority – First option

0.	It is assumed that the consumer NF knows the UE group ID/S-NSSAI that a NF is going to be discovered (which is already supported by the current specifications).

1.	The consumer NF queries the configuration of the UE Group or Slice from UDR to get the corresponding EPDI that is the energy policy ID should be applied for this group of UE or slice.

2.	The consumer NF receives the EPDI from the UDR.

3.	The consumer NF includes the EPDI in the Nnrf_NFDiscovery_Request in addition to the other criteria should be considered for NF discovery (e.g., NF type, etc.)

4.	The NRF discovers the NFs using the provided criteria.

5.	The NRF selects the only one of the energy priorities configured for the discovered NFs which corresponds to the provided EPDI.

6.	The NRF sends the list of the profile of the discovered NFs including the selected energy priority to the consumer NF.

7.	The consumer NF selects the NF using the provided NF profiles considering the energy priority.

The figure 6.x.2-2 shows the second option of the procedure for NF discovery in the case of multiple energy priority configured per NF, the main difference w.r.t. option 1 is that the NRF queries the configure energy policy from the UDR.

Figure 6.x.2-2: Procedure of NF discovery in the case of multiple energy priority – Second option

0.	It is assumed that the consumer NF knows the UE group ID/S-NSSAI that a NF is going to be discovered (which is already supported by the current specifications).

1.	The consumer NF includes the group ID or slice ID in the Nnrf_NFDiscovery_Request in addition to the other criteria should be considered for NF discovery (e.g., NF type, etc.)

2.	The consumer NRF queries the configuration of the UE Group or Slice from UDR to get the corresponding Energy Policy Differentiation Information (EPDI) that is the energy policy ID should be applied for this group of UE or slice.

2.	The NRF receives the EPDI from the UDR.

3.	4.	The NRF discovers the NFs using the provided criteria.

5.	The NRF selects the only one of the energy priorities configured for the discovered NFs which corresponds to the provided EPDI.

6.	The NRF sends the list of the profile of the discovered NFs including the selected energy priority to the consumer NF.

7.	The consumer NF selects the NF using the provided NF profiles considering the energy priority.

### 6.x.3	Impacts on existing services, entities and interfaces

The solution has the following impacts:

NRF:

-	Support storing multiple energy priority values (one priority value per energy policy) in the NF profile

-	Support selecting only one energy priority from the NF profile based on the EPDI (either provided by the consumer NF or it is queried from the UDR)

UDR:

-	Store configuration of the Group of UE or network slices including the energy policy differentiation information

NFs (consumer NFs):

-	Provide EPDI or EU Group ID or Network Slice ID to the NRF

********** End of Changes **********