SA 2WG2 Meeting #170	S2-2506456

# August 25 – 29, 2025, Gothenburg, Sweden

Source: 	NTT DOCOMO

Title: 	KI#3: Interim agreements

Document for: 	Approval

Agenda Item: 	20.4.1 (FS_EnergySys_Ph2)

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes interim agreements for KI#3 of FS_EnergySys_Ph2.

# 1	Discussion

Basically, two categories of solutions, which are also related to each other, are proposed for KI#3:

- Enhancing the generic NF discovery and selection mechanisms (including the NF profile content and the related procedures) to consider the energy-related information

- Extending UP path adjustment mechanism to be energy-aware

This paper summarizes the concepts of the proposed solutions as the interim agreements for the normative work.

# 2 Proposal

It is proposed to update the solution for TR 23.700-67 according to the following text.

********** First Change **********

### 7.1.x	Agreed Principles for KI#3

The following principles are agreed for KI#3:

-	NF profile (stored in NRF) may include "energy priority" field which is provided by OAM based on the operator's energy policy.

-	In addition to other information of the NF profile (e.g., load), the consumer NF takes the "energy priority" into account for selecting network functions based on operator's configuration.

-	In case of multiple configured energy policies in network, the "energy priority" is an array of priorities. Each entry of the array is the priority of the NF in the corresponding energy policy configured by the operator.

-	SMF may be triggered (e.g., by EIF and/or NRF in the case of significant change in PDU/UPF energy-related information) to adjust UP path(s) of (a subset of) established PDU session(s) via UPF reselection to optimize the operator's energy policy subject to SSC mode of the PDU sessions.

********** End of Changes **********