SA WG2 Meeting #170	S2-2506462

25 – 29 August, 2025, Goteborg, SE

Source:	Apple

Title:	[WT#4] 6G System Support for Sensing

Document for:	Agreement

Agenda Item:	20.6.4

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: In this contribution we provide justifications for a revised scope of WT#4 on Sensing. The justification includes the relation of 6G Sensing with 5G-A Sensing in SA2, use cases of interest, an introduction to the concept of UE-centric Sensing, and an overview of the envisioned 6G architecture for Sensing.

# 1	Justifications

In this contribution, we aim to provide enhancements to the sensing architecture as defined in TR 23.700-14 for 5GA. TR 23.700-14 focuses on Network-based sensing only, with the gNB being the only allowed Sensing Entity. However, sensing the environment or objects can be performed both by the User Equipment (UE) or the 3GPP access network. Both approaches have advantages and disadvantages; for example, it is assumed that indoor sensing can benefit by using the UE as a sensing device to avoid signal degradation. In this section, we present justifications to enable the UE to perform sensing and be a self-contained sensing device in the 6G system, thus benefiting future 6G users.

In terms of use cases, an entity that contains a sensing transmitter and receiver (e.g. a UE or base station) may be used to sense its surroundings to understand if there are objects that are passing nearby, and approximate characteristics of such objects e.g. velocity of movement, size, proximity to the sensing transmitter/receiver. The use cases above can be assumed to fall under the generic category of ‘Ambient Sensing’, i.e. detecting the surroundings as well as any objects and their characteristics in the surroundings of the UE. This can be based on both mono-static or bi-static sensing modes.

Current 3GPP support for ‘Ambien Sensing’ use cases can be found in the following references:

- TR 22.837 Study on Integrated Sensing and Communication

  - Clause 5.1 / 5.6 Use cases of intruder detection

  - Clause 5.21 Use case of seamless XR streaming (sensing-aided communications)

  - Clause 5.27 Use case public safety search and rescue or apprehend

- TR 22.870 Study on 6G Use Cases and Service Requirements

  - Clause 7.12 Use case on enhanced XR user navigation

  - Clause 11.8 Use case on collaborative AMRs

Further, a UE is a portable device with a small form factor and unique mobility capabilities, all of which makes the UE a valuable sensing device. In UE mono-static sensing, the UE can transmit a sensing signal (Tx) and may receive the echoes of such a sensing signal (Rx). Alternatively, the UE can receive a sensing signal (Rx) that is transmitted by the TRP (Tx). Furthermore, depending on the UE compute capabilities, the sensing measurements (sensing data) might be or might not be processed at the UE itself. We argue however that 6G UE’s should be powerful enough to be capable of processing sensing results.

Hence, for 6G, we envision sensing being performed both by the network and the UE. Within each of these two broad categories, additional sensing modes and approaches can be defined depending on a variety of parameters, as shown in Table 1 below.

Table 1:	Classification of sensing modes and approaches

|  | # UE-Centric | # UE-Based | # UE-Based / NW-Assisted | # Network-Based or Network-Based/UE-Assisted | # Network-Exposed or Network-Exposed/UE-Assisted |
| --- | --- | --- | --- | --- | --- |
|  | UE is the Sensing Service Requester and Sensing Service Consumer | UE is the Sensing Service Requester and Sensing Service Consumer, but the Sensing Result will be reported to the NW (like MO-LR in Positioning) | UE is the Sensing Service Requester and Sensing Service Consumer, but the Sensing Result will be reported to the NW. The NW might participate in performing the Sensing Tx (e.g. bi-static Sensing) | NW is the Sensing Service Initiator and Sensing Service Consumer. In case of UE-Assisted, the NW might involve the UE to support in performing the sensing (e.g. bi-static, multi-static sensing) | A 3rd party entity is the Sensing Service Requester (e.g. AF) and Sensing Service Consumer. The NW is performing the sensing on behalf of the 3rd party entity. The sensing result (or sensing data) is shared with the 3rd party entity. In case of UE-Assisted, the NW might involve the UE to support in performing the sensing (e.g. bi-static, multi-static sensing) |
| Sensing Initiator | UE | UE | UE | NW | Third Party |
| Manage, Control, and Configure Resources | NW | NW | NW | NW | NW |
| Perform Sensing Tx | UE | UE | UE and/or NW | UE and/or NW | UE and/or NW |
| Perform Sensing Rx | UE | UE | UE and/or NW | UE and/or NW | UE and/or NW |
| Process Sensing Data | UE | UE | UE | NW | NW |
| Consumer of Sensing Result | UE | UE and/or NW | UE and/or NW | NW | Third Party |

Furthermore, it is to be expected that the WT#4 scope for 6G is be based on the study work and outcomes documented in TR 23.700-14 for 5GA. Therefore, it’d make sense a structure for the 6G WT on Sensing to align with the WT structure for the 5GA FS_Sensing_ARC item in SA2.

## 1.1	UE-centric Sensing

In Table 1 we introduce a UE-centric sensing as a sensing approach whereby the UE:

- requests a sensing grant via a sensing service request message from the 6GS

- transmits and receives a sensing signal

- may have the compute and storage capabilities to process the measurements (sensing data)

- is the consumer of the sensing results (processed sensing data)

UE-Centric sensing is influenced by the 5GC-MO-LR procedure defined for positioning in TS 23.273, where the UE sends a MO-LR location request to the network indicating which kind of location service the UE is interested in. The network’s LCS architecture processes the request and responds with an answer related to the location service that was previously requested. Like the 5GC-MO-LR a MO-SR (Mobile Originated-Sensing Request) is envisioned: The UE sends a MO-SR to the network indicating which kind of sensing service the UE is interested in. Depending on the requested sensing service, the network (e.g. SeMF, Processing Entity) responds with the result for such a sensing service request. A sensing service request might refer e.g. to a sensing grant, sensing results/data from the network, or to provide sensing results/data to the network.

UE-centric sensing is a privacy-preserving mechanism to perform sensing. If sensing data or sensing results must leave the UE, attention must be paid to privacy and security, which is critical for some use cases related to personal safety and radar.

## 1.2	6G Architecture for Sensing

In the following, we briefly discuss the key entities we envision are needed to support sensing as part of the 6G architecture.

Figure 1: Proposed Sensing Architecture to support the UE-Centric sensing type.

### 1.2.1	Sensing Management Function

The Sensing Management Function is envisioned to be a control and management entity in the 6G Core Network for all sensing services and activities. The Sensing Management Function shall be built up on what will be defined for 5GA. Next to what will be defined for 5GA, some new services or features may be required to support sensing being performed at the UE, and in particular UE-centric sensing, e.g.:

- Coordinating resources allocated to base stations and UEs involved in sensing operation

- Authorization of UEs as sensing entities

- Configuration of UEs to perform in sensing operation

- Etc.

### 1.2.2	Sensing Policy and Authorization Function

The Sensing Policy and User Function is envisioned to be for sensing what the UDM and PCF is for communication. Like UDM this function might handle:

- Subscription Sensing Management: Stores and manages authentication credentials, and sensing service subscriptions (e.g. a UE-Centric sensing service subscription)

- Authentication & Authorization for Sensing: Generates authentication vectors and manages subscriber access credentials

- User Identity Management for Sensing: Handles the user identity and privacy protection

The policy functionality of the Sensing Policy and Authorization Function might be responsible for:

- Traffic Control: to control that the user sensing traffic does not exceed the negotiated capacities

- Policy Decision Making: Makes real-time policy decisions for sensing session management, sensing mobility, and sensing service access

- Quality of Service (QoS) Control: Defines and enforces QoS policies for sensing

- Charging Policy Management: Controls charging rules and policies for different sensing services and subscribers

### 1.2.3	Sensing Processing Entity

The Sensing Processing Entity is envisioned to process sensing measurements (i.e. sensing data) into sensing results which are useful for a Sensing Service Consumer (SSC). Thus, the functionality of this entity can be summarized as follows:

      - Processing of Sensing Data from various TRPs and UEs

      - Compute Sensing Results

The Sensing Processing Entity might be required for Network-Based sensing types and for UE-Centric sensing types. In case the UE doesn’t have the compute resources to do the processing on its own it can fall back to Sensing Processing Entities in the CN or DN (e.g. AF). Summarized, the processing can be located on the UE, in the DN (e.g. at an AF), or in the CN (e.g. a Sensing Processing Entity).

## 1.3	Summary

The following key aspects summarize our current view on 6G sensing:

- The study should follow a similar structure to the 5GA sensing study

- The scope should include sensing performed both at the network and the UE

- Apple is interested on UE-centric sensing as an approach to preserve privacy

- The architecture for 6G sensing should be based on a Sensing Management Function Sensing Policy and Authorization Function, and a Sensing Processing Entity

**** First Change ****

# Annex A.X. WT#4 Scope

WT#4: Study the integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported and other sources of sensing data.

The following aspects will be studied:

- WT#4-1: Architecture and function enhancements to support sensing.

- WT#4-2: Service authorization and revocation. This includes investigating mechanisms for authorization and revocation sensing service requests from the service consumer.

- WT#4-3: Discovery and selection of sensing entities based on service requirements triggered by the service request and capability of the sensing entities.

- WT#4-4: Configuration parameters for the support of ISAC services.

NOTE X1:	WT#4 needs to consider the work and conclusions of R20 FS_Sensing_ARC.

NOTE X2:	The scope of WT#4 includes sensing performed at both network and UE sides.

NOTE X3:	Sensing data and the associated information collection and transport, as well as the mechanisms for providing sensing associated information and result are in scope of WT#5.

NOTE X4:	Coordination with RAN WG for RAN aspects, SA3 for privacy and consent aspects, and SA5 for OAM aspects will be needed.

**** End of Changes ****