3GPP TSG-SA WG2 Meeting #170	S2-2506472

Stor-Göteborg, Sweden, Aug 25 – 29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6357 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Parameter Alignment for UP Path Change Subscription in TS 23. 501 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 4.3.6.3 of TS 23.502, “Notification of User Plane Management Events To support UE-Satellite-UE (UE-SAT-UE) communication, the event that satellite identifier serving the UE has changed is represented by the event a DNAI has changed and the DNAI value can be derived from the satellite identifier.” Npcf_PolicyAuthorization_Notify and Nsmf_EventExposure_Notify include only the DNAI in UP path change notifications, which is derived from the satellite identifier, rather than sending the satellite identifier redundantly. However, in clause 5.4.14.3 of TS 23.501, the notification received by the PCF and P-CSCF is described as containing the satellite ID of a target satellite, without mentioning the DNAI. This creates an inconsistency between the two specifications regarding how satellite-related user plane event information is conveyed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the input of Npcf_PolicyAuthorization_Notify and Nsmf_EventExposure_Notify includes the DNAI, not the satellite identifier. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The inconsistency regarding the content of the early and late notification will persist, leading to potential implementation complexity and operational confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.14.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.4.14.3	Support for UE-satellite-UE communication when serving satellite changes

NOTE 1:	The following mechanism assumes gNB, UL CL/L-PSA, IMS-AGW are onboard the same satellite.

In the UE-SAT-UE communication, if the UE's serving satellite changes (e.g. due to UE movement or the satellite movement), the Xn/N2-based handover procedure can be performed to change the serving gNB from that on the source satellite to that on the target satellite. The AMF includes the target satellite ID in the Nsmf_PDUSession_UpdateSMContext Request to the SMF. The SMF decides whether to route IMS user plane traffic to the PSA on the ground or retain the UL CL/L-PSA on the satellite based on local configuration and other factors as specified in clause 6.3.3.

NOTE 2:	The SMF can decide to route the IMS user plane traffic to the PSA on ground, e.g., due to no connection between the source satellite and the target satellite.

If the SMF decides to route the IMS user plane traffic to the PSA on the ground and the procedure for removing the UL CL/L-PSA as described in clause 4.3.5.5 of TS 23.502 [3] is to be performed, the SMF provides an early notification without DNAI via Nsmf_EventExposure_Notify to PCF. The PCF forwards this notification to the P-CSCF via Npcf_PolicyAuthorisation_Notify, indicating that the 5GC user plane needs to fall back to the ground.

If the SMF decides to retain the UL CL/L-PSA on the satellite, it provides an early notification with a DNAI associated with the target satellite via Nsmf_EventExposure_Notify to PCF and the PCF forwards this DNAI to the P-CSCF via Npcf_PolicyAuthorisation_Notify. In this case, the IMS informs 5GC whether the UE-SAT-UE communication can or cannot continue, as outlined in clause AE.5.2.1 of TS 23.228 [15], i.e. the PCF receives a response from the P-CSCF and forwards it to the SMF:

-	If the response indicates that the UE-SAT-UE communication can continue, the SMF allocates the UL CL/BP and L-PSA on the target satellite by performing a simultaneous change of UL CL/BP and additional PSA for the PDU Session, as described in clause 4.3.5.7 of TS 23.502 [3], where the UL CL/BP and L-PSA rule shall be updated with the IMS-derived transport address and optional N6 traffic routing information associated with target DNAI as received in the response. A late notification is sent by SMF to the P-CSCF via PCF afterwards, as outlined in clause 4.3.6.3 of TS 23.502 [3].

-	If the response indicates that the UE-SAT-UE communication cannot continue, the SMF removes the UL CL/BP and L-PSA on the source satellite by following the procedure described in clause 4.3.5.5 of TS 23.502 [3].

NOTE X: The event that the satellite serving the UE has changed is represented by  a change of the DNAI, and the DNAI value can be derived from the satellite identifier.

# * * * * End of change * * * *