3GPP TSG-SA WG2#170	S2-2506497

Gothenburg, Sweden, 25-29 August 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	New Solution – SMS over NAS

Document for:	Discussion/Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC

Abstract of the contribution: the paper proposes a new solution to be documented in the TR for the support of SMS over NAS.

# 1.	Discussion

The study item on the support of Short Message Service to Emergency Response Centre in both EPS and 5GS, based on the requirements specified in TS 22.101, focuses on three main aspects:

-	WT-1: How to identify a Short Message to Emergency Response Center based on the use of emergency numbers. Address aspects when serving PLMN does not support this feature, but UE supports it.

-	WT-2: Whether and how to identify the type of emergency service (police, ambulance, fire brigade, etc.) using the same identification mechanism as for emergency calls.

-	WT-3: How to route a Short Message to Emergency Response Center (i.e. PSAP) serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location).

NOTE 1:	Depending on the type of emergency service, Short Messages can be routed to different emergency response centres.

NOTE 2:	This study assumes that Short Messages initiated by the Emergency Response Center in response to the Short Message to the Emergency Response Center are treated as regular Short Messages.

UE in limited service state will not be in scope of this study item.

When considering the current market status, a significant number of UEs cannot support SMSoIP (e.g. satellite, various IoT devices) and would benefit from support of emergency SMS over NAS. Moreover, some HLOS platforms already support emergency SMS over NAS for AML for roaming UEs, and the authors believe it is useful to take advantage of such support.

The solution proposed in this paper is based on the architectures for SMS over NAS (see TS 23.272, TS 23.401, TS 23.501, TS 23.502) and the architectural principles of Emergency Services (see TS 23.167), and addresses all the aspects in the three WTs. For SMS over NAS there is a “template” for support of SMS to emergency number (112, 911) for roaming users that is used for transporting AML using SMS, with the solution documented in clause 6.4.2 and Annex D.2 of GSMA NG.119 (https://www.gsma.com/newsroom/wp-content/uploads//NG.119-V2.1-2.pdf). This is supported in current UEs and deployed:

-	It is based on the principle that the UEs (supporting AML for roamers), SMS client in HLOS will replace the current SMSC address with the visited SMSC address (+CC112/911). Destination number is still unchanged (Transaction Processing TP-Destination-Address=112/911 or local emergency numbers).

-	The MSC (when using SGs) or MME or SMSF in VPLMN will be responsible to route the SMS to the visited SMSC.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-65. The text in the third change is all new.

# >>>>BEGINNING OF CHANGES<<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[x]	3GPP TS 23.272: “Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2

[y]	3GPP TS 23.401: “General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”

[z]	3GPP TS 23.167: “IP Multimedia Subsystem (IMS) emergency sessions”

[w]	3GPP TS 23.221: “Architectural requirements”

[k]	3GPP TS 23.040: “Technical realization of the Short Message Service (SMS)”

[j]	3GPP TS 29.500: “5G System; Technical Realization of Service Based Architecture; Stage 3”

[l]	3GPP TS 23.540: “5G System: Technical realization of Service Based Short Message Service; Stage 2”

# >>>>NEXT CHANGE<<<<

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# |  |  |  |  |

# >>>>NEXT CHANGE<<<<

## 6.X	Solution #X: Support of emergency SMS over NA

### 6.X.0	High-level solution principles

This section describes support for the emergency Short Message Service (SMS) over NAS. This solution is based on the architectures for SMS over NAS (see TS 23.272 [x], TS 23.401 [y], TS 23.501 [2], TS 23.502 [3]) and the architectural principles of Emergency Services (see TS 23.167 [z]).

Emergency SMS over NAS in this context is SMS over NAS using an emergency number as destination address. A key requirement for this service is that the emergency Short Message (SM) must be routed to a PSAP in the country where the UE initiating the emergency SM is located. This implies that emergency SMS for roaming subscribers cannot be home routed while non-emergency SMS is typically home routed.

This solution applies to UE detectable emergency numbers.

### 6.X.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). (Sub) clause(s) may be added to capture details.

#### 6.X.1.1	General

This clause lists the additional functionality that is required to support emergency SMS over NAS for roaming and non-roaming users.

High level description and signalling flows are described in TS 23.272 [x] Annex C for EPS and TS 23.502 [3] for 5GS. Emergency SMS over NAS in 5GS is based on the architecture defined in clause 4.4.2. It can be supported over 3GPP and non-3GPP access.

There is no capability negotiation between the UE and AMF.  The UE that supports emergency SMS over NAS shall  replace the SMSC Address with the country code and the detected emergency number and the detected emergency number as destination address as defined in TS 23.167 [z] for the UE functionality.

#### 6.X.1.2	Service assumptions

The following service assumptions are considered for supporting emergency SMS over NAS:

-	The UE that supports emergency SMS over NAS and detects emergency SMS over NAS based on used emergency numbers. Provisioning of emergency numbers and associated types as specified in TS 23.167 [z] clause 4.1 bullets 2a, and 2b) apply.

-	SM messages from the PSAP to the UE are routed like any other SM.

-	Serving network can support emergency SMS over NAS for own subscribers and inbound roamers.

-	SMS Centre in the serving network supports emergency SMS over NAS.

-	There is no indication from the serving PLMN to the UE that Emergency SMS in supported. After the UE sends Emergency SMS over NAS it can get informed about the success or failure based on the existing SMS delivery notification using existing procedures.

#### 6.X.1.3	How the solution solves the key issues

How does the solution address the study item

-	How to identify a Short Message to Emergency Response Center based on the use of emergency numbers.

-	This is based on the existing routing of SMS and the emergency number used by the UE which is provisioned with emergency numbers and associated types. PLMNs already support detection of emergency numbers, and the SMSF or the MME selects the Emergency Response Center based on the emergency number

-	Address aspects when serving PLMN does not support this feature, but UE supports it.

-	Serving PLMNs are assumed to support emergency services to configured emergency numbers and thus support emergency SMS over NAS and the ability to identify a Short Message to Emergency Response Center

-	Whether and how to identify the type of emergency service (police, ambulance, fire brigade, etc.) using the same identification mechanism as for emergency calls.

-	The UE uses the provisioned emergency numbers which identify the type of service and the user selects e.g. from UI of the phone with mechanisms out of scope of 3GPP.

-	How to route a Short Message to Emergency Response Center (i.e. PSAP) serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location).

- this is based on existing mechanisms for routing of SMS, and in addition the SMSF or MME uses the UE location and the emergency number to select the PSAP.

#### 6.x.1.4	UE Functionality

The UE that supports emergency SMS over NAS shall support the functionality listed below (which needs to be added to TS 23.167):

-	Identify the need to send Emergency SMS over NAS based on the use of emergency numbers.

-	Performs domain selection for Emergency SMS over NAS as described in TS 23.221 [w].

-	Replaces the SC Address that is configured with country code of the UEs serving PLMN and the emergency number e.g.  +32112.

NOTE 1: If UE cannot determine country from serving PLMN, UE can use methods out of scope of the present solution to determine the county.

NOTE 2: After the UE sends Emergency SMS over NAS it can get informed about the  success or failure based on the SMS delivery notification using existing procedures.

#### 6.x.1.5	Domain Selection for UE originating SMS

To allow for appropriate domain selection for SMS delivery, the Domain Selection for UEs originating SMS in TS 23.221 [w] needs to be augmented with the following:

-	If the UE does not support emergency SMS over IMS, and if the UE supports emergency SMS over NAS and UE detects that the SM destination address is an emergency number as defined in TS 23.167 [z], the UE shall attempt to deliver the SMS over NAS signalling

### 6.X.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

The MO SMS over NAS in CM-IDLE (baseline) procedure in clause 4.13.3.3 of TS 23.502 [3] is modified as follows.

Figure 6.x.2-1: MO SMS over NAS

1.	The UE performs domain selection for UE originating SMS as defined in clause 5.16.3.8 of TS 23.501 [3] if SMS delivery via non 3GPP access is allowed and possible. If an UE under CM-IDLE state is going to send uplink SMS message, then UE and network perform the UE Triggered Service Request procedure firstly as defined in clause 4.2.3.2 to establish a NAS signalling connection to AMF.

2a.	The UE builds the SMS message to be sent as defined in TS 23.040 [k] (i.e. the SMS message consists of CP-DATA/RP-DATA/TPDU/SMS-SUBMIT parts). The SMS message is encapsulated in an NAS message with an indication indicating that the NAS message is for SMS transporting. The UE send the NAS message to the AMF. For emergency SMS the UE follows the procedures in TS 23.167 [z].

2b.	The AMF forwards the SMS message and SUPI to the SMSF serving the UE over N20 message by invoking Nsmsf_SMService_UplinkSMS service operation. In order to permit the SMSF to create an accurate charging record, the AMF adds the IMEISV, the current UE Location Information (ULI) of the UE as defined in clause 5.6.2 of TS 23.501 [2] and if the UE has sent the SMS via 3GPP access, the local time zone.

When the AMF determines that the UE has the MPS for Messaging indication set (enabled) in the UE context, the AMF includes a Message Priority header to indicate priority information. Other NFs relay the priority information by including the Message Priority header in service-based interfaces, as specified in TS 29.500 [j].

2c.	The SMSF invokes Namf_Communication_N1N2MessageTransfer service operation to forward SMS ack message to AMF.

2d.	The AMF forwards the SMS ack message from the SMSF to the UE using downlink unit data message.

3-5.	The SMSF checks the SMS management subscription data. If SMS delivery is allowed, the procedure defined in TS 23.040 [k] or TS 23.540 [l] applies. The SMSF sets proper DRMP value based on received Message Priority header in step 2. If the SMSF supports emergency SMS, and detects from the SMSC Address and the destination address contained in the SM the SMSF that the SM is for emergency, it sends the SM to the local SMS-GMSC/IWMSC for emergency.

6a-6b.	The SMSF forwards the submit report to AMF by invoking Namf_Communication_N1N2MessageTransfer service operation which is forwarded to UE via Downlink NAS transport. If the SMSF knows the submit report is the last message to be transferred for UE, the SMSF shall include a last message indication in the Namf_Communication_N1N2MessageTransfer service operation so that the AMF knows no more SMS data is to be forwarded to UE.

NOTE:	The behaviour of AMF based on the "last message indication" is implementation specific.

If the UE has more than one SMS message to send, the AMF and SMSF forwards SMS /SMS ack/submit report the same way as described in step 2a-6b.

6c-6d.	When no more SMS is to be sent, UE returns a CP-ack as defined in TS 23.040 [k] to SMSF. The AMF forwards the SMS ack message by invoking Nsmsf_SMService_UplinkSMS service operation to SMSF.

### 6.X.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP services, entities and interfaces.

UE:

- detects that the SM destination address is an emergency number and replaces the SC Address that is configured with country code of the UEs serving PLMN and the emergency number e.g.  +32112

SMSF/MME

- detects from the SMSC Address and the destination address contained in the SM the SMSF that the SM is for emergency, it sends the SM to the local SMS-GMSC/IWMSC for emergency.

# >>>>END OF CHANGES<<<<