SA WG2 Meeting #170	S2-2506506

25 – 29 Aug, 2025, Goteborg, SE

Source:	MediaTek Inc.

Title:	[WT#4, all topics] Integration of Sensing and Communication for 6G

Document for:	Agreement

Agenda Item:	20.6.4

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution addresses scoping of WT#4 on Integration of Sensing and Communication 6G. Motivation, Gap analyses and expected Technical Impact and suggests some potential scope for relevant Key Issue(s).

# 1. Justifications

5G-A study in Rel-20 has already started on Integrated Sensing and Communication, scoped mainly on gNB-based sensing, aligned with Rel-20 5G-A RAN ISAC study scope (see SP-250833):

-	WT-1: Architecture and function enhancements to support sensing.

-	WT-2: Service authorization and revocation. This includes investigating mechanisms for authorization and revocation sensing service requests from the service consumer.

-	WT-3: Discovery and selection of sensing entities based on service requirements triggered by the service request and capability of the sensing entities.

-	WT-4: Sensing data and the associated information collection and transport mechanisms for result calculation.

-	WT-5: Mechanisms for providing sensing associated information and result (including sensing result and contextual information) exposure, for one time, periodic or event-based reporting.

-	WT-6: Configuration parameters for the support of ISAC services.

Architectural Motivation and Gap analysis:

Existing 5G-A study in Rel-20 is mainly focused on gNB-based sensing.

A wider study in the context of 6G is expected to address different modes of sensing beyond BS-based sensing considering 6G use cases [see TR 22.870] in alignment with RAN WGs.

Technical Impact:

Possible impact expected on

- System architecture to support sensing

- Authorization and revocation to support sensing service

- Sensing entity and sensing function discovery/(re)selection

- Sensing Data and the Associated Information Collection and Transport

- Sensing result exposure

- Configuration Parameter Provisioning of Sensing Entities.

**** First Change ****

# Annex A.X. WT#4 Scope

Extracted below from SP-250806:

WT#4: Study the integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported and other sources of sensing data. NOTE 5: The detailed scope of WT#4 will be determined considering the scope and work of R20 FS_Sensing_ARC.

Note the below update is just intended as information in the TR and no change is expected in the WT wording in the SID.

This WT focuses on the integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported and other sources of sensing data based on 6G use cases [see TR 22.870] in alignment with RAN WGs.

Below aspects are to be studied:

-	Architecture and function enhancements to support sensing.

-	Service authorization and revocation. This includes investigating mechanisms for authorization and revocation sensing service requests from the service consumer.

-	Discovery and selection of sensing entities based on service requirements triggered by the service request and capability of the sensing entities.

-	Sensing data and the associated information collection and transport mechanisms for result calculation.

-	Mechanisms for providing sensing associated information and result (including sensing result and contextual information) exposure, for one time, periodic or event based reporting.

-	Configuration parameters for the support of sensing services.

NOTE 1: 	Data framework applicable for Sensing data collection and transport mechanisms or exposure mechanism will be addressed as part of as part of WT#5 of 6G SI on Data Framework.

NOTE 2: The detailed scope will be determined considering the scope and work of R20 FS_Sensing_ARC.

**** Second Change ****

# 5.X. Key Issue #X: Integration of Sensing and Communication

This key issue focuses on the integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported and other sources of sensing data based on 6G use cases [see TR 22.870] in alignment with RAN WGs.

Below aspects are to be studied:

-	Architecture and function enhancements to support sensing.

-	Service authorization and revocation. This includes investigating mechanisms for authorization and revocation sensing service requests from the service consumer.

-	Discovery and selection of sensing entities based on service requirements triggered by the service request and capability of the sensing entities.

-	Sensing data and the associated information collection and transport mechanisms for result calculation.

-	Mechanisms for providing sensing associated information and result (including sensing result and contextual information) exposure, for one time, periodic or event based reporting.

-	Configuration parameters for the support of sensing services.

NOTE 1: 	Data framework applicable for Sensing data collection and transport mechanisms or exposure mechanism will be addressed as part of as part of Key Issues related to WT#5 of 6G SI on Data Framework (see S2-250xxxx).

NOTE 2: The detailed scope will be determined considering the scope and work of R20 FS_Sensing_ARC.

**** End of Changes ****