SA WG2 Meeting #170	S2-2506509

25 – 29 Aug, 2025, Goteborg, SE

Source:	Boost Mobile Network, EchoStar, Telecom Italia S.p.A, China Mobile, Cisco Systems, SK Telecom

Title:	[General Aspects] 6G Architecture support Stateless NF Services

Document for:	Approval

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes that 6G system architecture shall aim to support cloud native features of all NF services being stateless.

# 1. Justifications

The 6G Study item objective says the following: “The study should consider at least the following aspects: cloud native, sustainability and energy efficiency, robustness and resiliency, etc.” Supporting cloud native has started in 5G where SBA has been supported in addition to restful interfaces. This contribution proposes that 6G system architecture supports the cloud native feature of stateless NF services.

# 2. Discussion

There are several types of benefits of cloud native technology that 6G architecture and deployment can benefit from. Some of the commercial benefits are reduced cost, faster time to market and enhance customer experience. Other benefits that improve the operational efficiency of the 6G architecture deployment are rapid provisioning of resources, increased agility and flexibility, fully automated and secure CI/CD, and In-service up-grade and downgrade. In addition, stateless services is one of the important yet critical features of cloud-native.

Having a 6G system supporting stateless NF services as an architectural principle provides many advantages which enable scalability, fault tolerance and resilience, simplified deployment and management, enhanced performance and efficiency, etc.

#### 2.1 Scalability:

- Having 6G system supporting stateless NF services ensures all 6G network functions support being stateless. This is fundamental for horizontal scalability where adding more servers or instances can handle increased load.

- Supporting stateless NF services means no session dependency and each request is independent and can be handled by any available instance where enhanced and efficient load balancing can be achieved.

- In addition, stateless NF services do not require server-side data session storage, thus frees up memory and CPU resources.

#### 2.2 Fault tolerance and resilience

- Supporting stateless NF services allows failure isolation where the failure on one server does not impact other servers.

- In case of failure, requests can be seamlessly routed to other available instances where service is not interrupted.

- Finally, it eliminates the need for complex state maintenance and replication and thus it provides easier recovery and failure management.

#### 2.3. Simplified deployment and management

- Stateless services have simpler configurations based on predefined rules or policies and don't need to maintain connection states.

- Stateless services can be deployed and updated independently allowing quick adoption for new features based on faster response to new or modified requirements.

- Debugging and troubleshooting are simplified as each transaction is treated independently, making it easier to pinpoint the source of issue.

#### 2.4 Challenges and Gap Analysis

- During the development of 5G system architecture, SA2 had an extensive discussion to have all 5GC network functions support full statelessness. Because of the real-time nature of 3GPP compliant mobile and telecom network, it has been concluded that some critical 5GC NF, e.g., AMF, SMF, may experience negative latency performance impact when accessing the session’s information due to increased singling.

- In other words, despite 3GPP has defined UDSF as a mechanism to achieve full statelessness for 5GC NF, yet UDSF solution has introduced latency to time-sensitive 5GC NFs, e.g., AMF and SMF, due to the extra signalling.

- In addition, vendors usually have vendor specific variables which not standardized and that means even if using UDSF solution to achieve 5GC NF full statelessness, this solution ends up being a vendor specific one.

- It is highly recommended that SA2 has an architectural target where all 6GC NFs and NF services are fully stateless while avoiding the above-mentioned shortcomings.

# 3. Conclusion

Although, services and network functions being stateless is not new to 3GPP standardization and systems, there is an increased need for 6G system architecture to adopt an architecture principle where it is mandatory for all NF services of the 6G system to support being stateless. This is driven by the need to have a highly scalable, resilient to failures, and easy to deploy and manage 6G system.

Stateless 6G system NF services and network functions address these demands by decoupling the session state from the application logic. This allows for a greater flexibility and efficiency in 3GPP mobile network environments

Finally, despite the challenges that SA2 experienced during 5G system architecture development as mentioned above, it is highly recommended that SA2 agree to have a 6G architectural principle where the target is for 6G network functions and NF services to support the cloud-native benefit where every NF service is fully stateless. This is important for the 6G system to garnish the cloud native benefits of high scalability, resilient to failures, and easy to deploy and manage 6G system.

# 4. Potential Architectural Requirements

- 6G system architecture shall aim to support the cloud-native benefit where every NF service is fully stateless

- It shall be documented why a specific NF service in the 6G system architecture cannot achieve full statelessness, e.g., performance impact.

# 5. Proposal

It is proposed to add the following architecture potential requirements to the TR 23.801 “Study on Architecture for 6G System”.

* * * Start of changes * * * *

# 4	Architectural Assumptions and Requirements

……….

……….

## 4.2	Architectural Requirements

Editor's Note: This clause defines the architectural requirements that serve as the foundation for the study.

- 6G system architecture shall aim to support the cloud-native benefit where every NF service is fully stateless.

- It shall be documented why a specific NF service in the 6G system architecture cannot achieve full statelessness, e.g., performance impact.

* * * End of changes * * * *