SA WG2 Meeting #170	S2-2506510

25 – 29 Aug, 2025, Goteborg , SE

Source:	China Mobile, SK Telecom, CATT

Title:	[WT#3, Agent for Network] Proposals of the Use of AI Agent in 6G Architecture

Document for:	Agreement

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This document elaborates on the limitations of the existing architecture in supporting AI-agent-related use cases and proposes an in-depth investigation into enabling the integration of AI agents in 6G architecture.

1. Introduction

As most MNO markets grapple with stagnant revenue growth and escalating operational costs, deeper integration of AI into communication systems will present substantial opportunities to extract more value from network infrastructure in the 6G era (e.g., reduce operational costs, develop novel AI-enabled network services, etc.).

AI agents, which are capable of autonomously performing tasks on behalf of users, systems, and/or applications, have received widespread attention in the realm of telecommunications.

The approved SA2 6G SID in SP-250806 contains the following WT, suggesting the potential role of AI agents in 6G architecture study:

WT#3: Study how to support and enable use of AI in 6G (e.g. AI agent, framework).

Many use cases related to AI agents have been proposed in 3GPP SA1, including AI-agents communication, AI agent collaboration, intent-driven network service provision, etc, further highlighting the great potential of integrating AI agents into 6G system for reducing operating expenditure and driving service innovation.

Accordingly, this document gives more detailed analysis of the limitations of the existing architecture in supporting AI-agent related use cases, particularly including customized service provision, network capability exposure and multi-agent collaboration, and then, several key issues related to AI agents are proposed.

2. Gap Analysis

Use Case #1：Customized service provisioning based on AI Agents

Figure 1: Customized service provisioning based on AI Agents

In this use case, a user interacts with a network AI agent via an operator portal (e.g., an app or a webpage) for desired services, e.g., experience assurance during a railway journey. Upon clarifying the user request, the network AI agent autonomously translates the user intent into an executable task plan, decomposes the plan into subtasks, assigns these subtasks to different NFs for execution, and finally generates network assurance policies for the user.

Due to the following gaps, the existing architecture could not support this use case:

- Lack a mechanism for intent interaction and understanding. The existing network architecture only supports to interact with UEs via strictly defined signalling, and only provides limited user perception capabilities (e.g., QoE analysis and mobility analysis related to a UE) to enable differentiated service provisions based on pre-configured rules. However, considering the diversity of user requirements, it is infeasible for the existing network to deliver services that exactly meet the user demands without human-interventions, and users have to spend a considerable amount of time choosing from calling packages that offered by the operators. By leveraging the semantic comprehension and reasoning capabilities of LLMs, AI agents could provide one-stop and intent-driven solutions for the service and operation plan design, selection, and enables autonomous network resource (e.g., QoS, power, etc.) allocation via initiating corresponding signalling procedures, thereby enhancing the ability of networks to offer more personalized services.

- Lack a mechanism for flexible capability coordination. In existing network architecture, the interaction patterns among NFs are rule-based and pre-programmed, aiming at providing highly deterministic services in controlled and well-defined scenarios, such as mobility management and session management, etc. Yet, in the face of sudden, atypical service scenarios, this architecture reaches its limits, necessitating human intervention that inevitably introduces additional complexity and delay the launch time. This gap is particularly prominent in the existing AI framework centred on NWDAF. Once new analytic services need to be introduced, the NWDAF and other supporting NFs have to be upgraded, such as defining new analytics IDs, new interfaces and designing new service flows. However, given the emergence of novel use cases in evolving networks, exhaustive coverage of all potential analytic tasks is infeasible. By introducing network AI agents and defining network capabilities as tools that can be invoked by AI agents, the design of new services will be simplified and accelerated by leveraging the autonomous task planning and execution capabilities of network AI agents, enabling a more adaptive and agile system architecture.

Use Case #2: Unified network capabilities invocation for AI agents

Figure 2: Network capabilities exposure for AI agents

In this use case, through a unified tool-calling framework, the network could expose its capabilities as tools to trusted third-party AI agents, and third-party tools could also be seamlessly invoked by network AI agents. When new tools are introduced, the AI agent is capable of autonomously discovering them and figuring out how to use them without human intervention, thereby enabling flexible capability expansion for itself.

As a new type of network service consumers, AI agents were not taken into account in the initial design of the existing network capability exposure and invocation framework, therefore inevitably leaving gaps between the existing architecture and the requirements of emerging AI agents.

- Lack a unified network capability invocation mechanism for AI agents. The existing architecture only supports to expose network capabilities to third-party via NEF. However, the present NEF interface is highly specialized and is not compatible to the existing AI agent ecosystem (e.g., tool invocation via MCP), which inevitably induces additional complexity for AI agent design. Furthermore, the existing architecture lacks a network capability discovery mechanism through the NEF, consequently, when novel capabilities are introduced into the network, manual updates to third-party AI agents are imperative, or these capabilities remain inaccessible  to the agents. Moreover, it is also essential for network AI agents to access third-party resources (e.g., the road map of the city, the train schedule, etc) to support the understanding and interpretation of user intents. Accordingly, designing a unified tool-calling framework to simplify the cross-domain capability access for both network and third-party AI agent is significant for accelerating the integration of telecommunications industry and AI agent ecosystem.

Use Case #3：Collaborative task execution among multiple AI agents

In this use case, a user initiates a service request to the network assurance agent to enhance service experience. Upon receiving the request, the assurance agent first queries the user profiling agent for user-specific information—including current identity (e.g., high-speed railway passenger), location, subscription details, and device capabilities—to formulate an optimal assurance policy tailored to the user.

- Lack a mechanism to support collaborative task execution among multiple AI agents. Compared with single-agent system, multi-agent system (MAS) allows coordinating multiple agents to achieve a common goal, where each agent could be designed to handle a specific domain task. This distributed structure enables the system to tackle more complex problems and scale more efficiently by dividing tasks across agents. Considering the distributed nature of network architecture, the complexity of user-initiated service requests, and the high dynamics of network conditions, supporting collaborative task execution among multiple AI agents in 6G architecture could be of vital importance for unlocking its full potential. However, the existing architecture merely supports NWDAF collaboration for specified analytics tasks through horizontal/vertical federated learning and analytics aggregation. For multi-agent collaboration, new mechanisms—encompassing autonomous task planning/decomposition and agent discovery/communication—are required.

Figure 3: Collaborative task execution among multiple AI agents

3. Technical Impact

- The AI agent can be introduced into 6GC as a standalone entity to handle intent input, including intent understanding, clarification and translation, task planning, and network capability coordination.

- New mechanisms should be designed to enable the AI agent to flexibly interact with network functions.

- New mechanisms should be designed to enable goal-directed collaborations among multiple AI agents, possibly including network AI agents, third-party AI agents and terminal AI agents.

- The 6G system should support a unified framework to enable flexible network capability exposure for AI agents.

**** First Change ****

Annex A.X. WT#3 Scope

The scope of WT #3 is presented in SP-250806. No update to the Working task description

WT#3: Study how to support and enable use of AI in 6G (e.g. AI agent, framework).

NOTE 4: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

**** Second Change ****

5.X. Key Issue #X: Support of AI Agent in 6G Architecture

It is proposed to study the following aspects related to AI agents in 6GS:

-	Study how to define the network functionalities of AI agents, such as intent understanding, intent translation, task planning, network capability coordination, etc.;

-	Study how the network functions supporting AI agents interact with other network functions in 6GC, including the interfaces, the mechanism to ensure the security/reliability for interactions, etc.;

-	Study how to support the registration, discovery and update of network AI Agents;

-	Study how to coordinate the task execution among multiple AI Agents, including the interfaces, the task decomposition mechanism, etc.;

-	Study how the 6G system exposes network capabilities to support trusted third-party AI agents and terminal AI agents, including the interfaces, authorization/authentication mechanism, etc.;

-	Study how the network AI agents invocate the capability provided by trusted third-parties, including the capability registration, discovery and authentication mechanisms, etc.

**** End of Changes ****