3GPP TSG SA WG2 Meeting #170	S2-2506514

25 – 29 Aug, 2025, Goteborg , SE	(revision of S2-250xxxx)

Source:	China Mobile

Title:	Solution #14 update on UE data collection over the user plane

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: Proposed the update for Solution #14: UE data collection over the user plane.

1. Discussion

This proposal proposes to update solution #14: UE data collection over the user plane on the following aspects.

- 1. To achieve differentiation of the traffic for model training for charging purposes, when the UE initiates a PDU session establishment procedure , the UE will indicate to the 5GC the new PDU session is used only for UE data collection, With such indication, 5GC may charge the user differently for the traffic associated with this PDU session as compared to traffic in other PDU sessions.

- 2. The EN is removed since it is no doubt that the UE performs data collection after PDU session establishment,.

Editor's note:  Whether to trigger the UE to perform data collection before or after PDU session establishment is FFS.

- In step3, NWDAF-C will provide the UMTES address information to the NWDAF-U or ADRF or DCCF, so the EN can be removed.

Editor's note: How to deliver the data to the UMTES is FFS.

- 4. One new NOTE is added to say ”The entity to perform UE data collection could be NWDAF or ADRF or DCCF” for making Clarification, so the EN can be removed.

Editor's note: Which entity to perform UE data collection is FFS.

- Text Proposal

It is proposed to capture the following changes in TR 23.700-04.

# * * * * First change * * * *

6.14	Solution #14: UE data collection over the user plane

6.14.1	High level principles

The high level principles of the solution are:

-	The NWDAF-C directs AMF to initiate the UE data collection procedure and provides the IP address of NWDAF-U or ADRF or DCCF to UE.

-	The NWDAF-U or ADRF or DCCF receives the collected UE related data from UPF via N6 or N9 interface.

-	To report the uplink data to NWDAF-U or ADRF or DCCF received from the NWDAF-C, the UE may request to initiate a new PDU session.

In this solution, the NWDAF could be splited into NWDAF-C and NWDAF-U in case that NWDAF-U is used to store data collected. The NWDAF-C is responsible for control plane, e.g. handling signalling of the data collection request by AF, triggering the UE to establish the PDU session and provide some information e.g. about data transfer session to the NWDAF-U while the NWDAF-U is responsible for UE data collection, data storage, data processing (e.g. data verification) and data exposure, etc.

6.14.2	Description

This solution is to address Key Issue#1 for UE data collection over UP.

6.14.3	Procedures

Figure 6.14.3-1: Procedure for UE data collection over user plane

1.	The UMTES (UE Model Training Entity Server) sends the UE data collection request including UE ID(s), area of interest and time duration to 5GC functions, e.g. NWDAF-C via the NEF. The request is authorized by the NEF for the untrusted AF.

2.	The NWDAF-C checks the UE consent for data collection for these UE ID(s) and if it is granted, the NWDAF-C stores the UE ID(s), area of interest and time duration and determines the AMF(s) based on the UE ID or the UE ID list.

3.	NWDAF-C provides information about data transfer session to the NWDAF-U or ADRF or DCCF including the UMTES address information.

4.	The NWDAF-C sends the UE data collection request to the UE via the AMF including UE data collection indicator and NWDAF-U or ADRF or DCCF IP Address.

5.	Based on UE data collection indicator, the UE may initiate a PDU session establishment procedure , indicating to the 5GC the new PDU session is used only for UE data collection, With such indication, for example, 5GC may charge the user differently for the traffic associated with this PDU session as compared to traffic in other PDU sessions.

6.	After PDU session establishment, the UE sends the UE data collection response to NWDAF-C and NWDAF-C responses to UMTES via the NEF.

7.	The UE performs UE data collection and sends the uplink data to the NWDAF-U or ADRF or DCCF, whose IP Address is received in step 3, which stores the UE data.

8.	Based on the UMTES address informaiton proved by NWDAF-C in step 3, the NWDAF-U or ADRF or DCCF exposes the data to UMTES optionally via the NEF for the untrusted AF.

NOTE X:	The entity to perform UE data collection could be NWDAF-U or ADRF or DCCF.

6.14.4	Impacts on services, entities and interfaces

NWDAF-C:

-	Supports to handle signalling of the data collection request by UMTES and trigger the UE to establish the PDU session.

-	Sends the UE data collection request to UE via the AMF including UE data collection indicator and NWDAF-U Address.

NWDAF-U or ADRF or DCCF:

-	Supports to perform UE data collection, data storage, data processing, data exposure, etc.

-	Receives the UE data from UPF via N6 or N9 interface and stores the data.

-	Exposes the data to UMTES via the NEF.

UE:

-	Initiate a PDU session establishment procedure based on UE data collection indicator.

-	Performs UE data collection and sends the uplink data to NWDAF-U.

# * * * * End of Changes* * * *