SA WG2 Meeting #170	S2-2506517

25 – 29 Aug, 2025, Goteborg , SE

Source:	Deutsche Telekom, AT&T, Cisco Systems, KPN N.V., Rakuten Mobile, Sateliot, SK Telecom, T-Mobile USA, Telecom Italia

Title:	[WT#1.1] NAS for 6G

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution covers the non-access stratum functionality and provides an overview of current limitations fully or partially stemming from the current architecture, based on which a scope for the related (sub-)Work Task (WT) and related Key Issue (KI) are proposed.

# 1. Justifications

In this contribution, we aim at providing an overview of current limitations fully or partially stemming from the current architecture, based on which a scope for the related (sub-)Work Task (WT) and related Key Issue (KI) are proposed. We present justifications reflecting learnings gained during the last years and we propose a WT scope and KI for 6G based on scope described in SP-250806.

We see a need to develop generic mechanisms for UE-Core Network (CN) interaction to support various services, such that functionality can be defined in a modular way (i.e. with as little interdependencies as possible). This includes also a means support different sets of functionalities (e.g. a minimum set). The new mechanism should ensure that UE-CN interaction is distributed across multiple CN functions and each function can be deployed, configured, monitored and scaled independently.

## 1.1	Current functional split and architecture for UE-CN interaction

Interactions between the UE and the CN are realized by the N1 interface, which corresponds to Non-Access Stratum (NAS) functionality. The NAS protocol, in its 5G iteration, is described in TS 24.501 and contains hundreds of Information Element (IE) definitions, for which thought was given to how big each IE should be, and (currently) spans more than 1,200 pages. It realizes functionality related to Mobility Management (MM), Session Management (SM), SMS, UE Policy and Location Services (LCS).

As it is currently designed, all NAS functionality is relies on (i.e. depends on) NAS-MM. As shown in Figure 1.1-1, NAS-MM forms the basis of the NAS protocol stack, with other types of NAS building on top of it.

Figure 1.1-1: NAS transport for SM, SMS, UE Policy and LCS, adapted from Figure 8.2.2.1-1 of TS 23.501

The N1 interface is dependent on the N2 interface for transport (for more details on current N2 limitations, see S2-2506518). Figure 1.1-2 shows how NAS-SM message are transported between the UE and the SMF. The same method is used with other NAS types terminating at SMSF, PCF, GMLC.

In order to transport NAS containers over SBI, MIME Multipart within the JSON body and a “3GPP” vendor (see also RFC 6838) are used:

- application/vnd.3gpp.5gnas, application/vnd.3gpp.ngap

- application/vnd.3gpp.lpp, application/vnd.3gpp.sms

Figure 1.1-2: Control Plane protocol stack between the UE and the SMF, adapted from Figure 8.2.2.3-1 of TS 23.501

The AMF plays a critical role in UE-CN interactions, and it is tasked with, among other functions, the following functionality relevant to this WT (see clause 6.2.1 of TS 23.501):

- Termination of RAN CP interface (N2)

- Termination of NAS (N1)

- NAS ciphering and integrity protection

- Registration management

- Connection management

- Reachability Management

- Mobility Management

## 1.2	AMF Functional Dependency for any NAS functionality

As described in section 1.1, NAS-MM is the basis for all other NAS functionality. This results in a dependency between the NF providing NAS-MM functionality (i.e. the AMF) and other NFs providing other NAS functionality (SMF, SMSF, PCF, GMLC), even if TS 23.501 states “RM/CM NAS messages in NAS-MM and other types of NAS messages (e.g. SM), as well as the corresponding procedures, are decoupled”.

This results in basically all NAS-related functionality changes having an AMF impact. While 5G-NAS did introduce some concepts of modularity allowing for different functionality built on top of NAS-MM, the dependency on NAS-MM and the AMF further contributes to the difficulties in deploying a scalable, resource-efficient, cloud-native AMF described in S2-2506518.

A single N1 interface for carrying all the NAS information results in a scaling limitation for a large-scale system where one aims to achieve independent deployment and scaling of different functions, e.g. MM and SM functionality.

An AMF failure results in a total failure of UE connectivity (CP and UP). Hence, failover mechanisms combined with (over-)dimensioning such that no bottleneck is possible, i.e. with enough spare capacity and redundancy to minimize the possibility of an outage, are necessary in operator deployments.

In terms of functionality, providing a means to make NAS-MM and other NAS modules fully independent would allow the following objectives described by SP-250806:

- Implementing a UE with a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities

- Help with enabling the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities (see also section 1.3)

As a consequence of the current functional split, security for non-access stratum functionality relies of hop-by-hop security. Security between the UE and the CN is only possible up to the AMF, with anything behind the AMF being implicitly trusted by the UE.

### 1.2.1 Summary

Architectural Motivation: Be able to deploy, configure, upgrade and scale a given NAS functionality such that the AMF does not need to always support or is impacted by the NAS functionality.

Gap Analysis: Currently, all NAS functionality is built on top of NAS-MM, i.e. dependent on AMF

Technical Impact: Following impacts are expected to be covered to resolve this gap:

- Functional split between AMF and other CN NFs regarding non-access stratum functionality

- Interactions between MM and non-MM NAS functionality to orthogonalize non-access stratum functionality (i.e. accomplish independence of NAS functionality from NAS-MM)

- UE impact should be minimized

- Identification of possible security impacts to functional split at various affected CN NFs such that SA3 can work on them

- Optimal NF service design such as service authorization for allowing/denying NAS services at various CN NFs can be studied by SA3

- Identification of failure scenarios involving partial NAS failures, as opposed to the current single failure scenario (e.g. loss of NAS- MM as a CN anchor point limiting mobility impact to other CN NFs)

- Identification of impacts related to Timers, Cause Codes

- Interworking and migration to/from 6G

## 1.3	AMF Topological Dependency for any NAS functionality

Similarly to the case for N2 (see S2-2506518), a result of the AMF termination mandated by the current architecture is that NAS-SM, UE policy, SMS and LCS NAS messages must be routed via the AMF. With the current architecture, this limitation will also extend to any future non-access stratum functionality.

In 5G, Non-Public Network (NPNs, see TR 23.734, TR 23.700-07), deployments considered scenarios where localized and centralized deployments are combined. Additionally, Non-Terrestrial Networks (NTNs) raised the question of what core network elements/network functions that should be placed on board the satellite(s) (see TR 23.700-29).

The fact is, that the 5GS aims to cover deployment scenarios ranging between a centralized deployment, and a distributed deployment where network components maybe be tens of thousands of kilometres apart (e.g. terrestrial, LEO, MEO, GEO).

Figure 1.1.3-1: Deployment diversity the 3GPP system aims to cover

Depicting both the issues of the underlying N2 transport detailed in S2-2506518, and the described N1 topology restrictions, for the case of NAS-SM (similar case for PCF, SMSF, GLMC and any future non-access stratum functionality), operators are restricted to a topology shown in the picture below.

Figure 1.1.3-1: CN and RAN topology depicting the N2-SM and NAS-SM containers

We consider it a valid point to question whether having the AMF as single-point-of-entry in the CN restricts deployment possibilities and whether 6G could provide enhancements in this regard, both in terms of reliability and deployment topology.

To enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities, the study should cover that non-access stratum functionality can be provided without having to pass via the AMF. The reasoning being that such a topological limitation impacts for the AMF deployment, latency, load and dimensioning.

### 1.3.1 Summary

Architectural Motivation: Be able to deploy NAS functionality such that AMF failure does not impact functionality unrelated to registration/MM and also that CN elements can be efficiently distributed.

Gap Analysis: Currently, NAS must be routed via the UE’s AMF, NAS security termination is performed by the AMF

Technical Impact: Following impacts are expected to be covered to resolve this gap:

- Functional split between gNB and CN regarding the transport of N1 messages over N2

- How NAS messages can be routed to CN FNs, including interactions between UE and CN and gNB and CN

- Identify possible security impacts such that SA3 can work on them (e.g. implications of loss of topology hiding if the gNB can reach an SMF)

- Optimal NF service design such that service authorization for allowing/denying NAS services at various CN NFs can be studied by SA3

- Identify additional RAN complexity if NAS routing functionality is moved to RAN

- Identification of impacts related to Timers, Cause Codes

- Interworking and migration to/from 6G

## 1.4	Linkage with interworking aspects

Since the CN for 6G is may end up re-using elements of the 5GC and 5G SA adoption may increase during the next years, we think that it is important to include a linkage to WT#2, such that solutions must either include interworking aspects or explain why interworking is not necessary.

**** First Change(all text new) ****

# Annex A.1.1.X WT#1.1 Control signalling for 6G services

WT 1.1 covers the study of the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

This topic is scoped by the WT descriptions in Table A.1.2.X-1.

Table A.1.2.X-1: Work tasks for non-access stratum functionality

| Work Task ID | Work Task(s) | Work Task Description |
| --- | --- | --- |
| NAS_WT_#1.1.1 | NAS modularization | - How can other NAS functionality be decoupled from NAS-MM - Interactions between UE and CN - Interactions between UE and RAN, as well as between RAN and CN necessary to achieve 1.2 (NOTE V, NOTE W, NOTE X, NOTE Y) |
| NAS_WT_#1.1.2 | NAS message routing | 2.1 How NAS messages be routed between the UE and a target NF in the NF 2.2 Interactions between UE and CN 2.3 Interactions between UE and RAN, as well as between RAN and CN necessary to achieve 2.2 (NOTE V, NOTE W, NOTE X, NOTE Y) |
| NOTE V: Topology hiding and security aspects are to be evaluated by SA3 NOTE W: Work on the N2 interface regarding transport of NAS messages to be conducted together with RAN3 NOTE X: Interworking aspects for NAS are covered by WT#2. NOTE Y: It is acknowledged that performance impact related to message encoding is an important aspect, which is in CT1 scope |  |  |

Editor’s Note: N2  refers to functionality and interface for 6G. The final terminology (e.g. 6G-N2, eN2, etc.) has yet to be decided.

**** Second Change (all text new) ****

# 5.X. Key Issue #X: Control signalling for 6G services

In 5G, NAS provides signaling between the UE and the 5GC, is based on a single N1 NAS signalling connection, which is used for each access to which the UE is connected. The single N1 termination point is located in the AMF, and also provides transport to SM, SMS, UE policy and LCS-related functionality, as described in clause 8.2.2.1 of TS 23.501 [X].

This key issue focuses on how to enable:

-	the introduction of a new non-access stratum functionality in a variety of deployment scenarios without impacting other non-access stratum functionalities, especially without impacting the AMF

-	identifying a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities, especially dependency on the AMF

Solutions for this key issue will describe, based on the WT scope described in Annex A.1.1.X WT#1.1 Control signalling for 6G  services:

-	How NAS functionality can be specified independent of each other and minimizing dependencies between NFs

-	How NFs providing NAS functionality can be flexibly and efficiently deployed

-	What interactions between UE-and-CN, UE-and-RAN and RAN-and-CN are required

Solutions are expected to include interworking aspects (see KI#Y) or argument why interworking aspects are not necessary to be considered.

Editor’s Note: N1, N2, AMF all refer to functionality and interfaces for 6G. The final terminology (e.g. 6G-N2, eN2, etc.) has yet to be decided.

**** End of Changes ****