SA WG2 Meeting #170	S2-2506519

25 – 29 August, 2025, Goteborg, Sweden

Source:	InterDigital Inc.

Title:	[WT#1.3, Multi-Access Communications] Multi-Access Communications

Document for:	Agreement

Agenda Item:	20.6.1.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides more details about what can be studied within Work Task #1.3, on the topics related to Multi-Access Communication.

# 1. Justifications

Work Task #1.3 of the FS_6G_ARC study (SP-250806) states: "Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access”.

This contribution addresses the “multi-access data connections" aspect of WT#1.3.

Multi-access data connection functionality is an integral part of the 5G System. The functionality is used in a number of features developed by RAN and SA. In particular, the 5G System supports Access Traffic Steering, Switching, and Splitting (ATSSS), where the multi-access is between a 3GPP access and a non-3GPP access, and where the multi-access is controlled at the granularity of a service data flow. The UE manages splitting, switching, steering, and duplication of uplink traffic, while the UPF manages splitting, switching, steering and duplication of downlink traffic. Traffic decisions are based on conditions (e.g. round trip time, packet loss rate, availability, etc.) of each access. SA2 also completed a study item related to support of DualSteer for the 5G System. In DualSteer, the multi-access is between two 3GPP accesses. However, no normative work was pursued at the completion of the SA2 study item.

Architectural Motivation: What architectural challenges or opportunities does this WT/KI address?

Leverage the unique characteristics of the accesses supported by devices.

SA1 has identified a number of use cases and service requirements that leverage the multi-access capabilities of devices. As part of the DualSteer study (see TR 22.841 and TS 22.261), use cases highlighted the diverse combinations of 3GPP access networks using the same or different RATs, including terrestrial NR plus NR, or NR plus E-UTRA (e.g. using a combined EPC and 5GC), a mix of terrestrial and non-terrestrial NR, as well as dual non-terrestrial NR access (using same or different NTN orbits, e.g., GEO/MEO/LEO). The use cases also highlighted combinations where the two 3GPP access networks connect to the same or different PLMN networks or the same or different NPN networks. One interesting observation from these use cases, is that the network may “smartly” use the very unique characteristics of each access network. For example, switch certain traffic over an NTN access network that has a large coverage area.

6G presents an opportunity to design a single baseline multi-access system supporting all the different access network and mobile network combinations (e.g. access networks: non-3GPP, 3GPP (LTE, NR, 6G, TN, NTN) & mobile networks: (HPLMN, VPLMN, NPN)). Furthermore, the multi-access system may take advantage of the diversity of the access networks. That is, leveraging the unique characteristics of each access network, to allow “smart” steering, switching, splitting and duplication decisions.

Gap Analysis: What are the limitations or gaps in existing architecture (from Rel-15 to Rel-20) that this proposal aims to resolve?

Leverage the unique characteristics of the accesses supported by devices.

This paper proposes to update the work task to include studying whether and how to design a single multi-access system that supports all access network and mobile network combinations (e.g. access networks: non-3GPP, 3GPP (LTE, NR, 6G, TN, NTN) & mobile networks: HPLMN, VPLMN, NPN). That is, supports both ATSSS-like behaviour and DualSteer-like behaviour. In addition, the work task includes studying new conditions to help make traffic decisions.

Technical Impact: How would this WT/KI influence existing 5G functionality?

- The overall system architecture would be impacted to deal with how QoS is handled for a device that is using multiple accesses, including support for new QoS capabilities in 6G. Furthermore, more consideration should be given if the device is treated as DualSteer device with multiple UEs.

- The WT/KI would impact registration management (RM), session management (SM), policy management (PM), and mobility management (MM) in order to deal with the device obtaining service over multiple access networks (which may be simultaneous).

- The interaction between network functions would be impacted to deal with the changes to RM, SM, PM, and MM procedures.

**** First Change ****

# Annex A.X. WT#1.3 Support multi-access data connections

One aspect of this work task is to study whether and how to support and/or enhance support of multi-access data connections in 6G. Aspects that can be studied include:

- Whether and how to enable multi-access connections, where each access may be over any access network (e.g. non-3GPP, 3GPP (LTE, NR, 6G, TN, NTN)) and mobile network (e.g. HPLMN, VPLMN, NPN) combination.

This may include a number of sub-tasks:

- Study whether and which new conditions are needed help make traffic decisions (selecting one access over another access),

- Study impacts to QoS handling, access control, registration management, session management, policy management, and mobility management for devices supporting the multi-access connections.

**** Second Change ****

# 5.X. Key Issue #X: Support multi-access data connections

This key issue focuses on how to support multi-access data connections in 6GS. The key issue includes:

- Study whether and how to enable multi-access connections, where each access may be over any access network (e.g. non-3GPP, 3GPP (LTE, NR, 6G, TN, NTN)) and mobile network (e.g. HPLMN, VPLMN, NPN) combination.

This may include a number of sub-tasks:

- Study whether and which new conditions are needed help make traffic decisions (selecting one access over another access),

- Study impacts to QoS handling, access control, registration management, session management, policy management, and mobility management for devices supporting the multi-access connections.

**** End of Changes ****