SA WG2 Meeting #170	S2-2506520

25 – 29 Aug, 2025, Goteborg , SE

Source:	InterDigital Inc.

Title:	[WT#7, NTNs] Non-Terrestrial Networks

Document for:	Agreement

Agenda Item:	20.6.7

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides more details about what can be studied within Work Task #7 on the topics related to service continuity.

# 1. Justifications

Work Task #7 of the FS_6G_ARC study (SP-250806) states: "Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects”.

This contribution addresses the “service continuity" aspect of WT#7.

One of the key goals of 6G is to provide ubiquitous connectivity and seamless service continuity. From Recommendation ITU-R M.2160-0, this "would provide digital inclusion for all by meaningfully connecting the rural and remote communities, further extending into sparsely populated areas, and maintaining the consistency of user experience between different locations including deep indoor coverage."

NTNs are ideally positioned to provide ubiquitous connectivity, but ensuring seamless service continuity between terrestrial networks and NTNs, or between NTNs, is difficult to achieve with current 5G NTN technology.

Architectural Motivation: What architectural challenges or opportunities does this WT/KI address?

Motivation 1 – Leverage the multi-orbit deployments for service continuity and ubiquitous coverage

5G NTN deployments suffer from Unavailability Periods. These unavailability periods occur during initial deployments stages, or due to decommissioning of satellites, or due to satellite failures. During these unavailability periods, a UE will not have access to the satellite, and the 5G NTN will not meet the requirements for either ubiquitous coverage or service continuity. Multi-orbit deployments can be used to counter the unavailability periods. For example, relying on a satellite in a higher orbit (with larger coverage) when a lower orbit satellite is decommissioned.

6G presents an opportunity in that the baseline system can be designed to efficiently use multi-orbit deployments to provide ubiquitous coverage and seamless service continuity.

Motivation 2 – On-board network functions

IoT NTN deployments enable Store and Forward (S&F) satellite operation. Such deployments enable an MME (or part of an MME) to be deployed on-board a satellite. This main goal is to deal with the case when a satellite feeder link and satellite service link are not available at the same time. For example, when a service link is available, the satellite stores the message received on the service link. Then later when the feeder link becomes available, the satellite forwards the message over the feeder link.

6G presents an opportunity in that the baseline system can be designed to allow flexible deployment and selection of on-board network functions. This would allow reduction in signaling latency, help manage the unavailability periods, address the S&F operation, and simplify UE-satellite-UE communications.

Gap Analysis: What are the limitations or gaps in existing architecture (from Rel-15 to Rel-20) that this proposal aims to resolve?

Gap 1 – Service Continuity not clearly defined

This paper proposes to update the work task to detail the specific scenarios for service continuity: TN-NTN, NTN-TN, NTN-NTN (across various orbits)

Gap 2 – Leverage multi-orbit deployment

This paper proposes to update the work task to include studying whether and how to enable multi-orbit deployments.

Gap 3 – On-board network functions

This paper proposes to update the work task to include studying whether and how to enable flexible deployment and selection of on-board network functions.

Technical Impact: How would this WT/KI influence existing 5G functionality?

- The overall system architecture would be impacted to enable multi-orbit deployments. One aspect is the selection of NFs on board the satellites.

- With multi-orbit deployments, the WT/KI would impact network/orbit selection, and routing of traffic using inter-satellite links.

**** First Change ****

# Annex A.X. WT#7 Support of NTN

One aspect of this work task is to study whether and how to support and/or enhance support of support service continuity aspects in 6G NTN. Aspects that can be studied include:

- Study whether and how to enable Service Continuity for various cases: TN-NTN, NTN-TN, NTN-NTN (across various orbits)

- Study whether and how to enable deployment and selection of NFs on-board satellites

- Study whether and how to enhance network/orbit (re)selection and cell (re)selection in multi-orbit deployments

- Study whether and how to set up and maintain ISLs for multi-orbit deployments

**** Second Change ****

# 5.X. Key Issue #X: Support of NTN

This key issue focuses on how to support NTN in 6GS. The key issue includes:

- Study how to support 6G RAT for NTN

Note: this is based on RAN decision

- Study whether and how to enable Service Continuity for various cases: TN-NTN, NTN-TN, NTN-NTN (across various orbits)

- Study whether and how to enable deployment and selection of NFs on-board satellites

- Study whether and how to enhance network/orbit (re)selection and cell (re)selection in multi-orbit deployments

- Study whether and how to set up and maintain ISLs for multi-orbit deployments

**** End of Changes ****