SA WG2 Meeting #170	S2-2506521

25 – 29 Aug, 2025, Goteborg, SE

Source:	InterDigital Inc.

Title:	[WT#1.1, beyond connectivity services] Support for an Identity Service

Document for:	Agreement

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This p-CR proposes to include an identity service as one of the “Beyond Connectivity Services” that are in scope of work task 1.1.

# 1. Justifications

Work Task 1.1 states:

1.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

This p-CR proposes to update the work task to include an identity service as one of the “beyond connectivity services”. Although the sub-bullets of WT#1.1 focus on NAS services, it is the understanding of authors of this paper that not all services within this work task are necessarily a NAS services.

Architectural Motivation:

By allowing for the creation and utilization of identities that identify a user of a UE or an application instance that runs on the UE, operators will be able to provide enhanced user experience, optimized performance, and offer services to users and application providers. For example, network settings can be adapted, and services can be offered to application providers and users according to their needs, which may be different from the default needs of the UE subscription that was used to establish the connection. These identities will allow the operator to charge and provide service differentiation based on the identifier.

Gap Analysis:

Requirements for identifying UE applications and users have existed since Rel-16 (e.g. to TS 22.101 and TS 22.115 as part of the UIA (SP-180328) work item). In Rel-19, SA2 studied how to add support for human user identities (TR 23.700-32 as part of the FS_UIA_ARC (SP-231804) study). SA2 was not able to conclude on how to best add support for identifying the user of a UE. One challenge that companies could not agree on was how to best add support for user identities in the Rel-19 5G System in a backward compatible way.

This 6G study presents an opportunity because SA2 can consider supporting an identity service from the first release of 6G.

Technical Impact: How would this WT/KI influence:

- The features would likely impact the overall system architecture because identities are provided by the UE, verified by the network, and associated with UE traffic.

- The study would need to address what happens when a 6G UE moves to 5G.

**** First Change – All New Text ****

# Annex A.X. WT#1.1 Scope of a Beyond Connectivity Service for Identities

One of the beyond connectivity services that is in scope of work task 1.1 is support of an identity service. This part of the work task includes studying the following aspects of a 6G identity service.

-	Definition of the architectural assumptions that are necessary to support associating an identifier with all of a UE’s traffic or only the traffic of a UE application.

-	What information is stored as part of an identity profile (e.g., an identifier, associated security credentials, associated devices, user specific QoS settings). Including how profiles are acquired, stored, and updated in the network.

-	Whether and how identifiers are linked and unlinked (i.e., associated) with subscriptions in an operator-controlled manner.

-	Whether and how identity specific QoS settings are considered in order to provide service differentiation when providing communication services.

-	How identities are verified and authorized.

NOTE 1:	Aspects of the study of this service will depend on interaction with SA WG3.

NOTE 2:	Solutions to this part of the work task do not need to be NAS based (i.e. an identity service does not need to be NAS based).

**** Second Change – All New Text ****

# 5.X. Key Issue #X: Beyond Connectivity Service for Identities

This key issue is about whether and how an identity service can be supported in the 6G system. The key issue covers the following topics.

-	What architectural assumptions should be established to support associating an identifier with all of a UE’s traffic or only the traffic of a UE application.

-	What information is stored as part of an identity profile (e.g., an identifier, associated security credentials, associated devices, user specific QoS settings). Including how profiles are acquired, stored, and updated in the network.

-	Whether and how identifiers are linked and unlinked (i.e., associated) with subscriptions in an operator-controlled manner.

-	Whether and how identity specific QoS settings are considered in order to provide service differentiation when providing communication services.

-	How identities are verified and authorized.

NOTE 1:	Aspects of this key issue will depend on interaction with SA WG3.

NOTE 2:	Solutions to this key issue do not need to be NAS based (i.e. an identity service does not need to be NAS based).

**** End of Changes ****