SA WG2 Meeting #170	S2-2506522

25 – 29 Aug, 2025, Goteborg, SE

Source:	InterDigital Inc.

Title:	[WT#1.2, Network Exposure Framework] Network Exposure Framework

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides more details about what can be studied within Work Task #1.2 on the topic of Network Exposure Framework.

# 1. Justifications

Work Task #1.2 of the FS_6G_ARC study (SP-250806) states "Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.".

This contribution addresses the " network exposure framework" aspect of WT#1.2.

Architectural Motivation:

Since Release 15, 3GPP has specified exposure of network capabilities via the SCEF in EPS and the NEF in the 5G System.

In Release 18, the Exposure Framework of the 5G System was enhanced to support exposure of network events directly from the UPF or via a Local NEF. RAN Information, such as QoS Monitoring Information and Available Bitrate are example of information that can be exposed via the UPF.

Interworking between the EPS and 5GS is achieved with a combined SCEF+NEF node that supports southbound interfaces of both the EPC and 5GS and a single API interface. The SCEF+NEF can provide the functionality of many APIs without the API consumer needed to know whether the UE is registered in EPS or 5GS.

Gap Analysis: What are the limitations or gaps in existing architecture (from Rel-15 to Rel-20) that this proposal aims to resolve?

Until Release 18, all RAN information was exposed via the AMF to via both the AMF and SMF. In Release 18, a more direct reporting path was added via the UPF but much exposure functionality is still achieved via paths that are very indirect (i.e. RAN -> AMF -> SMF -> NEF).

Intent-based APIs consider the API consumer’s objectives rather than requiring the consumer to determine and perform multiple API calls. For example, a consumer can provide an intent that describes a desired outcome, as opposed to a traditional approach where the focus is on determining and performing multiple API calls to achieve a desired outcome. The network can then translate the intents into list of actions and/or policies that can be applied to the network. The network can then activate the intent by executing the actions and/or policies. The network could also monitor network conditions to see if the intent is being met and consider executing different actions or policies to maintain the intent.

One limitation of existing exposure mechanisms require an API consumer to have a good understanding of the underlying network to achieve a goal. Intent based APIs are generally easier for application developers to utilize since they require less understanding of how the underlying network work and allow the network to inter-relate a set of actions to an API consumer’s goal. An intent based APIs provide a level of abstraction that is higher than what is provided by the Nnef APIs that are specified in TS 29.522.

Technical Impact:

- The overall system architecture would be impacted because 6G network functions would likely need to expose information and possibly allow for some degree of control request inputs from external sources.

- Enhancements under this work task need to consider interworking with legacy systems. For example,

  - How can the functionality that is exposed by EPS and 5GS (TS 29.122 and TS 29.522) be exposed in 6G?

  - How should the framework deal with a case where a 6G-only capability is exposed and the UE moves to an earlier generation system where the functionality is not available?

**** First Change – All New Text ****

# Annex A.X. WT#1.2 Network Exposure Framework

One aspect of this work task is to study whether and how to support and/or enhance the network exposure framework in 6G. Aspects that can be studied includes:

- Whether and how to support the exposure of network capabilities that provide at least the same functionality that is exposed by the SCEF in EPS and NEF in 5GS?

- For each exposed capability, what is the most efficient exposure/reporting path?

- Whether and how the network exposure framework should support intent-based exposure, including:

  - If intent-based exposure is supported in the 6G system, what capabilities, if any, are needed in the network exposure framework to support intent-based exposure?

  - If intent-based exposure is supported in the 6G system, what are the exposure point(s)?

**** Second Change – All New Text ****

# 5.X. Key Issue #X: Key Issue Title

This key issue is to study whether and how to support and/or enhance the network exposure framework in 6G. Aspects that can be studied in this key issue includes:

- Whether and how to support the exposure of network capabilities that provide at least the same functionality that is exposed by the SCEF in EPS and NEF in 5GS?

- For each exposed capability, what is the most efficient exposure/reporting path?

- Whether and how the network exposure framework should support intent-based exposure, including:

  - If intent-based exposure is supported in the 6G system, what capabilities, if any, are needed in the network exposure framework to support intent-based exposure?

  - If intent-based exposure is supported in the 6G system, what are the exposure point(s)?

**** End of Changes ****