SA WG2 Meeting #170	S2-2506523

25 – 29 Aug, 2025, Goteborg , SE

Source:	Ofinno

Title:	[WT#1.1, control signalling] NAS functionality for 6G

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes the WT scope and introduces a new KI associated with WT#1.1 to support control signalling for 6G, with a particular focus on 6G NAS functionality.

# 1. Justifications

This contribution proposes the WT scope and a key issue to work on the WT#1.1 of the approved 6G SID (SP-250806):

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network: NOTE 1: The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split. 1.1. Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following: a. Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities. b. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities. c. Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services. 1.2. Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access. 1.3. Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access. 1.4. Study whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G. In addition to the work tasks above, this study will identify other 5G features that will be supported and/or adapted in 6G.

Architectural Motivation: 6G services will include both basic connectivity services (e.g. registration, mobility management, paging, session management) and beyond connectivity services (e.g. operator services over N6 such as AI/ML or sensing services). In 5GS, NAS functionality operates uniformly regardless of service type, with all NAS messages anchored in the AMF for MM or routed via the AMF to SMF for SM. While this approach simplifies architecture, it creates processing bottlenecks in AMF and prevents service-specific optimization. For 6G, there is an architectural opportunity to introduce extended NAS functionality that can terminate in existing or new NF types, adapt NAS message paths based on service requirements, and minimize unnecessary AMF involvement to reduce control-plane congestion.

Gap Analysis: From Rel-15 to Rel-20, the NAS architecture has remained AMF-centric, with no mechanism to route NAS messages directly to service-specific NFs or to bypass AMF when its involvement is not required. This limitation restricts flexibility in supporting diverse service requirements and increases control-plane load. Current specifications also lack a framework to selectively provide minimal NAS functionality for general connectivity while offering additional NAS capabilities for specific UE types (e.g. NB-IoT) or operator services.

Technical Impact: The proposed work will:

- Enable flexible NAS termination points, including in new NF types, allowing service-specific control signalling paths.

- Reduce AMF processing load and control-plane congestion by introducing NAS routing methods that minimize or bypass AMF involvement where appropriate.

- Provide mechanisms to tailor NAS functionality according to service type and UE capability, ensuring minimal mandatory NAS for general connectivity and extensible NAS for advanced services.

- Introduce procedures for efficient NF discovery and NAS capability determination, supporting scalable architecture evolution.

- Define methods for determining 6G NAS capability in home and visited networks to ensure consistent service support in roaming scenarios.

**** First Change ****

# Annex A.X. WT#1.1 Scope

WT#1.1: Study how to extend the Non-Access Stratum (NAS) functionality to support control signalling for 6G services. The 6G services to be supported by the new NAS functionality can be distinguished such as the basic connectivity services for communications (e.g. mobility management such as registration, paging, session management), and the beyond connectivity services (e.g. the operator services that can be existed over N6 such as AI/ML services or Sensing services). The following aspects will be, at least, considered:

- How to introduce a new NAS functionality while considering minimum impacts on other NAS functionalities, and how to minimize congestion/overhead in the existing control-plane-based NAS signalling.

- How to identify/discover a NF which is supporting the 6G NAS functionality.

- How to identify a minimal set of NAS functionalities for UEs using basic connectivity services in general, and additional sets of NAS functionalities for certain types of UEs (e.g. NB-IoT) and/or certain type of operator services (e.g. AI/ML services, Sensing services).

- How to support the new NAS functionality in roaming scenarios.

**** Second Change ****

# 5.X. Key Issue #X: NAS functionality for 6G

This key issue will study how to extend the NAS functionality to support the enhanced system architecture and procedures for 6G services.

The basis of 6G services can be the connectivity services (e.g. mobility management such as registration, paging, and session management) and the beyond connectivity services (e.g. the operator services that can be existed over N6 such as AI/ML services or Sensing services). Based on the connectivity and beyond connectivity services to be supported and the methods for their support, this key issue will identify the essential and minimum elements as well as additional elements of the extended NAS functionality for 6G.

In 5GS, the AMF and SMF are the NFs which support NAS functionality. The NAS for Mobility Management terminates in AMF and the NAS for Session Management terminates in SMF via AMF. In this key issue, for 6G system, the extension of NAS functionality may introduce new NAS functionality into the NF type(s) which has been supported NAS or has not been supported NAS. It may include the split or add of NAS message transfer path that can work with minimum AMF involvements.

The following aspects will be studied for this key issue:

- How to introduce 6G NAS functionality to support the connectivity of UE and 6G network, and/or to support the connectivity of UE and operator services beyond the 6G core network. This includes:

  - How to determine whether the UE/network supports the 6G NAS functionality.

  - How to identify/discover a NF which is supporting the 6G NAS functionality.

  - How to transport the NAS messages between the UE and the network that are supporting 6G NAS functionality with minimum involvements of other existing NFs.

  - How to minimize congestion/overhead in the NAS signalling.

  - How to support no or minimum impacts on other existing NFs, when the NF supporting 6G NAS functionality is used.

  - How to support the security of the 6G NAS functionality (e.g. which NF (AMF, or else) will be responsible for NAS security, and corresponding procedures), based on coordination with SA3.

- Identify a minimal set of NAS functionalities to be supported for all UEs using general connectivity services (e.g. registration, mobility, paging, using PDU Sessions), and the additional NAS functionalities to be supported for certain types of UEs (e.g. NB-IoT) or for certain operator services (e.g. AI/ML services, Sensing services). This includes:

  - Identify the required NF(s) to serve the UEs using general connectivity services, and the corresponding architecture and procedures.

  - Identify the additional NF(s) to serve the UEs using certain operator services or to serve certain types of UEs, and the corresponding architecture and procedures.

- How to support the 6G NAS functionality in roaming scenarios. This includes:

  - How to determine whether the Home/Visited network supports the 6G NAS functionality.

**** End of Changes ****