SA WG2 Meeting #170	S2-2506524

25 – 29 Aug, 2025, Goteborg , SE

Source:	Ofinno

Title:	[WT#1.2, User plane architecture, QoS framework] 6G QoS framework

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes the WT scope and introduces a new KI associated with WT#1.2 to support 6G QoS framework.

# 1. Justifications

This contribution proposes the WT scope and a key issue to work on the WT#1.2 of the approved 6G SID (SP-250806):

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network: NOTE 1: The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split. 1.1. Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following: a. Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities. b. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities. c. Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services. 1.2. Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access. 1.3. Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access. 1.4. Study whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G. In addition to the work tasks above, this study will identify other 5G features that will be supported and/or adapted in 6G.

Architectural Motivation:

In the 5GS architecture, based on the principle of separation between the control plane and the user plane for session management, the SMF and the UPF interact with the UE. This enables independent evolution of the control plane and the user plane.

However, the strict separation between the control plane and the user plane also introduces several challenges:

- SMF congestion due to all PDU session signalling passing through the control plane

All signalling related to a PDU session flows via the SMF in the control plane. While this is unavoidable for establishing the PDU session itself, after the session is established, this design may cause congestion at the SMF. As discussed during the 6G workshop, many core network functionalities and procedures rely heavily on both the AMF and the SMF, with the SMF often acting only as an intermediate relay. For example, relaying TSN-related containers or secondary authentication signalling may not necessarily require SMF involvement. A redefinition of the functional boundary between the SMF and UPF may therefore be needed.

- Dependency of SMF signalling on AMF availability

SMF-related signalling depends entirely on the availability of the signalling path via the AMF. If the AMF is congested or unavailable, signalling to the SMF is also affected. In cases where certain signalling (e.g., PS Data off) should still be possible under AMF/SMF congestion, the current architecture limits flexibility. Maintaining the strict SMF–AMF dependency in 6G may prevent full utilization of having two independent UE–core network paths. Revisiting the SMF–UPF separation could allow more robust PDU session management even when the AMF is impaired.

- Latency implications of multi-node involvement

The signalling path between a UE and the SMF can be longer than that between the UE and the UPF, resulting in higher latency—especially in cases such as secondary authentication for a DN. With advanced applications requiring more dynamic adaptation to changing traffic needs, even small QoS adjustments in 5G require UE–SMF signalling, though enforcement is ultimately at the UE and UPF. As low-latency requirements become more critical and UPF exposes more services to other network functions, more efficient mechanisms for PDU session handling will be necessary.

Gap Analysis:

In the 5G architecture, all PDU session–related signalling is delivered through the control plane, including the AMF. Direct signalling from the UPF to the UE is limited. For example, the initiation of reflective QoS is only possible from the UPF but without a more general signalling mechanism.

Technical Impact:

- The study may extend the current QoS control granularity in 5G, which is mainly at the QoS Flow and PDU Session levels, to support diverse 6G services such as AI/ML, sensing, immersive media, and time-sensitive networking.

- QoS control triggering, currently limited to network initiation in 5G, may also include UE-triggered and network–UE negotiated control, enabling faster adaptation during ongoing sessions.

- QoS control signalling paths may be expanded beyond the control-plane route (UE–RAN–AMF–SMF) to include in-band signalling over the user plane between the UE and the UPF, reducing latency and dependency on multiple control-plane nodes.

**** First Change ****

# Annex A.X. WT#1.2 Scope

WT#1.2: Study how to enhance the QoS framework and user plane architecture to support 6G services. The following aspects will be at least considered:

- How define and enhance QoS control granularity for 6G services such as AI/ML services, sensing services, immersive media, time-sensitive networking.

- How to enhance QoS control mechanisms to support dynamic and adaptive controls with reduced signalling dependencies and lower latency.

- How to enhance QoS control signalling path management with the aim of avoiding multi-node involvement (e.g. dependent on AMF availability), including the signalling via the control plane and signalling via the user plane supporting in-band signalling.

**** Second Change ****

# 5.X. Key Issue #X: 6G QoS framework

In the current 5GS architecture, the strict separation between control plane and user plane, and the dependency of SMF signalling on AMF availability, may limit the ability to provide timely and adaptive QoS adjustments for advanced services. Furthermore, even small QoS changes typically require UE–SMF signalling, although the actual enforcement is done at the UPF. These constraints can lead to additional latency and reduced flexibility, particularly under AMF or SMF congestion.

This key issue will study whether and how to enhance the QoS control mechanism to support dynamic and adaptive controls with reduced signalling dependencies and lower latency. The study will cover, but not be limited to, the following aspects:

- QoS control granularity for 6G services: the possible levels of QoS control (e.g. per QoS Flow, per PDU Set) to support diverse services for 6G such as AI/ML services, sensing services, immersive media, and time-sensitive networking.

- QoS control signalling path management: the signalling paths for QoS management and adjustments, including the conventional UE-SMF signalling via the control plane and possible direct signalling over the user plane between the UE and the UPF, supporting in-band signalling

NOTE X:	This key issue may need alignment with the key issues related with policy control aspects.

**** End of Changes ****