3GPP TSG-SA2 Meeting #170	S2-2506529

# Goteborg, Sweden, 2025-08-25 – 2025-08-29								   (revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0009 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment with RAN3 on NGAP for Indirect Connectivity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - AIOTF ID and Correlation ID: In the LS from RAN3 (R3-253941), RAN3 informs SA2 about agreements of the AIOTF ID and Correlation ID in case of indirect connectivity: - In case of indirect connectivity, as parallel sessions between gNB and AMF are supported, the AIOTF Identifier and the Correlation Identifier are included outside of the containers in all the Inventory related NGAP messages (in addition to including the Correlation Identifier inside the containers as previously agreed). - In case of indirect connectivity, as parallel Command procedures for different devices between gNB and AMF within the same session are supported, the AIOTF Identifier, the Correlation Identifier, and the RAN A-IoT Device NGAP ID are included outside of the containers in all the Command related NGAP messages (in addition to including the Correlation Identifier and the RAN A-IoT Device NGAP ID inside the containers as previously agreed). SA2 needs to align with RAN3 on Correlation ID and RAN AIoT Device NGAP ID in the Procedures between AIOTF and NG-RAN for Indirect Connectivity - Notification Target Address: The interactions between the AIOTF and the AMF belong to “Subscribe-Notify”. The AIOTF requests the AMF to deliver the N2 message (e.g., inventory request) towards the NG-RAN, and then implicitly subscribes to the AMF. When receiving the corresponding N2 messages (e.g. inventory report) from the NG-RAN, the AMF notifies towards the AIOTF. In this implicit subscription, the AIOTF needs to provide a Notification Target Address and a Notification Correlation ID to the AMF. With the Correlation ID provided, there are no needs to further provide the Notification Correlation ID by the AIOTF, as the Correlation ID can be used to differentiate the AIoT service operations. - In TS 38.413 BL CR (R3-253937), AIoT Session Release Request, AIoT Session Release Command and AIoT Session Release Complete are introduced. Those NGAP messages need to be reflected in TS 23.369. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Align with RAN3 on NGAP about Correlation ID and RAN AIoT Device NGAP ID in the Procedures between AIOTF and NG-RAN for Indirect Connectivity. - AIOTF provides Notification Target Address to the NG-RAN in Namf_AIoT_MessageDelivery request. - Align with RAN3 on AIoT Session Release Request, AIoT Session Release Command and AIoT Session Release Complete. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with RAN3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4, 7.3.2, 7.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>>BEGINNING OF CHANGES <<<<

### 6.2.4	Procedures between AIOTF and NG-RAN for Indirect Connectivity

An AIOTF and NG-RAN can use an indirect interface via an AMF as described in clause 4.2.2.4. The additional steps used for indirect interface between AIOTF and NG-RAN are shown in Figure 6.2.4-1.

Figure 6.2.4-1: Procedure for AIOTF and NG-RAN for indirect connectivity via an AMF

1.	The AIOTF sends Namf_AIoT_MessageDelivery message (NGAP AIoT information, NG-RAN ID, AIOTF ID, Correlation ID, [RAN AIoT Device NGAP ID], Message Type for NGAP AIoT information, Notification Target Address) to the AMF. The NGAP AIoT information may be Inventory Request Transfer, Command Request Transfer or AIoT Session Release Command Transfer, as specified in TS 38.413 [10]. The AMF stores the Notification Target Address with the corresponding AIOTF ID, Correlation ID and optional RAN AIoT Device NGAP ID.

The RAN AIoT Device NGAP ID is provided when Command Request Transfer is to be delivered.

2.	The AMF sends an NGAP message (AIOTF ID, Correlation ID, [RAN AIoT Device NGAP ID], NGAP AIoT information) to the target NG-RAN.

3.	The NG-RAN sends an NGAP message (AIOTF ID, Correlation ID, [RAN AIoT Device NGAP ID], NGAP AIoT information) to an AMF. NGAP AIoT information may be Inventory Response Transfer, Inventory Report Transfer, Inventory Failure Transfer, Command Response Transfer, Command Failure Transfer, AIoT Session Release Request Trasfer, AIoT Session Release Complete Transfer, as specified in TS 38.413 [10].

The RAN AIoT Device NGAP ID is provided when Inventory Report Transfer, Command Response Transfer or Command Failure Transfer is delivered.

The NGAP AIoT information in step 2 and step 3 can be a pair of request and response messages:

-	For inventory, Inventory Response Transfer and Inventory Failure Transfer are the response messages of Inventory Request Transfer.

-	For command, Command Response Transfer and Command Failure Transfer are the response messages of Command Reqeuest Transfer.

-	For session release, AIoT Session Release Complete Transfer is the response message of AIoT Session Release Command Transfer.

The NG-RAN node can also send NGAP AIoT information in step 3 actively:

-	To report inventory results, the NG-RAN node sends Inventory Report Transfer.

-	To request the release of the AIoT session, the NG-RAN node sends AIoT Session Release Request Transfer.

4.	AMF determines the Notification Target Address and sends the Namf_AIoT_Notify message (NGAP AIoT information, Correlation ID, [RAN AIoT Device NGAP ID]) to the AIOTF.

>>>>NEXT CHANGE <<<<

### 7.3.2	Namf_AIoT_MessageDelivery service operation

Service operation name: Namf_AIoT_MessageDelivery

Description: The NF consumer requests to send AIoT data  towards NG-RAN or AIoT devices.

Inputs, Required:

1)	NGAP AIoT Information to be delivered to NG-RAN.

2)	NG-RAN Node ID.

3)	AIoT NGAP Message Type ("Inventory" or "Command").

4)	AIOTF Identifier and Correlation Identifier, this is to allow identifying the association between NG-RAN and AMF.

5)	Notification Target Address.

Input, Optional: RAN AIoT Device NGAP ID.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure.

>>>>NEXT CHANGE <<<<

### 7.3.3	Namf_AIoT_Notify service operation

Service operation name: Namf_AIoT_Notify

Description: The NF consumer requests to receive AIoT data from NG-RAN or AIoT devices. If the NF consumer invokes the Namf_AIoT_MessageDelivery, the NF consumer implicitly subscribes to receive the AIoT data from NG-RAN or AIoT devices.

Inputs, Required:

1)	NGAP AIoT Information received from NG-RAN.

2)	Correlation Identifier.

Input, Optional: RAN AIoT Device NGAP ID.

Outputs, Required: Operation execution result indication.

Output, Optional: None.

>>>>END OF CHANGES<<<<