3GPP TSG-SA WG2 Meeting #170	S2-2506538

Gothenburg, Sweden, August 25 – 29, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	New KI for WT#1 to support AIoT services in Topology 2

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: The contribution discusses and proposes a new KI for WT#1 to support AIoT services in Topology 2.

1. Introduction

The “New SID on Architecture support of Ambient power-enabled Internet of Things – Phase 2” (SP-250834) has been approved in SA#108. In the SID, WT#1 is about how to support AIoT services in Topology 2 for indoor use cases:

WT#1: Study the support of AIoT services in Topology 2, including:

-	 Study and investigate how to support AIoT services under the RRC-based option for UE Reader connectivity.

NOTE 3:	Rel-19 TR solution and interim conclusion should be used as basis for rel-20 work. Solution submitted to rel-20 need to align with rel-19 TR interim conclusion. No rel-19 AIoT Device impact from this WT is expected.

In topology 1, RAN reader inside NG-RAN are used to communicate with AIOT devices. In topology 2, UE reader will be used as an intermediate node to communicate with AIoT devices.

[Observation-1] For topology 2, RRC-based option will be further studied and investigated.

In TR 23.700-13, there are interim conclusions reached for topology 2, including RRC-based option. In Rel-19, SA2 had performed comprehensive work, even though there are some aspects missing or not concluded. In our view, this KI can be proposed and agreed in sufficient detail level.

System architecture wise, the solution of topology 2 aligns with both connectivity options of topology 1, where NG-RAN and the AIOTF communicate directly and indirectly via an AMF. For the RRC based option, there shouldn’t be any difference on whether the inventory/command related communication results in activity at an "BS Reader" or an "intermediate UE".

Indeed, from signalling and NGAP interface perspective, RRC-based option of topology 2 aligns with the Rel-19 solution for topology 1 very well. Using Inventory Procedure in TS 23.369 as an example:

Within those steps (interactions in CN and from/to AF), step 1-6, step 11, and step 13-14 are the same for topology 1 and topology 2.

Step 7, 8, 10, 12 (interactons between CN and RAN) should be common for topology 1 and topology 2.

In Step 7, the following steps are to be performed for indirect connectivity of topology 1:

7.a. The AIOTF performs NG-RAN Node and RAN Reader Selection as specified in clause 5.3.3.

7.b. The AIOTF performs AMF discovery and selection as specified in clause 5.3.4, based on configuration.

7.c. The AIOTF sends Namf_AIoT_MessageDelivery message to the AMF as specified in clause 6.2.4. The AMF sends Inventory Request to the NG-RAN node as specified in clause 6.2.4.

Step 9 (interactions in RAN) is different but the AIoT Uu interface (interactions between reader and AIoT devices) should be kept the same. For topology 1, the NG-RAN node communicates with the RAN readers via internal interface, while for topology 2, the NG-RAN node communicates with the UE readers via RRC. But such differences are mainly to be handled in RAN WGs.

Among step 7, 8, 10, 12, deeper analysis on RRC-based option of topology 2 can be performed, based on the existing solution of topology 1. The alaysis below uses Inventory request towards a certain area as an example.

Step 7.a: NG-RAN Node and RAN Reader Selection

For topology 1, in TS 23.369, the NG-RAN node and RAN reader selection is performed based on OAM in the AIOTF:

The AIOTF selects NG-RAN node(s) and optionally RAN readers based on the configured NG-RAN node selection information. The AIOTF may also select the RAN reader based on the stored last known RAN Reader information.

The AIOTF obtains the NG-RAN selection information (AIoT Area list, RAN Reader list, and, optionally, the location served by each RAN Reader and each AIoT Area) via OAM or local configuration.

When the AIOTF receives an AIoT service request, based on the received Target Area information in the request and the NG-RAN selection information, the AIOTF selects the NG-RAN node(s) to handle the request, and determines a Requested Service Area Information for each selected NG-RAN node. If the RAN Readers within an AIoT Area is configured in AIOTF, the AIOTF can directly select the RAN Reader(s).

For topology 2, there were no concrete conclusions in Rel-19 study on the UE readers/NG-RAN nodes selection. How can UE readers be selected, considering the indoor use case where the target AIoT area is much smaller than a cell, as shown in the figure below?

Can we benefit from using the local configuration as in the AIOTF for topology 1? Or do we need to find other solutions to select UE readers and/or NG-RAN nodes in an AIoT area?

There are discussions ongoing that assume to concentrate, for topology 2, on deployment scenarios with fixed installed and well planned intermediate UEs, so assuming the same OAM solution for this kind of scenario appears to be appropriate.

[Proposal-1.1] Study on how the NG-RAN node and UE reader selection can be performed.

The agreed RAN WID in RP-251885 specifies that the objective of RAN3 includes "specficiation of UE reader authorization, including F1AP support and UE reader selection". And UE reader selection was removed from the SA2 SID.

[Proposal-1.2] SA2 will need to consider impacts on overall architectural impacts  if RAN3 outcome includes  further investigation into UE Reader Selection by CN.

Step 7.b: AMF Selection

For topology 1, in TS 23.369, the AMF selection is performed based on the configuration in the AIOTF:

The AIOTF selects the AMF that is corresponding to the selected NG-RAN nodes based on the local configuration in order to forward the AIoT service operation messages towards the selected NG-RAN node(s) (see NG-RAN Node and RAN Reader Selection in clause 5.3.3) via the selected AMF.

For topology 2, the same selection method can apply, and there are no aspects left to study on it in our view.

[Proposal-2] No need to further study on the AMF selection. The details can be figured out in normative phase, if there are.

Step 7.c: 	Namf_AIoT_MessageDelivery and Inventory Request

For topology 1, in TS 23.369, the AIoT includes

The AIOTF sends the AIoT service request to each selected NG-RAN node, either directly or through the selected AMF, the AIOTF sends the AIoT service request to each selected NG-RAN node along with the corresponding Requested Service Area Information for the NG-RAN node.

For topology 2, the same principle should apply in order to align network functions between topologies as much as possible. Regarding the Requested Service Area Information, a list of AIoT Areas and/or a list of Readers (which may include BS readers and UE readers) can be included (if it is not empty). For the fixed UE reader deployment (UEs are well planed and fixed installed), does the AIOTF needs to provide the a list of UE Readers to the NG-RAN? How about mobile UE reader deployment (UEs are handheld)? If needed, what is the Reader ID to be included in the Requested Service Area Information in case of topology 2, which can be understood by the NG-RAN and the AIOTF?

Can we assume that the Requested Service Area Information may contain a list of readers and NG-RAN is able to interprete each reader ID w.r.t. the applied topology? Which entity is assumed to assign the reader ID in case of topology 2? Can we assume a reader ID to be pre-configured and valid within an NG-RAN node, or would NG-RAN assign such ID dynamically? Or does the AMF or the AIOTF perform take over such task?

[Proposal-3.1] Study on how the AIOTF can provide the information about the UE readers to the NG-RAN node, if needed.

In our view, topology 2 should align with topology 1, i.e. the reader ID, irrespective of the topology, is managed by NG-RAN (RAN OAM), and left for further discussion in RAN WGs. The definition of NGAP IEs is also under the remit of RAN3, including Inventory Request Transfer, Requested Service Area Information, etc. The dependencies on RAN3 about the UE Reader ID can be foreseen.

[Proposal-3.2] The UE reader ID requires input from RAN3.

Step 8, 10, 12:

For topology 1, the NGAP messages (e.g., Inventory Response, Inventory Report) can be transferred from the NG-RAN node to the AIOTF via the AMF. With the AIOTF ID and Correlation ID provided inside the container and outside container, the NG-RAN node and the AMF are able to route the response/notification messages properly.

For topology 2, the same routing method can apply, and there are no aspects left to study on it in our view.

[Proposal-4] No need to further study on the message routing for the response/notification messages from the NG-RAN to the AIOTF. The details can be figured out in normative phase, if there are.

Besides the inventory procedure and command procedure, there are another important aspect to be concluded in topology 2: UE reader authorizaiton.

UE reader authorizaiton:

In Rel-19, there was sufficient discussion in the interim conlusion phase, with some ENs unresolved:

In clause 8.1.3.1 of TS 23.700-13, the followings have been agreed:

Subscription aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

Editor’s note:	Additional subscription information, e.g. validity information, for the UE Reader is FFS.

-	UE Reader subscription information is available to AMF and AIoTF.

-	If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, then AMF informs NG-RAN that the UE is authorized to operate as a UE Reader.

Editor’s note:	Whether and how to enable authorization to the UE is FFS.

In clause 8.1.3.4 of TS 23.700-13, the followings have been agreed:

The following interim conclusion are agreed for the UE Reader authorization in Topology 2:

-	The subscription data of UE Reader includes the indication that whether the UE is allowed to act as an AIoT Reader.

-	The AMF provides the UE Reader authorization indication to the NG-RAN serving the UE Reader.

Editor's note:	How to authorize the UE Reader will be determined later.

[Proposal-5] Study on how the UE reader authorization can be performed. The interim conclusions in TS 23.700-13 will be the basis, and the ENs  needs to be resolved.

[Proposal-6] It is proposed to capture the aspects above in the KI

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-30.

* * * Start of Change * * * *

## 5.x	Key Issue #x: Support AIoT services in Topology 2

### 5.x.1	Description

To enable the support of AIoT services in Topology 2, the following aspects will be studied in this KI:

-	How the UE reader authorization can be performed.

NOTE X:	The interim conclusions of UE reader authorization in Rel-19 will be the basis, and the open issues need to be solved.

-	Potential impact from how the NG-RAN node and UE reader selection can be performed, if RAN3 identifies the needs of UE reader selection by the CN.

-	How the AIOTF can provide the information of the UE readers to the NG-RAN node, if needed.

NOTE Z: The aspect of UE reader identification requires input from RAN3.

* * * End of Change * * * *