3GPP TSG-SA WG2 Meeting #170	S2-2506539

Gothenburg, Sweden, August 25 – 29, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	New KI for WT#2 to support DO-A Capable AIoT Devices

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: The contribution discusses and proposes a new KI for WT#2 to support DO-A Capable AIoT Devices.

1. Introduction

The “New SID on Architecture support of Ambient power-enabled Internet of Things – Phase 2” (SP-250834) has been approved in SA#108. In the SID, WT#2 is about how to support DO-A capable AIoT Devices:

WT#2: Study the support of DO-A Capable AIoT Devices, including:

-	Support of the AIoT Device informing the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure).

-	Support for an autonomous AIoT Device originated procedure to send data to the AIOTF, and support for routing the received data by AIOTF.

-	Naiotf and Nnef interface enhancements to provide the data received from an AIoT Device to the AF.

NOTE 4:	topology 2 aspect of WT#2 has dependency on WT#1.

As described in the first Sub-WT “Support of the AIoT Device informing the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure).”, a DO-A capable device needs to perform registeration-like procedure towards the network actively, so that it can send UL data (aka DO-A data) towards the network without waiting for the DL trigger from the network. That is, without the inventory or command procedure from the network, a DO-A capable device can send DO-A data to the network. Before sending the DO-A data, it is essential to have the security parameters exchanged between the AIoT device and the network, and have the security contexts established. The registration-like procedure needs to cover such aspect. And it is up to SA3 to study how the security parameters are exchanged and how security contexts are established.

[Proposal-1] It is proposed to study on how the DO-A capable device can perform a registeration-like procedure towards the network, while the security parameters exchange and security contexts establishment are expected to be studied by SA3.

As described in the second Sub-WT “Support for an autonomous AIoT Device originated procedure to send data to the AIOTF, and support for routing the received data by AIOTF.”, there are two aspects included:

- 	How the DO-A capable AIoT device sends DO-A data to the AIOTF? As the protection is performed for the command request and command response, for security reasons, the DO-A data are also expected to be protected. Such protection is under the remit of SA3.

[Proposal-2] It is proposed to study on how the DO-A capable device can send DO-A data to the AIOTF, while how DO-A data are protected is expected to be studied by SA3.

-	After the network receives the DO-A data form the AIoT device, it needs to forward the data towards the proper AF(s). The DO-A data and command response are UL data, but there is a significant difference in the message routing: the command response can be sent to the AF who issuee the command request, but for DO-A data, there is no such AF.

[Proposal-3] It is proposed to study on how the AIOTF can send the received DO-A data to the right target AF(s).

To support DO-A capable AIoT devices, the solution needs to developed be based on the existing solution framework. DO-A capable AIoT devices (type 2b/C) are much powerful than the Rel-19 passive devices (type 1). For the existing inventory procedure and command procedure, the improvements can be considered to make the communication between the AIoT devices and network more efficiently. For example, as the DO-A capable device and the network have the security context established, the “command-only” delivery method may be utilized, so that the command can be delivered more efficiently.

[Proposal-4] It is proposed to study on whether and how the inventory procedure and command procedure can be improved to make the communication between the DO-A capable AIoT devices and network more efficiently.

To support DO-A capable AIoT devices, the e2e support of DO-A traffic is required. Not only core network entities, but RAN/UE readers and NG-RAN node need to support DO-A. There may be some additional CN-RAN aspects identified during the study, and such aspects may require further work or coordination with RAN WGs.

[Proposal-5] It is proposed to identify any additional CN-RAN aspects that may require further work/coordination with RAN WGs.

[Proposal-6] It is proposed to capture the aspects above in the KI

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-30.

* * * Start of Change * * * *

## 5.x	Key Issue #x: Support of DO-A Capable AIoT Devices

### 5.x.1	Description

To enable the support of DO-A capable AIoT devices, the following aspects will be studied in this KI:

-	How the DO-A capable device can perform registration-like procedure towards the network.

NOTE X: During the registration-like procedure, the security parameter exchange and security context establishment between AIoT devices and AIOTF are to be studied by SA3.

-	How the DO-A capable device can send DO-A data to the AIOTF.

NOTE Y: The protection of DO-A data is to be studied by SA3.

-	How the AIOTF sends the received DO-A data to the right target AF(s).

-	Wheter and how the inventory procedure and command procedure can be improved to make the communication between the DO-A capable AIoT devices and network more efficiently.

-	Identify any additional CN-RAN aspects that may require further work/coordination with RAN WGs

* * * End of Change * * * *