3GPP TSG SA WG2 Meeting #170			S2-2506552

Gothenburg, Sweden, August 25th-29th, 2025

# Source:	Ericsson

# Title:	New WID on AF Influence on Slice and DNN Activation

# Document for:	Approval

# Agenda Item:	30.1 (Submissions for TEI20)

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	WID on AF Influence on Network Slice and DNN Activation

Acronym:	TEI20_ InflAct

Unique identifier:			{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

In enterprise-driven use cases (e.g., when the UE is provisioned with an S-NSSAI that provides access to an enterprise network), it is sometimes required that an AF, via an exposure API, is able to temporarily activate or deactivate access to one or more DNNs or even an S-NSSAI (and all its associated DNNs), e.g., to boost the whole connectivity to enable the UE to access a premium QoS S-NSSAI/DNN. An AF, via exposure API, is already able to influence the DNN/S-NSSAI a UE should select for specific traffic, but this has dependencies with the UE support and implementation of URSPs. A mechanism fully controlled by the network to enable and disable UE access to a given DNN/S-NSSAI is not available.

Additionally, Operator Determined Barring function provides the operator means to bar different services for a UE. When a service is barred (either at UE level or DNN level), a flag is stored in UDR indicating that the service is barred and thus, should be treated equally as it had not been effectively provisioned in UDR. The benefit of a barring indicator includes not needing to store all the associated data to the barred service, which might be needed for a later recreation when the service is eventually unbarred.

While the currently ODB function provides either UE level or DNN level barring indicators, precluding a UE to establish a PDU session on the barred DNN, no network slice barring indicator exists. The definition of such network slice barring indicator would allow the operator to bar the registration and, consequently, PDU session establishment on the barred network slice and all its associated DNNs. Without this network slice barring indicator, an operator willing to bar access to a network slice and the associated data networks (e.g., an enterprise or organization network slice) would need to bar access to each of the DNNs, and this would need to be controlled in case additional DNNs are added later. This is because from the moment the slice is provisioned for the UE, the UE has immediate access, and therefore, if the operator needs to disallow access to network slice (temporarily or for a planned period of time), the operator would need to remove from the UDR the corresponding S-NSSAI from the 5GS subscription data, which might first require the removal of SMF selection data, SM data, etc.

# 4	Objective

The objective of this work includes:

- Support for an AF to influence on the activation/deactivation of a DNN for a UE or group of UEs.

- Support for an AF to influence on the activation/deactivation of a network slice for UE or group of UEs.

- Support for an operator, via provisioning means to bar a complete network slice for a UE without the need to remove from the UDR the associated data (e.g. SMF selection data).

## TU estimates and dependencies

| Work Tas ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT 1 |  | 0.5 | No | None |

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Description of the activation/deactivation capability in the External Exposure of Network Capabilities (clause 5.20) | TSG SA#109 |  |
| 23.502 | Adding the new parameters to be activated/deactivated in the Parameter Provision data in clauses 4.15.6 and 5.2. | TSG SA#109 |  |

# 6	Work item Rapporteur(s)

@ericsson.com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Support for an operator, via provisioning means to bar a complete network slice for a UE without the need to remove from the UDR the associated data (e.g. SMF selection data) impacts 3GPP TS 23.015 which is under the remit of CT4.

# 9	Supporting Individual Members

Supporting IM name

Ericsson