SA WG2 Meeting #170	S2-2506555

25 – 29 Aug, 2025, Goteborg , SE

Source:	InterDigital Inc.

Title:	WT#1.1, Support for NAS Signalling in the 6G Architecture

Document for:	Agreement

Agenda Item:	20.6.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution covers the non-access stratum functionality and provides an overview of current limitations fully or partially stemming from the current architecture, based on which a scope for the related (sub-)Work Task (WT) and related Key Issue (KI) are proposed.

# 1. Justifications

Work Task 1.1 in SP-250806 states:

1.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

This contribution provides an analysis of the limitations inherent in the current 5G NAS architectural framework. Building on these findings, a well-defined scope for a work task that focuses on Support for NAS in the 6G architecture is proposed.

Architectural Motivation: The motivations and need for a modular and flexible Non-Access Stratum (NAS) architecture in 6G include:

- Decoupling NAS Functions from each other: Enable deployment of specific NAS functionalities without requiring changes to Access and Mobility Management Function (AMF) or NAS-Mobility Management (NAS-MM). This reduces the AMF's role as a mandatory single point of entry and single point of failure. This enhances system scalability and efficiency.

- Minimal Functionality Set: Allow implementation of a minimal set of NAS functionalities that remain unaffected by additional NAS features (e.g. NAS functionalities that are added after Release 21). Such an approach will improve resource efficiency and support lightweight User Equipment (UE) implementations.

Gap Analysis: The contribution identifies key gaps in the current 5G NAS architecture that need to be addressed for 6G:

- Tight coupling of NAS Functionalities around AMF : All NAS functionalities (e.g., NAS-SM, SMS, UE Policy, LCS) are built on top of NAS-MM, creating a strong dependency on the AMF. This limits modularity and impacts scalability, as adding any new NAS functionality affects the AMF.

- Routing Constraints: NAS messages must be routed through the AMF (single entry point in the core network). This restricts deployment flexibility, especially in distributed scenarios like private/satellite networks. The architecture also creates a single point of failure.

- Point to Point NAS Security: NAS security terminates at the AMF, adding to the tight coupling of other NAS functionalities with AMF as mentioned above.

Technical Impact: To resolve the identified gaps, the document proposes the following technical impacts for the 6G NAS architecture:

- Functional Split: Redefine the functional split between the AMF and other Core Network Functions (NFs) to make NAS functionalities independent of NAS-MM. This includes removing dependenicies between NAS-MM and other NAS modules.

- Flexible NAS Message Routing: Develop mechanisms to route NAS messages directly to target NFs without mandatory AMF involvement, reducing latency and load on the AMF.

- Enhanced UE-CN and RAN-CN Interactions: Define new interaction models between UE, Radio Access Network (RAN), and CN to support direct NAS message transport, including modifications to the N1 and N2 interfaces.

- Distributed NAS Design: Develop generic mechanisms for UE-CN interactions that support operator services in a distributed fashion, minimizing interdependencies and enabling a minimal set of NAS functionalities for lightweight deployments. Furthermore, define a set of architectural principle for adding new NAS functionalities in future releases.

NOTE: 	Security aspects for distributed NAS aiming at reducing other NAS functionalities tight coupling with AMF are to be evaluated by SA3.

**** First Change ****

# Annex A.X WT#1.1 Support for NAS Signalling in the 6G Architecture

The objective of this work task is to study support for control signalling in 6G for connectivity services and/or beyond connectivity services, including at least the following:

- Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

The study will consider:

-	Whether and how to redefine the functional split between the Access and Mobility Management Functionality and other Core Network Functions (NFs).

-	Whether and how to remove dependencies between NAS features in order to support independent evolution and deployment.

-	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

The study will consider:

- Whether and how the 6G system can support distributed NAS design principles to enable minimal configurations.

- Whether and how to determine functional independence among NAS module within the distributed NAS design.

- Whether and how new NAS functionalities can be integrated dynamically without introducing backward compatibility or dependency issues on the core minimal set.

-	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

The study will consider:

-	Whether and how to allow for flexible UE-CN and RAN-CN interactions.

-	Whether and how to support diverse 6G deployment scenarios (e.g., distributed, NPNs, NTNs) through adaptable signalling paths and transport protocols.

NOTE: 	Security aspects for distributed NAS aiming at reducing other NAS functionalities tight coupling with Access and Mobility Management Functionality are to be evaluated by SA3.

**** Second Change ****

# 5.X. Key Issue #X (1.1.X): Support for control signalling in 6G NAS architecture

This key issue (KI) will study the support for control signalling in 6G for connectivity services and/or beyond connectivity services, including at least the following:

- Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

- Whether and how to redefine the functional split between the Access and Mobility Management Function (AMF) and other Core Network Functions (NFs).

- Whether and how to remove dependencies between NAS features in order   to support independent evolution and deployment.

- Reduce NAS routing constraints by supporting direct routing of NAS messages to target NFs..

- Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

- Whether and how the 6G system can support distributed NAS design principles to enable minimal configurations.

- Whether there is a need to define criteria for determining functional independence among NAS module

- Whether and how new NAS functionalities can be integrated dynamically without introducing backward compatibility or dependency issues on the core minimal set.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

- Whether and how to allow for flexible UE-CN and RAN-CN interactions, potentially bypassing the AMF where appropriate.

- Whether and how to support diverse 6G deployment scenarios (e.g., distributed, NPNs, NTNs) through adaptable signalling paths and transport protocols.

- Whether and how to enable the dynamic provisioning of NAS functionalities and policies in a service-agnostic, operator-customizable manner..

NOTE: Security aspects for distributed NAS aiming at reducing other NAS functionalities tight coupling with AMF are to be evaluated by SA3.

**** End of Changes ****