SA WG2 Meeting #S2-170	S2-2506570

25 - 29 August, 2025, Goteborg, Sweden

Source:	Samsung, SK Telecom

Title:	KI#2 Sol#27 update to remove ENs

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes updates to sol#27 to remove ENs.

# Discussion

There are several ENs left in solution#27 of TR 23.700-04. This pCR proposes updates to sol#27 to remove ENs.

Editor's note:	It is FFS whether the UPF can be a consumer or not.

For the above EN on whether UPF can be consumer or not, it is beneficial for network operation by considering the UPF as a consumer of the user plane management analytics. Other than taking the output analytics into account for traffic management, policy update, etc. to assist the specified procedures, the operators may use the output analytics to local configuration of UPF in operator domain.

Editor's note:	Collection of data from both UPF and OAM is FFS.

Based on the nature of AIML, richer input data is beneficial for the performance of both ML model training and inference. During SA2#169, one consideration on collection from UPF is the reporting load of UPF. However, based on information provided by some operators, in 5GS, UPFs are in general have high capacity and capability for handling heavy workload. Therefore, data reporting by UPF to NWDAF is not a major concern for operators. Furthermore, operators expect various data could exposed from UPF to NWDAF to maintain the quality of the output analytics for UP management.

Editor's note:	It is FFS whether and how the UPF can detect traffic hijacking and traffic spoofing.

Editor's note:	It is FFS whether the UPF can reports the specific data based on operator's pre-configuration or not.

Editor's note:	It is FFS if configuration needs to be done by the UPF itself or via the SMF using analytics.

The above three ENs are related to the same issue – configuration of UPF data reporting/exposure to NWDAF.

The same as the existing analytics in TS 23.288, based on the defined input data of an analytics ID, the data source will report data to NWDAF, e.g. based on NWDAF request and subscription. For this user plane management analytics, we also assume that the UPF will only expose the defined input data to NWDAF, not the parameters out of the defined NWDAF input data.

Traffic hijacking and traffic spoofing documented in the analytics input data table in Sol#27. Traffic hijacking and traffic spoofing can reflect the information of the abnormal data packet detected by UPF, which will potentially help NWDAF understand the characteristics and status of the abnormal traffic at UPF. How to detect the traffic hijacking and traffic spoofing and the threshold for UPF reporting could be based on operator implementation, as different operators may apply different criteria on UPF abnormal traffic detection. Therefore, the operator can determine the local configuration for UPF and used an unspecified way. There will be no need to use SMF to configure the UPF reporting in 3GPP specification.

Furthermore, the NWDAF may be also capable to configure the reporting threshold, the matrix for deciding the abnormal patterns and reporting periodicity of traffic hijacking and traffic spoofing via analytics subscription service operation. The NWDAF may request input data from UPF directly or via SMF. However, the configuration of UPF data reporting will be determined by NWDAF per analytics ID

# Proposal

It is proposed to adopt the following changes into TR 23.700-04.

*** Start of the change ***

### 6.27.2	Description

#### 6.27.2.1	General Description of UP Management Analytics

This solution aims to address the issues in KI#2, including:

-	How to provide analytics (e.g. enhancing existing in TS 23.288 [5] or introducing new) including the aspects of required inputs/outputs to improve performance of user plan.

-	Which consumers can subscribe/request the analytics and how the consumers use the analytics to take actions to improve user plane performance.

-	Whether and how to train ML models and perform the inference for the user plane analytics, considering solutions that minimize the load on Control Plane NFs.

Based on the two use cases of KI#2 documented in clause 5.1 of TR 23.700-04, the abnormal packets may cause some issues such as UE malfunctions, UPF performance degradation and system errors. Therefore, it would be beneficial to provide the statistics and/or perdition information of traffic characteristics from/to the application server. By taking this information into account, the network can take action to improve the network performance; for example the network would be able to allocate or reserve sufficient resources for data transmission, determine the appropriate configuration for user plane management. It would be also helpful to provide the statistics and/or perdition of UP working status and the corresponding causes of (potential or detected) UP failure or UP performance degradation. Therefore, the consumer of the analytics could take the above information into account to maintain the healthy working condition of user plane in a failure preventative manner.

The traffic characteristics may include the UL/DL/ round trip of: Data Burst Size, traffic patterns, traffic volume or the number of data packets, traffic rate (e.g. packet rate, bit flow rate, etc.), delay, loss rate, throughput, packet size distribution, jitter, (average) packet interval, etc.

The service consumer of the analytics may be a UPF, SMF, PCF, OAM, etc.

The consumer of these analytics indicates the following information in the request or subscription:

-	Analytics ID = "User Plane Management"

-	Target of Analytics Reporting: a list of NF instance ID(s) or NF Set ID(s), any UE, specific UE or group of UEs. E.g. the UPF (instance/set) IDs (list of) SUPIs or GPSIs.

-	Optional, matrix for UP performance level evaluation, e.g. the thresholds of the parameters of traffic characteristics.

-	Optional, application ID(s).

-	Optional, Area of Interest (AOI(s)).

-	Analytics Filter Information optionally includes:

-	(list of) UPF ID(s) .

-	DNN

-	Area of Interest (AOI(s))

-	S-NSSAI and NSI ID

-	RAT Type

-	List of analytics subsets that are requested, (see clause 6.27.2.3).

-	IP Packet Filter Set or PDR

-	Protocol information

-	An Analytics target period indicates the time period over which the analytics are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, reporting Thresholds, which apply only for subscriptions and indicate conditions on the levels to be reached for the reporting of respective analytics subsets (see clause 6.23.3 of TS 23.288 [5]).

*** Next change ***

#### 6.27.2.2	Input Data

The NWDAF supporting UP management analytics shall be able to collect information from AF, OAM and 5GC NFs.

Table 6.27.2.2-1: Input data related to UPF performance

| Information | Source | Description |
| --- | --- | --- |
| UPF info | UPF/ SMF/NRF | Identifying the UPF(s) that is associated to the collected information, e.g. the NF instance ID(s) or NF set ID(s) of the UPF(s), or the IP address or FQDN of the UPF. |
| UPF load | NRF | Load information indicates the current load of the UPF, e.g. CPU, memory, or percentage of load information. |
| DL/ UL Throughput | UPF/ AF | The throughput UL/DL over the measurement period, .e.g. peak value or average value. |
| UPF service area | UPF/ NRF | The service area information of the corresponding UPF(s). |
| Capacity GTP | UPF/ OAM | The maximum achievable UL and/or DL GTP transmission rate between PSA UPF and NG-RAN and between PSA UPF and UE, e.g. as defined in clause 6.3.11 of TS 28.554 [16]. |
| Data transmission over N3 interface | UPF/ OAM | Data packets or traffic volume of incoming and/or outgoing GTP data packets on the N3 interface (from (R)AN to UPF), e.g. as defined in clause 5.4.1 of TS 28.552 [15]. |
| Data loss over N3 interface | UPF/ OAM | Incoming GTP Data Packet Loss or Rejection in UPF and/or outgoing GTP Data Packet Loss that are not successfully received at gNB over N3, e.g. as defined in clause 5.4.1 of TS 28.552 [15]. |
| Throughput of N3 interface | UPF/ OAM | Upstream and/or downstream throughput at N3 interface, e.g. as defined in clauses 6.3.3 and 6.3.4 of TS 28.554 [16]. |
| Data transmission over N6 interface | UPF/ OAM | The link usage of N6 interface, including the usage of incoming traffic and/or outgoing traffic, e.g. as defined in clause 5.4.2 of TS 28.552 [15]. |
| N4 interface information | SMF/ OAM | The N4 interface information may include the number of requested, successful or failed N4 session establishments and/or N4 session reports, e.g. as defined in clause 5.4.3 of TS 28.552 [15]. |
| Data transmission over N9 interface | UPF/ OAM | The Number of incoming and/or outgoing GTP data packets on the N9 interface associated to the PSA UPF, e.g. as defined in clause 5.4.4.2 of TS 28.552 [15]. This may include the overall, successful or failed data packets transfer. |
| Delay | SMF/UPF/ OAM | The UL/DL/ round trip delay between UPF and NG-RAN, or between UE and UPF, e.g. as defined in clauses 5.4.7, 5.4.8 and 5.4.9 of TS 28.552 [15] or collected from UPF/ SMF. The delay could be the values of average, distribution, maximum, or variance. |
| Time stamp or measurement period | UPF/SMF/NRF/OAM | The associated timing information of the collected information. |

Table 6.27.2.2-2: Input data related to the service flow via user plane

| Information | Source | Description |
| --- | --- | --- |
| Application info | UPF/ AF | Identifying the application associated to traffic/ service flow, e.g. application ID, IP address or FDQN of the application. The data transmission might be from or to the application server. |
| UE IDs | SMF/ UPF/ AF | Identifying the UE(s) (e.g. SUPIs, GPSIs) that are associated to the corresponding data transmission from/to the application server(s). |
| Transport protocol information | SMF/UPF/AF/ | The transport protocol associated to the traffic transmission, e.g. RTP, SRTP, TCP, UDP, DCCP, SCTP, QUIC, etc. |
| IP Packet Filter Set or PDR | UPF/ AF | IP Packet Filter set as defined in clause 5.8.2 of TS 23.501 [2] or the PDR for traffic detection at UPF. |
| Data communication type | SMF/UPF/AF | Identify the data communication type, e.g. IP type, Ethernet type, or unstructured type data communication. |
| Traffic characteristics | UPF/ AF | The traffic characteristics may include the UL/DL/ round trip of: Data Burst Size, traffic patterns, traffic volume or the number of data packets, traffic rate (e.g. packet rate, bit flow rate, etc.), packet delay, loss rate, throughput, etc. within the measurement period. |
| Variation of traffic characteristics | UPF/ AF | The variation or standard deviation of the parameters belong to traffic characteristics within the measurement period, e.g. the variation of the traffic volume or data burst size, etc. within a given time period. |
| (Abnormal) data packet information | UPF | For example, malformed packets, duplicate packets, packet retransmission, traffic hijacking and traffic spoofing (NOTE 1). The UPF may report the abnormal data packet base on operation's pre-configuration or information in consumer's request. |
| Time stamp or measurement period | UPF/SMF/NRF/OAM | The associated timing information of the collected information |
| NOTE 1: detection of the traffic hijacking and traffic spoofing and the threshold of UPF reporting maybe based on operator configuration and implementation, and/or indicated by NWDAF within analytics subscription service operation. |  |  |

To reduce UPF reporting load, the UPF may combine the measurement data related to multiple event(s) subscription in one notify service operation and send combine notification to NWDAF.

NOTE:	UPF reports the specific data based on subscription to reduce UPF reporting load.

*** Next change ***

### 6.27.3	Procedures

Figure 6.27.3-1: Procedure for providing UP management Analytics

1.	The Consumer NF, e.g. UPF, SMF, requests or subscribes to UP management analytics from NWDAF (via NEF in the case that the consumer NF is an untrusted AF) and provides the request input information as specified in clause 6.27.2.1 to NWDAF.

2a.	The NWDAF may subscribe to the service data from 5GC NFs, e.g. SMF, UPF, etc. as defined in clause 6.27.2.2 by invoking the corresponding NF event exposure service operations.

2b.	The SMF may subscribe to QoS Monitoring information from UPF using the N4 Session level Reporting procedure defined in TS 23.502 [3]. The UPF notifies the subscribed event report directly to the NWDAF.
The SMF may use Handling of Payload Headers in clause 5.6.17 of TS 23.501 [2] to collect the data related to the protocol information and, accordingly, the UPF performs Handling of Payload Headers and may notify events related to the Handling of Payload Headers to SMF or NWDAF.

2c.	The NWDAF may collect data from the OAM following the procedure captured in clause 6.2.3.2 of TS 23.288 [5].

2d.	The NWDAF may subscribe to the service data from AF by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service (Event ID = UP management analytics, Application ID, Event Filter information, etc.) as defined in TS 23.502 [3].

3.	The NWDAF derives the requested analytics based on NWDAF internal logic and based on operator policies.

4.	The NWDAF provides the requested analytics to the consumer NF, using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service operation used in step 1.

After receiving the analytics results, the consumer NF may take the following actions:

-	The UPF as a consumer may control the number of DLDR (Downlink Data Report) messages sent to SMF by e.g. adjusting message frequency; or apply downlink traffic suppression (e.g. selective packet dropping) for AS-initiated traffic bursts..

-	The SMF as a consumer may initiate UPF reselection to distribute the load across UPF instances upon detecting abnormal traffic patterns (e.g. RTT fluctuations, abnormal PPS).

-	The SMF as a consumer may identify anomalies and take preventive actions, e.g. notifying UPF to execute traffic gating or shaping, enforcing maximum data rate (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.

-	The PCF as a consumer may provision policies to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.

5-7.	If the consumer NF subscribes to UP Management Analytics in step 1, once the NWDAF generates new output analytics, it provides the new analytics via Nnwdaf_AnalyticsSubscription_Notify to the Consumer NF.

*** End of the change ***