SA WG2 Meeting #170	S2-2506571

25 – 29 Aug, 2025, Goteborg, Sweden

Source:	Samsung

Title:	[WT#3, AI] AI for Network in 6G

Document for:	Agreement

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: this contribution proposes justifications and KI description for Work Task#3 AI in 6G.

# 1. Justifications

The following WT on AI has been agreed in 6G SID in SP-250806.

WT#3: Study how to support and enable use of AI in 6G (e.g. AI agent, framework).

NOTE 4: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

Architectural Motivation:

In 5G, core network automation was specified since earlier stage of 5G by leveraging AIML techniques to continuously enhance 5GC intelligence. Network data analytics function (NWDAF) was introduced as a centralised network function to enable AIML functionality. However, as more diverse use cases require the support of AI, this centralised approach limits the further deployment of core network intelligence. Considering the disadvantages of 5G Core AI, the 6G system is expected to be AI-native from day1 by enabling AI in Core a distributed approach. An AI-native 6G system will work more efficiently and provide more services to user in higher quality.

Based on the pervious SA2 discussion on 6G AI, the AI related aspects may include: AI for network, network for AI and agentic-AI.

AI for network: Core NFs and RAN nodes will be AI-powered. AI for network is the one of the fundamental enablers for 6G AI-native. By enabling AI functionalities in core NF, the core NF would be able to make their own AI-based decision or work with other AI-powered entities to make joint decision, which will be beneficial for network management and user experience improvement.

Network for AI: currently, more and more services requires high amount of computing load and AI support, e.g. sensing, XR, split computing. Providing AI computational support to these new services by 6G system may help operators to increase revenue and optimise user experience. The technical justifications on these services are expected to be discussed under the corresponding WTs, instead of the AI WT.

AI agent: a major difference between AI agent and other AI approach is that the AI agent is capable to accomplish a task by determining the work/call flow autonomously, rather than requiring hard coding of work/call flow. An AI agent might be based on large foundation model, e.g. LLM, to support natural language input. As an AI agent might be trained to focus on some specific tasks, for complex cases, multiple AI agents may work jointly. It might be beneficial to deploy AI agent in UE and application server side to processing complex tasks. However, from 3GPP perspective, considering the e2e procedures are well defined in specifications, how AI agent could work in conjunction with or optimise the critical 3GPP procedures is unclear. Furthermore, based on the current development of AI agent, the performance of AI agent might be unreliable. Proof to show the trustworthiness and deterministic of AI agent decision would be appreciated.

Gap Analysis

In 5G, NWDAF is the major NF that enables the AI functionality in Core. Even though LMF is also AI-capable, the LMF can be only used for positioning related use cases. As diverse use cases require the support if AI, a high number of Analytics IDs are specified in 5G. The output analytics is more considered as the assistance information by the service consumers for decision making. To resolve the issues in 5G, one of the fundamental enablers for 6G AI-native system is to support the AI-powered core network functions and RAN node. Supporting distributed AI-powered network function is beneficial for reducing the cost of collecting massive volume of data, easing the high requirements on high-computing power of the centralised AI NF. Furthermore, the distributed AI-native design of network functions will provide better overall system performance than the centralized AI system.

Based on task, the AI-powered NFs are capable to collect the necessary data from different data sources via data framework and make decision for the corresponding task. For complex issues, the AI-powered NF could work in conjunction in a federated approach with other NFs, and even with RAN node, OAM, UE and application server, as shown in Figure 1.

Figure 1 Native AI support in 6G

To enable native-AI, AI operation related data, model, or other information might to be exchanged between the AI-powered NFs, RAN node, UE, etc. In some cases, the use the existing SBI or NGAP interface is possible to be reused. However, considering the potential complexity of AI coordination in 6G wireless, deploying a data framework would be more beneficial. Therefore, WT#3 and WT#5 may have dependency on each other.

Technical Impact:

- Support of AI-powered core network functions, e.g. for AI model training and/or inference.

- AI operation related information, AI model or AI inference results will be exchanged or transferred between AI-powered NFs, e. g., based on SBI, data management framework.

- Interaction between AI-powered NFs, RAN node, UE and OAM might be required for AI operation, which required a cross domain framework.

- A new data framework will be required to support data collection of AI-powered core network functions.

- Core network may expose AI capability to 3rd party.

- Core network may need expose AI training or inference results to 3rd party, based on 3rd party request.

**** First Change ****

# Annex A.X. WT#3 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

The scope of WT#3 mainly focuses on how to support AI functionality in 6G architecture, including:

- Support of AI-powered core network functions, e.g. for AIML model training and/or inference.

- Support transfer of AI operation related information, AI model or AI inference results between AI-powered NFs.

- Support of interactions between AI-powered NFs, RAN node, UE and OAM, which requires a cross domain interworking.

- Support of data collection by AI-powered core network functions, e.g. via data management framework.

NOTE:	data management framework will be discussed under WT#5. Alignment between WT#3 and WT#5 will be needed.

- Support of AI information exposure to 3rd party, e.g. AI operation related information, AI training results or AI inference results, etc.

**** Second Change ****

# 5.X. Key Issue #X: Support of AI in 6G

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

- What architectural enhancements will be required to support AI-powered core network function (e.g. for AIML model training and/or inference)?

- What AI related information and how will it be transferred between AI-powered NFs, e.g. AI operation related information, AI training results or AIML inference results?

- How to support cross-domain interactions between AI-powered NFs, RAN node, UE for AIML operation?

- How to support data collection and transfer for AI operations by network functions, and what data will be collected?

NOTE:	if data management framework is used for AI related data collection and transfer, alignment between WT#3 and WT#5 will be needed.

- How to support AI capability exposure from 6G system to 3rd party?

- How to support AI related information exposure to 3rd party, e.g. exposure of AI operation related information, AI training results or AIML inference results?

**** End of Changes ****