3GPP TSG-SA2 Meeting #170	S2-2506577

Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025	(revision of S2-250xxxx)

Source:	LG Electronics

Title:	[WT#1.1, NAS functionalities] Scope and New Key Issue for NAS functionalities

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This contribution proposes scope refinement of WT#1.1 NAS functionalities aspects and new KI for NAS functionalities.

1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

Regarding the following WT#1.1 included in the FS_6G_ARC SID, scope needs to be refined and new Key Issue should be added to TR 23.801-01.

1.1. Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following: a. Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities. b. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities. c. Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

Architectural Motivation: The 6G system need to support not only new 6G services but also existing legacy services including IoT services. This implies that 6G system need to support various type of services for diverse types of UEs. As a result, 6G system needs to be designed with forward compatible manner and NAS signalling should be designed in the same way, i.e. NAS needs to support from common functionalities and additional functionalities depending on UE's capabilities without impacting other NAS functionalities.

Gap Analysis: In the 5G system, when a network operator introduces a new service that require support from a UE, legacy UEs cannot benefit from the new service because UE's NAS signalling is not designed in a modular way, i.e. adding a new NAS functionality impacts existing NAS functionalities and both UE and AMF needs to be updated to support a new service. As a result, operator faces challenges in deploying a new service. In order to resolve such problem, NAS signalling should be designed in a forward compatible manner so that NAS functionalities can be easily updated without impacting existing NAS functionalities.

Technical Impact: This WT/KI influence the following to support NAS signalling:

- Interaction between UE, RAN and core network functions to support transmission of NAS signalling.

- Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

- Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

- Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

# * * * * Start of 1st Change (All New Texts) * * * *

A.X	Work Task 1.1

WT#1.1.	Study the support of control signalling in 6G system for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

# * * * * Start of 2nd Change (All New Texts) * * * *

5.X	Key Issue #X: Support of NAS functionalities

This key issue studies support of NAS functionalities in 6G system. Specifically, this Key Issue studies:

-	Support of control signalling in 6G system for connectivity services (e.g. mobility management, session management, policy management, etc.) and beyond connectivity services (e.g. Sensing) in case of roaming and non-roaming scenarios.

-	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

-	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

-	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

# * * * * End of Changes * * * *