3GPP TSG-SA2 Meeting #170	S2-2506578

Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025	(revision of S2-250xxxx)

Source:	LG Electronics

Title:	[WT#1.2, network slicing] Scope and New Key Issue for network slicing

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This contribution proposes scope refinement of WT#1.2 network slicing aspects and new KI for network slicing.

1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

Regarding the following WT#1.2 included in the FS_6G_ARC SID, scope needs to be refined and new Key Issue should be added to TR 23.801-01. Specifically, this contribution focuses on network slicing aspects.

1.2. Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

Architectural Motivation: Network slicing enables the operator to create networks customised to provide optimized service for different market scenarios which demands diverse requirements, e.g. in a specific area, differentiated QoS, network isolation, etc. Operators are attempting to monetize slicing feature for differentiated services in 5GS. Therefore, 6G system should continuously support network slicing.

Gap Analysis: 5GS supports network slicing from the first release and after the first release, additional network slicing features are introduced in each release with multiple options. Some of those new features have requires UE support but specification also specifies how to apply those new features for non-supporting UE by re-using existing mechanisms, which can expedite deploying the new network slicing feature. However, such approach complicated network slicing in 5GS and most of features are not deployed in the network. Therefore, study needs to identify essential feature for 6GS and try to simplify overall complexity of network slicing.

Technical Impact: This WT/KI influence the following to support network slicing:

- Support of simplified network slicing.

- Interaction between UE, CN NFs and RAN node to support network slicing.

# * * * * Start of 1st Change (All New Texts) * * * *

A.X	Work Task 1.2.x

WT#1.2.x:	Study whether and how to simplify network slicing in 6GS.

# * * * * Start of 2nd Change (All New Texts) * * * *

5.X	Key Issue #X: Support of network slicing

This Key Issue studies:

-	Whether and how to simplify network slicing based on following:

-	identify network slicing features needs to be supported by mandatory and optional in the UE and network

-	remove multiple options for the same purpose e.g. multiple NSAC architecture options

-	reduce signalling overhead and configuration of UE and network in non-roaming and roaming scenario

-	support slice interworking with 5GS

# * * * * End of Changes * * * *