3GPP TSG-SA2 Meeting #170	S2-2506580

Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025	(revision of S2-250xxxx)

Source:	LG Electronics

Title:	[WT#1.2, Policy Framework] Scope and New Key Issue for Policy framework

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This contribution proposes scope refinement of WT#1.2 Policy aspects and new KI for Policy enhancements.

1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

Regarding the following WT#1.2 included in the FS_6G_ARC SID, scope needs to be refined and new Key Issue should be added to TR 23.801-01. Specifically, this contribution focuses on policy framework aspects.

1.2. Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

Architectural Motivation: One of goal of 6G system is creating lean and streamlined standards.

Gap Analysis: In 5GS, multiple PCFs are used for a single UE, i.e. PCF for AM policy association, PCF for UE policy association, and PCF for SM policy association. If a UE establishes multiple PDU Sessions, multiple SM PCFs can be used. In some cases, however, network needs to select the same PCF, e.g. for multiple PDU Sessions, for PCF for UE and PCF for PDU Session. In addition, AMF always selects the same PCF for AM policy and UE policy association. Therefore, it is not clear whether separate PCF needs to be used for each policy association. Also current policy framework does not support periodic event based policy control e.g. AF cannot provide a QoS requirement that lasts for an hour and repeats daily. Then, AF needs to send signalling to core network whenever QoS flow needs to be established or removed.

Technical Impact: This WT/KI influence the following to support Policy framework:

- System architecture to support Policy framework for AM policy, UE policy and SM policy associations.

- Efficient policy control for periodic events

# * * * * Start of 1st Change (All New Texts) * * * *

A.X	Work Task 1.2.x

WT#1.2.x:	Study whether and how to simplify and enhance policy framework.

# * * * * Start of 2nd Change (All New Texts) * * * *

5.X	Key Issue #X: Support of Policy framework

This key issue studies the Policy framework for the 6G system architecture i.e. PCF for AM policy association, PCF for UE policy association and PCF for SM policy association. Specifically, this Key Issue studies:

-	Whether an how to simplify policy framework considering

-	roaming and non-roaming scenario

-	pros and cons of using the same PCF for AM policy, UE policy and SM policy associations

-	How to enhance policy framework to support periodic (e.g. daily, weekly) events.

# * * * * End of Changes (All New Texts) * * * *