SA WG2 Meeting #170	S2- 2506585

Goteborg, Sweden, August 25 – 29, 2025

Source:	NTT DOCOMO

Title:	KI#2: Update Sol#26: User plane analytics derivation with efficient UP data collection

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution updates solution #26 for KI#2 by addressing the ENs on the list of the pre-processing instructions and proposes interim agreements for KI#2.

# 1	Discussion

This paper addresses some Editor's notes in Solution #26 and proposes interim agreements.

First, to address the following EN, a new pre-processing instruction utilizing an ML model to be run inside UPF is introduced is introduced.

Editor's note:	List of local data pre-processing instructions sent by the NWDAF to the UPF is FFS.

In the current 5GC design, the NWDAF is the entity responsible for providing analytics to other NFs. While it is under study/investigation to enable the UPF to generate UP analytics locally, the UP analytics are not a completely separate exception from other types of analytics. It is expected that in 5GC, the NWDAF also be able to provide UP analytics to other NFs such as the SMF or PCF as well (indeed, it would be a rather different design if SMF or PCF need to subscribe to UP analytics directly from the UPF).

However, providing UP analytics by NWDAF may cause considerable signalling overhead since significant amount of data may need to be transferred from the UPF(s) to the NWDAF. Solution #26 proposed that the data be locally pre-processed by the UPF(s) based on the instructions that the NWDAF may provide. However, the details of the pre-processing instructions is still FFS. The proposed updates in this paper address the Editor's note via adding an option for local data pre-processing using local ML Model(s) inside UPF.

Assuming that the local model is available in the UPF, to derive the analytics in the inference phase by the NWDAF to the PCF or SMF, the NWDAF instructs the UPF to run the specified model to (pre)process the data locally before sending the output to the NWDAF. The output of this model is used as pre-processed data (e.g., extracted traffic features from raw data), which becomes the input to the ML model in the NWDAF for deriving the UP analytics.

There are two options to provision the local ML model to the UPF. First, the NWDAF may train the ML model by itself and provide it to the UPF (e.g., as a part of local preprocessing instructions). Second, the local ML model may also be trained by UPF under the supervision of the NWDAF. For this purpose, the NWDAF provides an initial model to the UPF. The UPF then partially trains the ML model using local data and sends the intermediate results to the NWDAF. The NWDAF updates its ML model for the analytics and provides feedback (e.g., based on the model's loss function) to the UPF, which uses this feedback in the next round of training. This iterative process continues until the ML models in the UPF and NWDAF meet the required performance or accuracy criteria. Different machine learning techniques such as co-training, joint training, split training, etc., can be used to implement this mechanism, depending on the internal logic and implementation of the NWDAF and UPF.

Regarding the following EN, we add a description stating that statistical values such as averages and variances are calculated for data measured by UPF using values such as average packet size, packet inter-arrival time, flow duration, packet loss rate, jitter, etc.

Editor's note:	More explanation on statistic aggregation of UPF data is required.

Finally, regarding the following EN, we added the impacts on services of the UPF.

Editor's note:	Whether the existing event IDs can be reused for step 6 is FFS.

# 2 Proposal

It is proposed to update TR 23.700-04 according to the following text.

********** First Change **********

## 6.26	Solution #26: User plane analytics derivation with efficient UP data collection

### 6.26.1	High level principles

The high-level principles of the proposed solution are as follows:

-	Training ML models and then performing ML model inference to monitor user plan traffic (e.g. to detect burstiness or traffic abnormalities) need huge amount of packet-level information from the user plane. Collecting and transferring the information from UPF(s) to the NWDAF may cause significant control plane signalling overhead.

-	The existing mechanisms such as Processing & Formatting Instructions or bulk data transfer is not applicable when UPF is the data source. Accordingly, the main idea of the proposed solution is to extend the UPF event exposure services in a similar way that instead of sending raw UP information from the UPF to the NWDAF, the UPF may pre-process the data (in a manner which facilitates the ML model training/inference in the NWDAF) and sends the result of the pre-processing to reduce the signalling overhead.

-	In this solution, to derive UP related analytics using UP data by the NWDAF, the NWDAF sends a data collection request to the UPF, which also may include data pre-processing instructions (including the UPF local ML model(s)) that specify how the information required for model training/inference in the NWDAF should be obtained from the raw (packet-level) data in the UPF.

-	The UPF performs the local data processing (e.g., running local ML model(s)) and reports the locally pre-processed data to the NWDAF based on the received instructions.

-	The local ML model(s) of the UPF may be pre-trained by the NWDAF and provided to the UPF, or locally trained by the UPF under the supervision of the NWDAF.

### 6.26.2	Description

This solution addresses the key issue#2.

In this Release, NWDAF can derive analytics to optimize user plane (UP) performance using information provided by UPF such as UP traffic pattern, traffic information and application traffic measurement information. However, the detailed packet-level raw data of UP traffic for ML model training/inference is huge, so, as the key issue also described, a solution that minimizes the load on control plane (CP) NFs is required. To address this aspect of the issue, solutions would avoid sending large raw (packet-level) data from the UPF to the NWDAF.

One approach to reduce the signalling overhead of data transfer from UPF to NWDAF is to locally process the data in the UPF and send the pre-processed data to the NWDAF. Similar mechanisms are already specified in TS 23.288 [5] including the Formatting/Processing instructions of DCCF and bulk data transfer for NWDAF, DCCF/MFAF, ADRF. However, they are not applicable for UP analytics due to the following reasons:

-	Even if DCCF is used for data collection, the large volume of unprocessed data should be transferred from the UPF to the DCCF that cause significant signalling overhead.

-	The capabilities are specified only for NWDAF, DCCF/MFAF and ADRF. The UPF services do not support such capability to process data before exposing the consumer (e.g. NWDAF).

-	The existing formatting/processing instruction in DCCF are generic but for the UP analytics, the data can be pre-processed in a way that facilitate ML model training and/or inference in the NWDAF, i.e. the pre-processing can be Analytics specific.

Based on these observations, this solution proposes an analytics derivation mechanism based on efficient data collection from the UPF; wherein, the NWDAF sends the data collection request to the UPF, which may also include local data pre-processing instructions that specify local data pre-processing requirements for the UPF to obtain the information required for the analytics. It should be noted that the local data pre-processing instructions may depends on the Analytics ID and corresponding performance metrics (e.g. for a certain level of model accuracy, aggregated information within a relatively large time-window may be sufficient).

The following list is an example of what may be included in the local data pre-processing instructions. Based on the analytics and considering factors such as the trade-off between signalling overhead vs. pre-processing load, etc. different kind of data pre-processing can be performed locally by the UPF following the pre-processing instructions from NWDAF. The NWDAF decide whether to provide instructions or not based on internal logic or operator policy.

-	Data volume instructions: Instructions specifying the amount of data to be sent by UPF e.g. maximum amount of data, samples per data batch, total number of data batches, etc.

-	Sampling instructions: Instructions how to sample the UPF data. Examples of sampling instructions include Sampling Method (Periodic sampling, Event-driven sampling, etc.), Sampling Interval, Sampling Ratio, etc.

-	Statistical feature extraction instructions: Instructions how to statistically aggregate the UPF data. Examples of data aggregation methods include Feature type aggregation (Mean, median, variance, standard deviation, max, min, etc.) For example, this aggregation can be applied to data measured for values such as packet size, packet inter-arrival time, flow duration, packet loss, etc. for QoS flow or PDU level metrics, and time window to compute the features. In other words, it is possible to aggregate measurement data corresponding to a specific flow and calculate aggregate data such as averages and variances.

-	Compression and/or encoding instructions: Instructions for setting up the Compression/Encoding algorithm settings of UPF data

-	Local ML model: The NWDAF may ask the UPF to locally process the data using a specified ML Model(s), and the outputs of the local ML model are used as input of the ML model in the NWDAF. The UPF local model may be pretrained (e.g. by the NWDAF) or trained locally by the UPF under the supervision of NWDAF.

### 6.26.3	Procedures

This clause explains two procedures; the first one is mainly for the inference phase where UPF preprocesses the data locally for example by a trained model which is either already available in the UPF or is provided by the NWDAF. The second procedure addresses the case that the local ML model is going to be trained by the UPF under the supervision/coordination of NWDAF.

Figure 6.26.3-1shows the procedure of the proposed solution where the indirect (via SMF) subscription to the UPF events is used. The same mechanism of providing the local pre-processing instructions to the UPF is also applicable for direct subscription to the UPF event exposure and also when the SMF configures the UPF via PFCP reporting.

Figure 6.26.3-1: User plane analytics derivation with UP data

1.	Analytics consumer requests or subscribes for a UP analytics ID to the NWDAF using either Nnwdaf_AnalyticsSubscription service or Nnwdaf_AnalyticsInfo.

2.	If the request is authorized and in order to provide the requested analytics, the NWDAF determine whether to collect data from UPF for derive the analytics. Based on the analytics ID, corresponding requested ML model performance metric and NWDAF internal logic (considering the NWDAF load, etc.), the NWDAF may also prepare local data pre-processing instructions, which may also include information about local ML model (e.g., a pretrained ML model to be provisioned in UPF, or the ID of the ML model which is already available in UPF).

3.	If the NWDAF determined to collect the UP data in step 2, the NWDAF sends the Nsmf EventExposure_Subscribe with local data pre-processing instructions that indicates the information needed for the analytics e.g. time period, statistical characteristics, local ML model information described as step 2.

4.	The SMF received the request from the NWDAF selects the PDU session(s) and UPF as data sources. The SMF sends the request to the UPF using Nupf EventExposure_Subscribe or N4 PFCP Session Establishment/ Modification Request containing usage Reporting Rules including the local data pre-processing instructions in step 3.

5.	The UPF collects the raw data and then processes the raw data based on the local pre-processing instructions received from the NWDAF to obtain the required information (e.g. by running the ML model trained in the procedure 6.32.3-2 and using the collected local input data).

6.	The UPF sends the locally collected and pre-processed data to the NWDAF using Nupf_EventExposure_Notify.

7.	The NWDAF derives requested analytics using collected data which may also include the output of the UPF local ML model outputs.

8.	The NWDAF provides the analytics using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

Figure 6.26.3-2 shows the procedure of training UPF local ML model(s) under the supervision of (or coordination with) the NWDAF.

Figure 6.26.3-2: Procedure of training UPF local ML model(s)

1.	The NWDAF identifies conditions that trigger (re)training of ML model for UP traffic analytics. Based on the NWDAF internal logic, the training may need training of a local model in the UPF(s) as well (e.g., if the split training is used).

2.	The NWDAF discovers the UPFs that support local model training capabilities via enquiring NRF.

3.	Considering factors such as the UP analytics ID, UPF load, and UPF capabilities, the NWDAF prepares the local Models to be trained in the UPF and selects the appropriate UPFs for ML model training.

4.	The NWDAF provides the initial local model to the UPF.

5.	The NWDAF triggers a new iteration of ML model training via invoking Nupf_LocalTraining_Subscribe which includes the ML Model ID and (optionally) local model update assistance information (e.g., feedback of the NWDAF ML model performance).

6.	The UPF trains the local ML model via using the local data and considering the assistance information provided by the NWDAF.

7.	The UPF provides the intermediate result of training the local model to the NWDAF.

8.	The NWDAF trains the ML model using the UPF local model output as the input and also considering other data sources. If the stopping criteria are not satisfied, the steps 5-8 are repeated.

### 6.26.4	Impacts on services, entities and interfaces

NWDAF:

-	Generates the local pre-processing instructions (including UPF local ML models) based on analytics required from the analytics consumer.

-	Sends the local data pre-processing instructions to the UPF (via SMF).

-	Train ML model using the output of the UPF local model as the input and provide local model training assistance information to the UPF

SMF:

-	Sends the local data pre-processing instructions to the UPF.

UPF:

-	Pre-processes the local data following the instructions from the data consumer.

-	Sends the pre-processed data to the SMF or data consumer.

-	Extending service to send the pre-processed data to the data consumer.

-	Run local ML model to provide input to the NWDAF to derive the UP analytics.

-	Train local ML model using the local data and the assistance information provided by the NWDAF.

********** Second Change **********

### 7.1.Y	Agreed Principles for KI#Y

The following principles are agreed for KI#2:

-	Input data may be collected from the UPF, SMF, AF, OAM, and/or NRF without loading the control plane. NWDAF may include instructions regarding the type and amount of data to be sent to UPF when collecting data.

-	The UPF may use ML model which may be pre-trained (e.g., by the NWDAF) or locally trained by the UPF under the supervision of the NWDAF to pre-process the data locally.

********** End of Changes **********