3GPP TSG-SA2 Meeting #170	S2-2506589

Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025	(revision of S2-250xxxx)

Source:	LG Electronics

Title:	[WT#1.2, SBA framework] Scope and New Key Issue for SBA framework

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This paper proposes scope refinement and new KI for WT#1.2 about SBA framework.

1. Justifications

Regarding the following WT#1.2 included in the FS_6G_ARC SID, scope needs to be refined and new Key Issue should be added to TR 23.801-01.

WT#1.2 Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

Architectural Motivation: The SBA framework enables 5GS to offer flexibility, scalability, and cloud-native deployment. Therefore, it is desirable to reuse the SBA framework in 6G and extend the use of it.

Gap Analysis: Session management control interfaces in 5GS use the traditional point-to-point interface. While the N4 interface is supported by PFCP in 5GS, UPF partially support SBI for event exposure purpose. Using Both SBI and PFCP is not efficient. A study is needed to support SBA framework for the N4 interface.

Technical Impact: This WT/KI would influence the following to enable SBA framework in 6G system:

- 6G architecture based on the SBA framework

- Functions for 6G Session Management Function (SMF) and User Plane Function (UPF).

- Interactions between the SMF and UPF.

- Interactions between the UPF and Network Functions other than SMF

# * * * * Start of 1st Change * * * *

# !! All New Texts !!

A.X.	Work Task 1.2.x

WT#1.2.x:	Study how to reuse and enhance SBA framework and extend SBA over N4 interface.

# * * * * Start of Next Change * * * *

# !! All New Texts !!

5.Y	Key Issue #Y: Enhancement of SBA framework

The SBA framework applied in 5GS enables many advantages such as flexibility, scalability, and cloud-native deployment. Therefore, it would be desirable to support the SBA framework in 6G and extend its use. While the N4 interface is supported by PFCP in 5GS, UPF partially support SBI for event exposure purpose. Using Both SBI and PFCP is not efficient. All the network interfaces in the Core Network should be considered to support the SBA framework in 6G system.

This Key Issue aims to study 6G system to reuse and enhance SBA framework, specifically:

-	How to enable SBA framework over N4 interface

-	Functionalities for 6G Session Management Function (SMF) and User Plane Function (UPF).

-	Interactions between the SMF and UPF.

-	Which NFs can be service consumers of UPF?

# * * * * End of Changes * * * *