SA WG2 Meeting #170	S2-2506590

25 – 29 Aug, 2025, Goteborg, SE

Source:	CATT

Title:	[WT#5] Discussion paper on Data Framework

Document for:	Discussion

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution introduces CATT’s initial views and way forwards for data framework under WT#5 as defined in S2-2506096.

1. Background

1.1	3GPP 6G workshop and NWM discussion

On March 10-11, 2025, 3GPP held a 6G Workshop in Incheon, South Korea, where more than 20 papers from different companies mentioned data-related perspectives, focusing on the design of a unified data framework for 6G system driven by 6G services (e.g., AI services, sensing services, location services, other future services).

In the chair's summary of the 3GPP Workshop on 6G (i.e., 6GWS-250238 manuscript), the following data-related aspects are provided.

- ISAC aspects: Sensing Data Support including sensing data detection, collection, processing, and exposure.

- AI/ML aspects: Regarding 6G for AI, facilitating data collection, transmission, storage, and processing for AI applications.

- Unified Data Management Framework aspects:

  - Complexity in 5G Data Collection: Current approaches, including MDT-based, LPP-based, NWDAF-driven, DCAF-based methods, and UE level to support AI-driven RAN optimizations introduce significant complexity in data gathering across Control Plane and User Plane.

  - Unified Data Management Framework for 6G for cross domain data discovery, collection, storage, transmission, correlation, and exposure for various applications, including:

    - Efficient data detection across diverse sources and types (e.g., sensing data, AI-generated data).

    - Streamlined data collection, storage, processing, and exposure (covering UE, network functions, and third parties).

    - Robust validation mechanisms for data sources and consumers.

    - Implementation of data traceability mechanisms to ensure transparency, security, and trustworthiness in data operations.

    - Efficient data management and transmission protocol

- Security aspects：A generic user consent framework that can apply to any feature where user consent is required, e.g., Sensing, AIML, Positioning, covering the lifecycle management of the data being collected.

- Stage-3 Protocol aspects：Data Protocol to support efficient data management and transmission protocol, the coexistence of different protocols may be considered.

In April 2025, 3GPP SA2 started to discuss the 6G work areas, and the Common Data framework is described as below (see S2-2504414 for details).

Study common data framework for all aspects related to data handling including data collection, distribution, processing, storage and data access, with consideration of access control/user consent and privacy. The example of data may include data of AI and Sensing.

For the following NWM discussion in April, the work area of common data framework collected comments from about 43 companies. Most of them supported data framework as a separate working area and SA2 lead, mentioning the main research content including: data collection, data transmission, data processing, data storage, data exposure, and etc. A few companies think that it may not be possible as a separate work or may be SA5 lead.

Observation#1: Data Framework is an important topic in 6G, most companies support data framework to be a separate work area and led by SA2.

1.2	3GPP 6G progress in SA1 and SA2

In May 2025, 3GPP formed a draft of the 6G SA2 SID and endorsed it at the SA#108 meeting in June (SP-250806), with the following description of the data framework.

WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework.

NOTE 6:	The work split with SA3, SA5 and RAN WGs will require TSG coordination.

Meanwhile, in May 2025, 3GPP SA1 discussed an efficient data collection and consumption for 6G system use case and some potential requirements, and drafted the definitions of terms for 6G system data in TR 22.870 v0.3.1, as follows.

6G System Data: the data that is generated, and controlled by 6G system.

NOTE 1: 6G system data is different from traditional user traffic data which is application level data being transmitted through the 3GPP system for user related services.

Table 5.9.2.1-1: Heterogeneous 6G System Data

| Use case category | Data types | Data provider | Data Consumer | Data volume |
| --- | --- | --- | --- | --- |
| Sensing data collection/sensing result exposure (e.g. measurement) | Sensing measurement data requires further study and discussion. The potential sensing measurement can be channel estimation results, delay, Doppler, angle, signal strength, etc. | UE, RAN | Sensing Function | Related to use cases: e.g. data rate of measurement is about 10Mbps for objects creating hazards on roads |
| AI/ML data collection / analytics exposure (e.g. AI/ML for air interface, AI/ML for core network) | - For RAN data collection for Air interface, see TR 38.843 [243] - For core network data collection, see the data collection framework in clause 6.2, TS 23.288 [114], and the detailed data type are defined from clause 6.3 to clause 6.21, TS 23.288 [114]. | CN, RAN, UE, AF | CN, RAN, UE | Typically, the internal data collection for AI model training yields tens of thousands of samples. Related to AI model: e.g. the data volume is about 10k bytes to 100M bytes for AI model of beam management |
| Energy Efficiency (EE information exposure to AF) | still under the discussion in SA2 R19 EnergySys WID | CN, OAM | EIF (Energy Information Function) | Related to use cases: e.g. could be similar to data volume for charging |
| Positioning, | See clause 6, TS 23.273 [240] | UE, RAN | LMF, UE | Related to use cases |
| Network Exposure in core network | See clause 5. 20, TS 23.501 [140], and clause 4.15, TS 23.502 [30] | 5GC NF, AF | 5GC NF, AF | Related to use cases |
| … |  | … | … | … |

[PR 5.9.2.2-1] The 6G system shall support mechanisms for 6G System Data collection and consumption minimizing the impact to 6G services.

[PR 5.9.2.2-2] Subject to operator’s policy, regulation and user consent, the 6G system shall support processing of 6G System Data.

NOTE: examples of data processing are use case dependant, e.g. data fusion, data anonymization and data analysis.

[PR 5.9.2.2-3] Subject to user consent, regulation and operator's policy, the 6G system shall support secure means to expose 6G System Data to authorized trusted third-party, authorized network function or authorized UE.

[PR 5.9.2.2-4] The 6G system shall support security and privacy protection of the 6G System Data.

[PR 5.9.2.2-5] The 6G system shall be able to provide charging and accounting mechanisms for 6G System Data.

In June 2025, the 3GPP RAN#108 meeting endorsed RP-251881 with the following description:

- (3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

  - e)	Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

- (8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

  - ii.	Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Observation#2: There is a preliminary consensus on new data types emerging from 6G between the 3GPP SA1, SA2 and RAN WGs, including, e.g. AI data, sensing data, and network status data.

2. CATT’s initial views and way forwards

2.1	New services driven

The core objective of the mobile communication system defined by 3GPP (e.g., 5G System, 5GS) is to enable User Equipment (UE) to access the Data Network (DN) and to be able to use the data services provided by the network. To this end, 5GS defines Control Plane (CP) to support user access and control the signalling interactions within the system, and User Plane (UP) to support data transmission and QoS guarantee for user traffic data.

For 6G, SA1 defines the 6G System Data as above. 6G System Data, such as, Sensing data, AI data, NF context, is different from user traffic data. This type of data not only requires the 3GPP system to deliver application level data transparently between the UE and the DN, but presents some new requirements.

Examples of use cases for different data are given below:

- AI data：

  - AI data: data related to AI models/algorithms, which can be used to improve network performance and/or provide AI services (e.g. AI inference) for applications.

  - AI data including: e.g. raw data or processed datasets for training/testing/validating AI models, AI models and parameter data, AI inference input data and output data (e.g. AI analytic results).

  - Use cases: support AI model training data collection from UE, RAN, CN NFs (e.g. UPF, ADRF, other NFs) to Core Network for further processing to generate AI data set, AI model training, AI inference to generate analytic results, and/or storing for future use [refer to TR22.870 clause 5.9.2].

- Sensing data：

- Sensing data: data related to object or environment of interest for sensing purposes, which can be derived from 3GPP radio signals and also can be provided by non-3GPP devices.

- Sensing data including: e.g. sensing measurement data generated by sensing entities with sensing capabilities (e.g. UE, BS, and Non-3GPP devices), and sensing results generated after processing sensing data within the 6G system.

- Use cases: support sensing measurement data collection from UE, RAN to Core Network for further processing to generate sensing results and/or storing for future use [refer to TR22.870 clause 5.9.2, clause 5.4.6].

- NF context：

  - NF context: the “context” storage resource of the NF Service, which is fully or partially decoupled from the “compute” resource of the NF Service, e.g. UE context in 5G AMF, SM context in 5G SMF.

  - Use cases: support data sharing between different manufacturers or different places of the same manufacturer to avoid data silos; support trusted exposure of context data, such as expose network state data to trusted third parties or UEs.

Proposal#1: Regarding the data scope of Data Framework, considering the 6G System Data as defined by SA1, including e.g. AI, Sensing, and NF context. That is, the Data framework is designed for 6G System Data.

2.2	5G system gap analysis

Considering the following two aspects:

- Case-by-case data handling via CP/UP in 5GS

Beyond connectivity services (e.g. Sensing services, AI services) bring a large amount data (called 6G System Data) and some new data handling requirements. 6G System Data requires that the 6G system supports flexible and efficient data collection and processing for the data from different data sources (e.g., UEs, RANs, CN NFs) based on data consumer requirements.

However, traditional CP, designed for signalling delivery, which is not suitable for large amount of data transmission. UP as “pipeline” supports the data transmission between UE-UPFs for user plane traffic, and cannot well support the new transmission topology requirements (e.g., direct data transmission between RAN-CN) and on-demand data processing requirements.

At the same time, the current 5G data handling methods are varied and across different use cases (e.g., NWDAF-based for AI data handling, LPP-based UE positioning data handling, BS Sensing data handling is under discussion in R20), which introduces complexity to the architecture design, the case-by-case solutions have poor scalability support for future new services.

- Data storage architecture in 5G

For supporting "stateless" NFs, 5G support the "compute" resource is decoupled from the "storage" resource of the NF Service. However, the compute/storage separation of 5G is not thorough enough, for example, the context data in 5G AMF/SMF is still unstructured, and a large amount of data is still dispersed in different 5GC NFs, which leads to insufficient data sharing among different manufacturers and inefficient data collection methods.

5G utilizes UDRs and UDSFs to support the storage of network data, such as in UDM/PCFs and AMF/SMFs, and ADRFs to support the storage of the collected and analytic data in the core network. For 6G, more data such as AI data and Sensing data will be generated and controlled by system, the 5G data storage architecture needs to be redesigned to facilitate data monetization via unified data management and control.

Therefore, a common data framework is needed for 6G to serve multiple use cases and easy extend to support future new services, including, e.g. to improve data collection efficiency and avoid duplication data collection, to support efficient and flexible data transmission between UE, RAN, and CN NFs, to support on-demand data processing of different service logic, to support efficient data storage for future use, to support data exposure for data monetization.

Proposal#2: The Data Framework provides generic data functionalities and service interfaces at the system level for different services and across UE, RAN, and CN domains. The data functionalities including, e.g. data management and control, data collection, data transmission, data processing, data storage and data exposure.

2.3 Coordination between SA2 and SA3/SA5/RAN WGs

Since the data framework involves all aspects related to data handling, the study is large and has some relationship with SA3, SA5 and RAN WGs.

In the process of data collection, processing, storage and exposure, the user consent, authorization, authentication, privacy protection, the confidentiality and integrity during data transmission, data access control are necessary. Also, the operator's unified management and control of the data, including e.g. where the data comes from, where it goes to, and what is processed, are all needed to be considered. Therefore, access control/user consent and privacy aspects will be coordinated with SA3.

Regarding the work split on the data framework between SA2 and SA5. SA5 has discussed data management in 5G and also designs a data management system for Management Data. The Management data for 5G is classified into performance measurements, end-to-end key performance indicators and trace metrics, which are irrelevant to UEs and for the operation of the network. But for this data framework in 6G system across UE, RAN and CN domains, considering the real-time and the dynamic of data type (e.g. sensing data, AI data), data provider and data consumer (e.g. UE, RAN, CN NF), it should be studied by SA2.

Proposal#3: The data framework should be studied by SA2 and coordinated with SA3, SA5, RAN WGs, including e.g. access control/user consent and privacy aspects will be coordinated and aligned with SA3, charging and OAM aspects will be coordinated with SA5, Radio interface, RAN-CN interaction aspects will be coordinated and aligned with RAN WGs.

2.4 Coordination between WT#5 and other WTs

The WT#5 aims to study the data framework for 6G System with the considerations of different use cases (e.g. AI, Sensing services) and the data-related service requirements from SA1. The Data framework is designed to provide generic functionalities and service interfaces for different services. Therefore, this WT will to coordinate with other service-related WTs, such as WT#3, WT#4. More specifically, service-agnostic functionalities should be designed in this WT and service-specific functionalities will be coordinated and aligned with other WTs.

Considering the data transmission during the data collection, data processing, data storage and data consumption, the QoS and Policy of the data framework should be considered, and it will be coordinated and aligned with WT#1.2.

Proposal#4: The data framework focus on service-agnostic functionalities, interfaces and procedures design, the service-specific functionalities will be coordinated and aligned with other WTs (e.g. WT#3, WT#4), the QoS & Policy of data framework will be coordinated and aligned with WT#1.2.

3. Conclusions and Proposals

Based on the analysis above, CATT would like to provide the following proposals as way forward to be considered during Rel-20 6G SID discussion:

Proposal#1: Regarding the data scope of Data Framework, considering the 6G System Data as defined by SA1, including e.g. AI, Sensing, and NF context. That is, the Data framework is designed for 6G System Data.

Proposal#2: The Data Framework provides generic data functionalities and service interfaces at the system level for different services and across UE, RAN, and CN domains. The data functionalities including, e.g. data management and control, data collection, data transmission, data processing, data storage and data exposure.

Proposal#3: The data framework should be studied by SA2 and coordinated with SA3, SA5, RAN WGs, including e.g. access control/user consent and privacy aspects will be coordinated and aligned with SA3, charging and OAM aspects will be coordinated with SA5, Radio interface, RAN-CN interaction aspects will be coordinated and aligned with RAN WGs.

Proposal#4: The data framework focus on service-agnostic functionalities, interfaces and procedures design, the service-specific functionalities will be coordinated and aligned with other WTs (e.g. WT#3, WT#4), the QoS & Policy of data framework will be coordinated and aligned with WT#1.2.