3GPP TSG-SA2 Meeting #170	S2-2506591

Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025	(revision of S2-250xxxx)

Source:	LG Electronics

Title:	[WT#7, NTN] Scope and New Key Issue to Support 6G RAT for NTN

Document for:	Approval

Agenda Item:	20.6.7

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This paper proposes scope refinement and new KI for WT#7 about the support of 6G RAT for NTN, and support service continuity aspects.

1. Justifications

Regarding the following WT#7 included in the FS_6G_ARC SID, scope needs to be refined and new Key Issue should be added to TR 23.801-01.

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects. NOTE 8: The detailed scope for WT#7 will be coordinated and aligned with RAN

Architectural Motivation:

The support of TN with NTN in 6G system can provide ubiquitous connectivity and service continuity with the consistency of user experience in all locations, such as remote and sparsely populated area, as well as airspace for aerial operations.

Gap Analysis:

The 5GS architecture was not natively supported NR NTN, but the 6G system architecture shall natively support 6G RAT for NTN.

Technical Impact: This WT/KI would influence the following to support 6G RAT for NTN, and support service continuity aspects:

- System architecture to support 6G RAT for NTN

- Interworking with 4G/5G system

# * * * * Start of 1st Change * * * *

# !! All New Texts !!

A.X.	Work Task 7

WT#7.	Study how to support 6G RAT for NTN in 6G system, and how to support service continuity between (N)TN and NTN

NOTE 1:	This KI requires consideration of relationship and dependency with WT for migration and interworking.

NOTE 2:	This KI requires coordination with RAN WGs.

# * * * * Start of Next Change * * * *

# !! All New Texts !!

5.Y	Key Issue #Y: Support 6G RAT for NTN

The support of TN with NTN in 6G system can provide ubiquitous connectivity and service continuity with the consistency of user experience in all locations.

This Key Issue aims to study 6G system to support 6G RAT for NTN, specifically:

-	How to support service continuity between (N)TN and NTN

NOTE 1:	This KI requires consideration of relationship and dependency with Key Issue #X for migration and interworking.

NOTE 2:	This KI requires coordination with RAN WGs.

# * * * * End of Changes * * * *