3GPP TSG-WG SA2 Meeting #170	S2-2506609

25 - 29 August, 2025, Goteborg, Sweden	(revison of xxxx)

Source:	vivo

Title:	New solution for KI#4: Sensing Data and the Associated Information Collection and Transport via data tunnel

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes a new solution for KI#4: Sensing Data and the Associated Information Collection and Transport of the FS_Sensing_ARC TR 23.700-14.

1	Discussion

This paper proposes a new solution to address KI#4 Sensing Data and the Associated Information Collection and Transport.

Figure 1: System Architecture to support Sensing Services in reference point representation

As shown in this reference point architecture, the sensing signalling is transported based on control plane, while the sensing data is transported based on the new sensing data transport tunnel between sensing entity and the sensing function.

The solution is based on the following principles to support sensing data and associated information collection and transport:

1)	The sensing signalling is transported based on control plane, while the sensing data (i.e. sensing measurements data) and associated information (e.g. Sensing Service ID) are transmitted from the Sensing Entity to the Sensing Function via data tunnel.

2)	The data tunnel establishment procedure is based on control plane signalling interaction.

3)	The Sensing Function and Sensing Entity (i.e. gNB) exchange their tunnel information (e.g. TEID (Tunnel Endpoint ID) and/or IP address) for data tunnel establishment.

Based on the above principles, an end-to-end procedure for KI#4 is provided.

2	Proposal

It is proposed to include the following changes in TR 23.700-14 V0.2.0.

* * * * Start of Changes * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | X |  | X | X | X |  |
| #2 |  | X |  |  | X |  |
| #3 | X | X | X | X | X | X |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  | X |  |  |
| #6 | X | X | X |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |
| #10 |  | X |  |  |  |  |
| #11 |  | X |  |  |  |  |
| #12 |  | X |  |  |  |  |
| #13 |  |  | X |  |  |  |
| #14 |  |  | X |  |  |  |
| #15 |  |  | X |  |  | X |
| #16 |  |  | X |  |  |  |
| #17 |  |  | X |  |  |  |
| #18 |  |  |  | X |  |  |
| #19 | X | X |  |  | X |  |
| #20 |  |  |  |  |  | X |
| X |  |  |  | X |  |  |

* * * * Next Changes (All text new) * * * *

6.X	Solution #X: Sensing Data and the Associated Information Collection and Transport via data tunnel

6.X.0	High-level solution Principles

The solution is based on the following principles to support sensing data and associated information collection and transport via data tunnel:

1)	The sensing signalling is transported based on control plane, while the sensing data (i.e. sensing measurements data) and associated information (e.g. Sensing Service ID) are transmitted from the Sensing Entity to the Sensing Function via data tunnel.

2)	The data tunnel establishment procedure is based on control plane signalling interaction.

3)	The Sensing Function and Sensing Entity (i.e. gNB) exchange their tunnel information (e.g. TEID (Tunnel Endpoint ID) and/or IP address) for data tunnel establishment.

6.X.1	Description

This solution is to address KI#4 Sensing Data and the Associated Information Collection and Transport.

6.X.2	Procedures

Figure 6.x.2-1: Procedure for Sensing Data and the Associated Information Collection and Transport

0.	The Sensing Function selects the Sensing Entity and determines to establish data tunnel for sensing data transport. The Sensing Function allocates its SF tunnel information used for data tunnel establishment. The SF tunnel information includes the tunnel end point information and/or the address of the Sensing Function (i.e., TEID and/or IP address).

1.	The Sensing Function sends the Sensing service request including the SF tunnel information to the selected Sensing Entity (i.e. gNB) via the AMF.

2.	The AMF sends the Sensing service request including the SF tunnel information to the Sensing Entity via N2 message.

3.	After receiving the Sensing service request including the SF tunnel information, the Sensing Entity allocates its SE tunnel information used for data tunnel establishment. The SE tunnel information includes the tunnel end point information and/or the address of the Sensing Entity (i.e., TEID and/or IP address). The Sensing Entity sends a Sensing service response including SE tunnel information and operation execution result indication to Sensing Function. The operation execution result indication indicates whether the Sensing service request is accepted or not.

4.	The AMF sends the Sensing service response received in step 3 to the Sensing Function.

5.	After exchanging the tunnel information, a data tunnel is established between the Sensing Function and the Sensing Entity for sensing data and the associated information collection and transport.

6.	The Sensing Entity performs sensing operations to obtain sensing measurements data.

7.	After the sensing measurements data obtention, the Sensing Entity sends the sensing measurements data and the associated information to the Sensing Function via the established data tunnel. The associated information includes the Sensing service ID determined by the Sensing Function for binding the sensing measurements data and requested Sensing service.

8.	The Sensing Function performs sensing result generation based on the sensing measurements data and associated information received in step 7.

6.X.3	Impacts on services, entities and interfaces

The solution has impacts on the following entities:

Sensing Function:

-	Support to provide the SF tunnel information to Sensing Entity via the AMF.

-	Support to receive the SE tunnel information from the Sensing Entity via the AMF.

-	Support to receive sensing data and the associated information from the Sensing Entity via the data tunnel.

Sensing Entity:

-	Support to receive the SF tunnel information from the Sensing Function.

-	Support to provide the SE tunnel information to the Sensing Function.

-	Support to establish the data tunnel with the Sensing Function using the tunnel information received from the Sensing Function.

-	Support to transfer sensing data and the associated information to the Sensing Function via the data tunnel.

AMF:

-	Support to send the SF tunnel information received from the Sensing Function to the Sensing Entity via N2 message.

-	Support to send the SE tunnel information received from the Sensing Entity to the Sensing Function via SBI message.

* * * * End of Changes * * * *