3GPP TSG-SA2 Meeting #170	S2-2506617

Goteborg, Sweden, August 25 – 29, 2025	(revision of S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6364 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on S-NSSAIs supported by MWAB-gNB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During the NG Setup and RAN Configuration Update procedures, the MWAB-gNB has to provide its supported S-NSSAIs to the AMF as excerpted below from TS 38.413. >>>>TAI Slice Support List; M; Slice Support List 9.3.1.17; Supported S-NSSAIs per TAC, per PLMN or per SNPN.; -. There may be the case that some of S-NSSAIs used for BH PDU Sessions are NOT allowed in the BH PLMN/SNPN. This means the S-NSSAI in the MWAB Broadcasted PLMN/SNPN associated with such not-allowed S-NSSAI for BH PDU Session cannot be supported by the MWAB-gNB. In this case, providing from the MWAB-gNB to the AMF that all S-NSSAIs are supported by the MWAB-gNB is considered inappropriate from the point of view about serving UEs. But, it is unclear how the MWAB-gNB can identify/know which S-NSSAI(s) for BH PDU Sessions are allowed or not in the BH PLMN/SNPN and whether the MWAB-gNB provides S-NSSAI(s) supported by the MWAB-gNB to the AMF reflecting the situation about the S-NSSAI(s) for BH PDU Sessions allowed in the BH PLMN/SNPN. Therefore, clarification is needed regarding aspects on determining S-NSSAIs supported by MWAB-gNB and providing them to AMF. |
| Summary of change: |  | §5.49.1.4 - Clarify that the MWAB-gNB determines the S-NSSAI(s) supported by itself in the MWAB Broadcasted PLMN/SNPN based on the information on the Allowed S-NSSAI(s) provided by the MWAB-UE, and provides its supported S-NSSAI(s) to the AMF(s) it connects to. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Aspects on determining S-NSSAIs supported by MWAB-gNB and providing them to AMF are unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

#### 5.49.1.4	Support of network slicing for UEs served by a MWAB

A MWAB-gNB can be configured to associate S-NSSAI(s) in the MWAB Broadcasted PLMN/SNPN to the traffic descriptor for selection of the BH PDU Sessions parameters e.g. use dedicated S-NSSAI in BH PLMN/SNPN. If no such association exists for a specific S-NSSAIs of the MWAB Broadcasted PLMN/SNPN, the MWAB-gNB associates this S-NSSAI to default traffic descriptors for the selection of BH PDU Session(s) for the related N2 and/or N3 backhauling. The MWAB-gNB requests the BH PDU sessions as specified in clause 5.49.1.2.

When the MWAB-gNB obtains the S-NSSAI of a UE PDU Sessions from the UE PDU session context during PDU session establishment, it checks whether a configured association exists for the S-NSSAI of the PDU session of the UE and, if so, it uses the related BH PDU session for the UE's PDU Session.

If a new BH PDU Session needs to be established for supporting a UE's traffic for a S-NSSAI in the MWAB-Broadcasted PLMN/SNPN, after the completion of the PDU session establishment procedure, the MWAB-UE may trigger a PDU session resource modification procedure to shift the N3 traffic for the UE to the new BH PDU sessions.

NOTE:	Due to the limitation of the maximum number of PDU sessions supported by a UE, it is expected that there can only be a small number of dedicated BH PDU sessions for specific S-NSSAI(s) in the MWAB-broadcasted PLMN.

The MWAB-UE provides or updates information on the Allowed S-NSSAI(s) in the BH PLMN/SNPN to the MWAB-gNB. Based on this information from the MWAB-UE and the configured association of S-NSSAI(s) in the MWAB Broadcasted PLMN/SNPN to the traffic descriptor for selection of the BH PDU Sessions parameters, the MWAB-gNB determines the S-NSSAI(s) supported by the MWAB-gNB in the MWAB Broadcasted PLMN/SNPN. The MWAB-gNB provides or updates the determined S-NSSAI(s) as supported S-NSSAI(s) of MWAB-gNB to the AMF(s) it connects to by NG Setup and RAN Configuration Update procedures as specified in TS 38.401 [42]. Based on the supported S-NSSAI(s) of the MWAB-gNB, the AMF(s) may provide or update the Allowed S-NSSAI(s) to the serving UE(s).

* * * * End of Changes * * * *