3GPP SA WG2 Meeting #170	S2-2506629

Goteborg, Sweden, 25 – 29 August 2025	(revision of S2-25xxxxx)

Source: 	LG Electronics

Title: 	[KI#2, 5] Solution#2 update

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper proposes to update Solution#2 in TR 23.700-14.

1	Discussion

This paper proposes to resolve some ENs as below:

Editor's note:	It is FFS about whether and how to support authorization of further level of granularity than PDU Session level.

⇨	The UE subscription data needed contains 'Sensing Service Authorization' indicating whether the UE is authorized to use sensing services as a Sensing Service Consumer.

-	The AMF checks if the UE is authorized as a Sensing Service Consumer based on the Sensing Service Authorization information in the UE subscription data to determine that the AMF can support sensing service related operation for the UE.

-	The SSF checks if the UE is authorized as a Sensing Service Consumer based on the Sensing Service Authorization information in the UE subscription data.

Editor's note:	It is FFS about whether and which Sensing Service subscription data the SSF retrieve from the UDM.

⇨	In addition to the 'Sensing Service Authorization' information, the UE subscription data may contain the following information:

i)	(optional) Allowed sensing service operation(s) for the UE to request sensing service, e.g. object detection and tracking, environment monitoring, motion monitoring. If not exist, all sensing service operations are allowed for the UE.

ii)	(optional) Allowed area(s) for the UE to operate as a Sensing Service Consumer. If not exist, no restricted area for the UE.

iii)	(optional) Allowed time period(s) for the UE to operate as a Sensing Service Consumer. If not exist, no restricted time period for the UE.

The SSF can check validity of a sensing service request from the UE based on the above subscription information.

Editor's note:	It is FFS about details on how to use mechanism for establishing a secured user plane connection with LMF to establish a user plane connection with the SSF.

⇨	Details are proposed to be added including authorization for the UE and the correlation of the UE with this secured user plane connection using Binding ID performed by the SSF.

2	Text Proposal

It is proposed to agree the following changes into TR 23.700-14v0.2.0.

* * * * Start of 1st Change * * * *

6.2	Solution #2: Authorization to support sensing service request from UE and sensing result exposure to UE

6.2.0	High-level Solution Principles

High-level principles of this solution are as below:

a)	The UE operates as a Sensing Service Consumer while requesting sensing result to the core network.

b)	User plane connection is used between the UE and the core network for bullet a).

c)	This solution mainly focuses on how the UE is authorized to operate as a Sensing Service Consumer.

d)	For authorizing the UE to establish a PDU Session used for Sensing Service, the existing network slicing mechanisms as specified in clause 5.15 of TS 23.501 [3] are used.

e)	For authorizing the UE to operate as a Sensing Service Consumer, the UE subscription data contains 'Sensing Service Authorization' indicating whether the UE is authorized to use sensing services as a Sensing Service Consumer.

6.2.1	Description

The UE can request the core network to provide sensing result to the UE. Clause 5.23 "Use case on AMR collision avoidance in smart factories" of TR 22.837 [7] describes that the mobile system provides the sensing result to AMR (Autonomous Mobile Robot).

This solution proposes the following for UE to request the core network to provide sensing result and for the core network to provide sensing result to the UE:

1)	New NF called Sensing Support Function (SSF) that is a core network function supports interaction with the UE to receive sensing service request and provide sensing service result.

2)	The UE and the SSF communicate and interact each other via user plane for sensing service request and sensing result exposure.

3)	For a PDU Session to connect to the SSF, dedicated S-NSSAI and DNN are used.

4)	The UE is authorized based on its subscription data as below:

4-1)	Authorization for PDU Session using the dedicated S-NSSAI for Sensing Service.

-	The authorization of the UE to establish connectivity with the SSF for Sensing Service is based on whether dedicated S-NSSAI for Sensing Service is received as part of Allowed NSSAI following existing procedures as described in clause 5.15 of TS 23.501 [3].

-	To revoke the authorization of UE's Sensing Service, the core network removes the dedicated S-NSSAI for Sensing Service from the UE subscription data. Then, the dedicated S-NSSAI for Sensing Service is removed from Allowed NSSAI.

4-2)	Authorization for the UE to operate as a Sensing Service Consumer.

-	The UE subscription data needed by the AMF and SSF contains 'Sensing Service Authorization' indicating whether the UE is authorized to use sensing services as a Sensing Service Consumer.

-	The AMF checks if the UE is authorized as a Sensing Service Consumer based on the Sensing Service Authorization information in the UE subscription data in order to determine that the AMF can support sensing service related operation for the UE.

-	The SSF checks if the UE is authorized as a Sensing Service Consumer based on the Sensing Service Authorization information in the UE subscription data to determine that the SSF can support sensing service related operation for the UE.

5)	Address of the SSF (e.g. an FQDN) can be configured in the UE or provided by the core network (e.g. via Registration procedure, via Sensing Service related policy provisioning, due to UE request). Mechanism for establishing a secured user plane connection with LMF as specified in clause 6.18 of TS 23.273 [4] can be used to establish a user plane connection for Sensing Service with the SSF.

6.2.2	Procedures

Figure 6.2.2-1: Procedure for authorization to support sensing service request from UE and sensing result exposure to UE

1.	The UE sends a Registration Request message to the AMF. The UE requests the S-NSSAI dedicated for Sensing Service.

2.	The AMF obtains UE subscription data from the UDM. The UE subscription data includes the dedicated S-NSSAI and DNN for Sensing Service. The UE subscription data also includes the Sensing Service Authorization information as described in clause 6.2.1.

The AMF subscribes to be notified from the UDM when the subscription data requested is modified.

3.	The AMF sends a Registration Accept message to the UE. The Registration Accept message includes the dedicated S-NSSAI for Sensing Service as part of Allowed NSSAI.

4.	The UE sends a user plane connection establishment request to the AMF via a UL NAS TRANSPORT message, if there is no established secure user plane connection between the UE and SSF.

5.	The AMF selects an SSF which is capable to establish a user plane session for sensing services with the UE if the UE is authorized as a Sensing Service Consumer based on the UE subscription data. Otherwise, the AMF rejects the request.

One of the following is used to select a proper SSF by the AMF:

-	SSF ID in the UE subscription data, if exist.

-	Locally configured SSF.

-	Querying the NRF.

6.	The AMF sends a Nssf_Sensing_UPConfig Request towards the SSF to request set up of a user plane connection for the UE. The AMF shall include the target UE identity (SUPI and/or GPSI) in the request.

7.	The SSF obtains UE subscription data from the UDM. The UE subscription data includes the Sensing Service Authorization information as described in clause 6.2.1. The SSF checks if the UE is authorized as a Sensing Service Consumer based on the Sensing Service Authorization information in the UE subscription data.

The UE subscription data may contain the following sensing service operation validity information:

i)	(optional) Allowed sensing service operation(s) for the UE to request sensing service, e.g. object detection and tracking, environment monitoring, motion monitoring. If not exist, all sensing service operations are allowed for the UE.

ii)	(optional) Allowed area(s) for the UE to operate as a Sensing Service Consumer. If not exist, no restricted area for the UE.

iii)	(optional) Allowed time period(s) for the UE to operate as a Sensing Service Consumer. If not exist, no restricted time period for the UE.

The SSF subscribes to be notified from the UDM when the subscription data requested is modified.

8.	If the UE is authorized as a Sensing Service Consumer and there is no established secure user plane connection between the UE and SSF, the SSF sends a user plane information to the AMF to indicate the UE to accept and utilize user plane for sensing service. The user plane information includes address of the SSF (e.g. an FQDN) and Binding ID. The SSF allocates a Binding ID to associate the user plane connection to be established with the target UE. The SSF associates the target UE identity (SUPI and/or GPSI) with the Binding ID.

9.	When the AMF receives the user plane information from the SSF in step 8, the AMF forwards it to the UE via a DL NAS TRANSPORT message.

NOTE 1:	The UE can send a user plane connection establishment request to the AMF via a Registration Request message instead of a UL NAS TRANSPORT message. In this case, steps 5 to 8 are performed during the registration procedure and the user plane information described in step 8 is sent to the UE via a Registration Accept message.

10.	The UE establishes a PDU Session for Sensing Service by using the dedicated S-NSSAI and DNN for Sensing Service based on the URSP.

NOTE 2:	Step 10 can be performed before step 4.

11.	The UE establishes a secure connection for Sensing Service with the SSF over user plane via the PDU Session established in step 10.

After the secured user plane connection has been established successfully, the UE sends the Binding ID received in step 9 to the SSF via the secured user plane connection to enable the SSF to perform the correlation of the UE with this secured user plane connection. The Binding ID will be released once the correlation is complete.

NOTE 3:	Details of steps 12 to 15 are not covered by this solution.

12.	The UE sends a Sensing Service Request message to the SSF to obtain sensing result. The Sensing Service Request message includes sensing service type (object detection, object tracking, environment sensing, etc) and reporting mode (e.g. one time, periodic or event triggered).

If the UE subscription data contains one or more of bullets i), ii) and iii) described in step 7, the SSF checks whether the request from the UE is valid. If the request is valid, steps 13 to 15 are executed. Otherwise, the SSF sends a reject response to the UE.

13.	The SSF collects sensing data and the associated information.

14.	The SSF determines sensing result.

15.	The SSF provides sensing result to the UE.

Editor's note:	Any potential privacy aspects regarding sensing result exposure to the UE without involving NEF/AF and its feasibility from security perspective will require coordination with SA WG3.

Editor's note:	Sensing result exposure to the UE needs to be checked with SA WG1.

6.2.3	Impacts on Services, Entities and Interfaces

SSF (New NF that resides in core network):

-	Supports Sensing Service and the related operations.

-	Supports to handle sensing service request from UE and sensing result exposure to UE.

UE:

-	Supports for sensing service request to SSF and sensing result from SSF over user plane.

UDM:

-	Supports dedicated S-NSSAI and DNN for Sensing Service in UE subscription data.

-	Supports UE subscription data for Sensing Service including Sensing Service Authorization information and sensing service operation validity information.

AMF:

-	Supports sensing service related operations including SSF selection, forwarding the user plane information for sensing service received from the SSF to the UE.

* * * * End of Changes * * * *