SA WG2 Meeting #170	S2-2506634

25 – 29 Aug, 2025, Goteborg , SE

Source:	Nokia

Title:	[WT#8] – 6G IoT

Document for:	Approval

Agenda Item:	20.6.8

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: The contribution proposes the scope of WT#8.

# 1. Justifications

The SID on Architecture for 6G System (S2-2506096) includes the following WT#8:

- WT#8: Study whether and how to support cellular IoT enablers in 6G, based on RAN decision for 6G IoT.

NOTE 9:	The detailed scope for WT#8 will be coordinated and aligned with RAN. Ambient IoT is not in the scope of the study.

Architectural Motivation:

6G networks are expected to support a wide spectrum of IoT device categories — from ultra-low-power sensors to more capable industrial devices — across diverse verticals. To achieve cost efficiency and ecosystem scalability, maximum feature commonality across device types is essential. At the same time, some requirements applicable for IoT devices such as extended battery life, secure and efficient access control, and optimized handling of small, infrequent data transfers remain critical.

Gap Analysis:

While 5G already provides features such as extended idle mode DRX, lightweight data transfer for IoT, and basic access control frameworks, gaps remain in supporting:

- Advanced UE power saving mechanisms that enable extended battery life and improved energy efficiency for various IoT device categories, with adaptability to different service requirements and operating conditions.

- Flexible and fine-grained access control suitable for massive numbers of IoT devices. 5G access control is supported mostly at subscription/profile level, lacking dynamic, scalable control for large IoT fleets.

- Highly optimized small and infrequent data transfer procedures that reduce signaling and energy overheads. In 5G, the CIoT CP/UP optimization mechanism exists, but it is uncertain if they will be sufficient and still applicable for 6G.

Technical Impact:

- Overall system architecture (e.g., reference points, functional models)?

  - Updates to functional models and reference points to incorporate unified low-power operation control, dynamic access control hooks, and lightweight small-data transfer procedures.

- Interaction between network functions/services?

  - Harmonized interactions between AMF, SMF, and application exposure functions to support proposed features for both IoT-specific and general-purpose devices.

NOTE:	Coordination with SA3 and RAN WGs is foreseen to address any security related topics and RAN related signalling, respectively.

**** First Change ****

# 2. WT Scope

# Annex A.X. WT#8

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

It is proposed to study the following aspects to support IoT devices in 6G:

- Study advanced UE power saving mechanisms for 6G, enabling longer battery life and improved energy efficiency across various IoT device categories.

- Study scalable access control applicable to massive IoT fleets.

- Study lightweight procedures for small-size, infrequent data transfer, reducing signalling overhead and enabling efficient energy use.

NOTE 1:	The enhancements from this WT should be designed to be reusable across different device types (e.g., both 6G regular UEs and 6G IoT UEs).

NOTE 2:	Coordination with SA3 is required for any security and privacy related topics.

NOTE 3:	Coordination with RAN WG is required for any features related to RAN procedures.

**** Second Change ****

# 3. Potential KI(s) Scope

# 5.X. Key Issue #X: IoT support in 6G network

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

It is proposed to study the following aspects to support IoT devices in 6G:

- Study advanced UE power saving mechanisms for 6G, enabling longer battery life and improved energy efficiency across various IoT device categories.

- Study scalable access control applicable to massive IoT fleets.

- Study lightweight procedures for small-size, infrequent data transfer, reducing signalling overhead and enabling efficient energy use.

NOTE 1:	The enhancements from this KI should be designed to be reusable across different device types (e.g., both 6G regular UEs and 6G IoT UEs).

NOTE 2:	Coordination with SA3 is required for any security and privacy related topics.

NOTE 3:	Coordination with RAN WG is required for any features related to RAN procedures.

**** End of Changes ****