SA WG2 Meeting #170	S2-2506636

25 – 29 Aug, 2025, Goteborg, SE

Source:	vivo

Title:	[General aspects, Energy efficiency] End-to-end energy saving and energy efficiency

Document for:	Agreement

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes to study the WT and KI for end-to-end energy saving and energy efficiency for 6G system.

Introduction

The WT#1 of the approved SA2 6G SID aims to define the overall 6G architecture as collection of capabilities and high level functionalities. It also includes the following description under WT#1:

In addition to the work tasks above, this study will identify other 5G features that will be supported and/or adapted in 6G.

In 5G, both UE energy saving (e.g. DRX, LP-WUS) and network energy saving (R19 EnergySys/ R20 FS_EnergySys_Ph2) have been defined. In 6G, the energy related topic is still very important for the sustainability of the whole system, so this paper proposes to support and adapt the energy saving and energy efficiency related 5G features in 6G under WT#1.x.

1. Justifications

1.1 Architectural Motivation

a) Motivation-1: requirements from external organizations and SA1 requirements:

- In the ITU-R Recommendation "Framework and overall objectives of the future development of IMT for 2030 and beyond", sustainability is considered as the design principles commonly applicable to all usage scenarios. Supporting end-to-end energy efficiency is an important design target for the sustainability of 6G system.

- In 3GPP SA1 TR 22.870 for 6G requirements, several use cases related with energy saving and energy efficiency of 6G system are proposed. At lease the following service and architecture requirements are included:

- Improving energy efficiency of 6G system compared with 5G system;

- Improving UE energy efficiency, including users’ preference for UE energy efficiency;

- Joint energy saving for the network and UE;

- Energy information exposure

Observation-1: The end-to-end energy efficiency including UE and network is an important architecture consideration and motivation of 6G system.

b) Observation-2: UE power saving mechanisms is important for the devices, including UE energy efficiency, UE energy saving services.

Motivation-2.1: UE energy efficiency

- In 6G system, more and more types of terminals, such as AR glasses, robots, AI enabled smart phones, will emerge and become popular. These terminals will integrate into our daily lives as an essential part. Certain immersive terminals, especially the wearable and light-weighted XR terminals such as AR glasses, are very sensitive to the power consumption due to the limitation of the battery capacity and weight. Furthermore, these light-weighted XR terminals also require to guarantee the QoS requirements for better service experience.

So it is critical and required for the 6G network to improve the terminals’ energy efficiency, i.e., guarantee the QoS requirements and reduce the devices’ communication power consumption.

Motivation-2.2: New business model for UE energy saving

- UE energy efficiency and power saving technologies are beneficial for the UE and subscribers. Considering the fact that these power saving technologies may require the radio resources consumption, if there is a new business model for the UE energy saving, it will be the driven for the operators to deploy the UE power saving technologies.

The UE energy saving service could be a new business model beneficial for both operators and users. Certain devices or VIP users may subscribe the UE energy saving service in order to improve the UE energy efficiency, i.e., guarantee the QoS requirements and reduce the communication power consumption for the UE.

Figure 1. New business model for UE power saving

c) Observation-3: End-to-end cooperation (including core network, RAN and UE) for network energy saving is beneficial for the design of the 6G network energy saving.

Motivation-3: End-to-end cooperation (including core network, RAN and UE) for network energy saving

- The UE related network energy information determination/prediction enables the energy based policy control for the UE or the exposure to the AF/UE.

Considering the fact that the energy consumption in the base station is the main part of the network energy consumption, the information reflecting the UE’s relative location within the cell from the UE or RAN would be helpful for the core network to improve the network energy information determination and prediction. Furthermore, the energy based policy control could consider more actions for the purpose of network energy saving, e.g. assist network selection.

- The interaction between the UE and network is needed for network energy saving. Some network energy saving actions may degrade the QoS and impact the service experience. The UE or subscriber may not prefer to degrade QoS at certain time or under specific UE conditions, e.g., in case of running important applications, but they have no means to communicate with the network in time to notify that they don’t want to degrade the QoS currently. Furthermore, the subscribers may be worried that they won’t be able to know when QoS is degraded due to network energy saving. So due to the fact that the UE or subscriber is unaware of the execution of network energy saving actions, they may not be willing to support network energy saving subscriptions. In general, the network energy saving needs to consider more interactions with the subscribers or UEs.

1.2 Gap Analysis

- Gaps on UE energy efficiency

  - UE energy efficiency is related with both the UE power consumption and QoS performance, e.g., data rate, latency. In 5G system, some UE power saving technologies, e.g., LP-WUS, C-DRX, etc., have been specified. However, the improvement of the UE energy efficiency needs to consider both the usage of the power saving technologies and the guarantee of QoS requirements.

In 6G era, considering more and more light-weighted XR devices, e.g., AR glasses, are integrated into our daily lives, the RAN needs to consider the both the UE’s QoS requirements and power saving technologies in order to meet the UE energy efficiency requirements for these devices or applications.

  - In 5G network, the main business model between the operators and subscribers is based on the data traffic transmission. Considering more and more communication technologies are applied into the 6G system, the exploration for the business model is also an important part for the operators to consider whether to deploy these technologies.

UE energy saving as service is a new business model in 6G system. This service is based on the deployment of the UE energy saving technologies. This new service not only helps the devices to reduce communication power consumption and guarantee the service experience, but also provides the high-value service scenario for the operators. This new business model has the following two advantages:

- The operators could provide the new value added service and attract more subscribers by deploying the energy saving technologies;

- The devices could get better service experience with guaranteed QoS and reduced communication power consumption.

- Gaps on end-to-end cooperation for network energy saving, e.g., R19 EnergySys/R20 FS_EnergySys_Ph2

  - The 5G R19 EnergySys has defined EIF for the energy related information collection and determination. However, the determination of the energy consumption is mainly based on data volume information. The radio energy consumption level related information with assistance from UE or RAN, e.g., the information reflecting the relative location of the UE within the cell, helps to improve the UE related energy consumption information determination and can be further considered in 6G system.

  - Network energy saving actions may impact the service experience. The current mechanisms in 5G R19 relies on energy saving indicator within UE subscription. So the network performs the network energy saving actions on the UEs which have the subscription on supporting network energy saving. However, this static indication in the subscription, lacking of UE/subscriber’s timely participation, has the following two issues:

    - It can’t reflect the subscribers/ UEs current status or preference on whether the network energy saving is preferred at current situation.

    - There is no notification to the subscribers/ UEs when the service experience is impacted due to network energy saving.

In 6G system, the UE/subscriber participation or assistance for network energy saving is needed, e.g., provision of UE preference, notification to UE.

1.3 Technical Impact

- Whether the new 6G energy entity or 5G EIF enhancements for energy related information determination/prediction/ exposure is needed?

- End-to-end support on the UE energy efficiency (i.e., reduce UE communication power consumption and without QoS degradation), including UE, RAN and core network.

- Support for UE energy saving service, e.g., subscription, potential interaction between CN and RAN, energy related policy control at PCF.

- End-to-end collaboration for network energy saving, e.g., UE participation/assistance, more information input from UE or RAN reflecting the radio energy consumption level information.

- The interworking between the 6G energy related features and 5G energy related feature may be needed. It may be further decided based on the clear view of 6G energy features.

**** First Change ****

Annex A.X. WT#X Scope on end-to-end energy saving and energy efficiency

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

This WT will study:

- How to support end-to-end collaboration for 6G energy saving and energy efficiency, e.g., UE energy efficiency, whether to support new 6G energy entity or 5G EIF enhancements, end-to-end collaboration for network energy saving.

**** Second Change ****

5.X. Key Issue #X: System architecture design to support end-to-end energy saving and energy efficiency

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This Key Issue will study:

- How to improve UE energy efficiency, e.g., UE power consumption reduction and QoS guarantee, UE energy saving service;

- How to support end-to-end collaboration for network energy saving, e.g., UE preference indication, enhancements on the UE related energy consumption determination/prediction with more information input compared with the data volume based calculation model in 5G;

- Whether and how to support interworking between 6G and 5G for energy features.

**** End of Changes ****