SA WG2 Meeting #170	S2-2506639

25 – 29 Aug, 2025, Goteborg, SE

Source:	vivo

Title:	[WT#1.2, User Plane Architecture] User Plane Architecture for 6GS

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper provides justifications for study on user plane architecture and propose KIs for study on user plane architecture.

1	Justifications

The following aspects should be considered when designing 6G user plane architecture:

1.1	General

The 6G user plane considers the following directions.

-	#1: QoS monitoring over UP

-	#2: PSA UPF selected for PDU Session mismatch with the PSA UPF with dedicated functionality for specific services

-	#3: GTP-U Limitation

-	#4: Small data packet transmission

1.2	QoS monitoring over UP

1.2.1	Architectural Motivation

In 6G network, an efficient end-to-end QoS monitoring and information exchange mechanism between the UE and UPF has the following benefits:

a)	Reflecting the actual QoS guarantee result and effects: the UE’s application client could make some adjustments beforehand to adapt to the network status according to the QoS monitoring result.

b)	Providing actual QoS evaluation data to the NW for AI model training: in this case, with the actual QoS monitoring data, the network could provide more suitable QoS policy decision for the UE according to the AI model inference.

c)	Improving end-user QoE experience: when the network transmission performance is getting worse, the network can identify the problem and apply mechanisms to improve the QoE experience according to the QoS monitoring result.

d)	Optimizing the terminal’s control efficiency: providing an accurate data support for real-time optimization of terminal service data flows, and improving the end-user service experience.

For example, the end-to-end QoS monitoring for RTT information between the UE and UPF is needed. This parameter denotes the latency information within the mobile network, so the UE may provide this information to the UE’s application client for the codec adjustment in the application layer or apply this information for AI model training and QoS prediction. Furthermore, the RTT information measurement needs to be real-time in order to be timely used in the UE. So an efficient measurement for this information is necessary to improve the end-user service experience.

Observation 1: An efficient end-to-end QoS monitoring and information exchange mechanism between the UE and UPF is needed.

1.2.2	Gap Analysis

The 5G network supports the QoS monitoring for the specific QoS parameters, e.g., round trip delay, congestion information, available bitrate, etc. The current QoS monitoring mainly focuses on the following two aspects:

a)	RAN information reporting to the UPF via GTP-U header extension, e.g., congestion information, available bitrate;

b)	For the end-to-end network performance, e.g., round trip delay, the measurement includes two parts: the measurement on N3/N9 interface and the measurement for radio interface, wherein:

1)	the measurement on N3/N9 interface can be supported via GTP-U header extension field(s).

2)	the measurement on radio interface is based on the RAN implementation.

Observation 2: QoS measurement between the UE and UPF is not efficiently supported in 5G network.

For example, the QoS monitoring for round trip delay defined in 5G system is divided into following two parts:

a)	Measurement of the delay between the RAN and PSA UPF (i.e., T[RAN<-> UPF]).

This measurement relies on the enhancements on the GTP-U header to include the time stamp information.

b)	Measurement of the delay in the radio interface between the UE and RAN (i.e. T[UE<-> RAN]).

T[UE<-> RAN] is determined by RAN and relies on the RAN implementation. Especially for the UM mode at RAN, it would be hard for the RAN to obtain the value T[UE<-> RAN]. It doesn’t support to include the time stamp information at radio interface.

Then the PSA UPF gets the round trip delay according to the two parts (i.e. Rond Trip Delay = T[ UE<-> RAN] + T[ RAN<-> UPF]). However, the measurement of the delay at radio interface depends on RAN implementation and brings the uncertainty for the measurement of round trip delay.

Figure 1.2-1: QoS monitoring in 5G network

If the QoS monitoring for certain parameters can be measured between UE and UPF directly, it would be more efficient and minimize the impact on the RAN.

1.2.3	Technical Impact

This aspect has the following impact:

-	UE and UPF interaction for QoS monitoring, including:

-	what parameters can be supported by QoS monitoring between UE and UPF (e.g., RTT measurement).

-	procedures and protocol to support UE and UPF interactions.

1.3	PSA UPF selected for PDU Session mismatch with the PSA UPF with dedicated functionality for specific service

1.3.1	Architectural Motivation

From Rel-15 to Rel-20, more and more dedicated functionalities are required for PSA UPF. Apart from basic user plane functionality (e.g. packet detection, routing, and forwarding), some enhanced functionalities are also introduced to UPF, e.g.

-	in XRM R18: PSA UPF supporting PDU Set based handling,

-	in XRM R19: PSA UPF supporting MoQ relay functionality,

-	in UPEAS R19: Handling of Payload Headers.

During PDU Session establishment procedure, there is no specific requirement from the UE, and a common PSA UPF is selected. When the UE activates the application later, e.g. sending FQDN to MoQ server, the common PSA UPF cannot serve the MoQ relay functionality. Figure 1.3-1 describes an example of PSA UPF selected for PDU Session mismatches with a dedicated UPF functionality.

Figure 1.3-1: Example of PSA UPF selected for PDU Session mismatches with a dedicated UPF functionality

For this case, it delayed the procedure to setup another PDU Session and select an appropriate UPF.

On the other hand, although 5GS allows a re-establishment of a PDU session upon receipt of new requirement from upper layer. However, the number of PDU sessions that a UE supports to maintain is very limited in real commercial scenario, and the establishment of PDU sessions leads to plenty of SM signallings. In the meantime, the time for establishing PDU sessions brings time delay and impacts the user experience.

Observation 3: PSA UPF selected for PDU Session mismatch with the PSA UPF with dedicated functionality for a specific service is a problem to be resolved.

1.3.2	Gap Analysis

In 5G, if EAS discovery with EASDF procedure is used, after SMF has received FQDN in DNS message report from EASDF, SMF determines the FQDN is for a specific service (e.g. MoQ) and instruct EASDF to return the address of specific PSA UPF (e.g. UPF supporting MoQ Relay). The returned UPF may be local UPF. That is, the network should hijack UE’s DNS query and use the DNS to determine whether a specific service is required by the UE. Figure 1.3-2 describes an example of SMF hijacks UE’s DNS query.

Figure 1.3-2: Example of SMF hijacks UE’s DNS query

Observation 4: The network hijacks UE’s DNS query and use the DNS to determine a specific UPF is required by the UE.

On the other hand, single PDU session with multiple PDU session anchors are supported in 5G. SMF can insert a ULCL UPF (based on UL Classifier functionality) or BP UPF (based on IPv6 multi-homing) as defined in 5.6.4 of 3GPP TS 23.501. The two mechanisms can be summarised as follows:

Table 1.3-1: Summary of ULCL and IPv6 Multi-homing

|  | ULCL | IPv6 Multi-homing |
| --- | --- | --- |
| UE impacted | No | UE shall support corresponding IETF specification, indicate SM capability, and select IPv6 prefix |
| IP address | Single IP address/IPv6 prefix | A PDU Session may be associated with multiple IPv6 prefixes |
| Supported PDU session type | All (IPv4/IPv6/IPv4v6/Ethernet） | IPv6 type only |
| mobility | temporary N9 forwarding tunnel between the source UL CL and target UL CL | Based on RFC 4191 |
| Others | Dependency on AF deployment and should use local DN | Optional support of local DN |

The two mechanisms have the following drawbacks:

a)	ULCL UPF has a dependency of AF deployment

The insertion of ULCL UPF is also controlled by PCC rule triggered by AF or local configuration. It has a strong dependency on whether the AF supports the Application Function influence on traffic routing, or an intelligent and appropriate local policy. Besides, ULCL has a deep bounding with MEC mechanism.

b)	BP UPF is limited to PDU session with IPv6 type

The insertion of BP UPF is controlled by the SMF based on UE and UPF capabilities. The UE shall indicate a support of IPv6 multi-homing capability to SMF for a PDU session and the multi-homing of a PDU session applies only for PDU sessions of IPv6 or IPv4v6 type. Besides, multi-homing of a PDU session relies on the support of IETF specifications (e.g. SMF notifies the UE of the new IP prefix of PSA UPF using IPv6 Router Advertisement message as defined in RFC 4861, SMF sends IPv6 multi-homed routing rule along with the IPv6 prefix to the UE using IPv6 Router Advertisement message as defined in RFC 4191, etc.)

Observation 5: UL Classifier and IPv6 multi-homing is not flexible to resolve the PSA UPF mismatch issue.

1.3.3	Technical Impact

This aspect has the following impact:

-	study on 6G user plane architecture, including:

-	potential architecture of 6G user plane, including N3 and N9 reference point.

-	how to dynamically insert PSA UPF while maintaining the continuity of IP address.

-	how to support single PDU session with multiple PDU session anchors in 6G (e.g. ULCL+, BP+).

1.4	GTP-U limitation

1.4.1	Architectural Motivation

When supporting the high throughput applications (e.g. XR games), the load of UPF is a critical issue. In 5G, the GTP-U is the user plane protocol over the N3 reference point between RAN and UPF. The GTP-U determines a tunnel between two endpoints (i.e. RAN and UPF). It causes a lot of signalling for SMF to relocate a UPF once a UPF suffers a heavy load.

On the other hand, when the UE has moved out of the service area of the UPF that was previously connecting to the RAN, the SMF may decide to insert an I-UPF to maintain the PSA UPF.

In above case, the change of endpoints leads to a complex signalling. This is because the GTP-U is a point-to-point transport protocol. Any change of endpoints leads to a complex tunnel management procedure.

On the other hand, the growth of IPv6 adoption as user packet data protocol has been observed. SRv6 is a combination of Segment Routing technique and IPv6 transport protocol. According to RFC 8986, SRv6 explicitly defines the forwarding path through the SID (Segment Identifier) list in the SRH (Segment Routing Header) extension header. It is observed that the SRv6 supports real-time adjustment of the UP path based on dynamic requirements (e.g. latency, bandwidth, UPF load, etc.).

Observation 6: SRv6 enables a dynamic UP path and real-time adjustment of the path.

1.4.2	Gap Analysis

In 5G, the GTP-U is the user plane protocol over the N3 reference point between RAN and UPF. The GTP-U determines a tunnel between two endpoints (i.e. RAN and UPF). Each tunnel is identified by a TEID and transfers the user plane traffic corresponding to a PDU session. The endpoint assigns the tunnel info used by another endpoint and establishes the context of the tunnel. The PSA UPF relocation leads to the change of N3/N9 GTP-U tunnel.

Change of endpoints often leads to a complex signalling. In 5G, SMF should select a target UPF and triggers the establishment of new N3 tunnel, and the source UPF then release the old N3 tunnel. The PDU session with source UPF should be released as well. Such N3 tunnel management causes time delay for transmission and impacts the user experience.

Observation 7: The change of GTP-U endpoints leads to a complex signalling management.

1.4.3	Technical Impact

This aspect has the following impact:

-	study on 6G user plane protocols, including:

-	candidate protocols for 6G user plane (e.g. GTP-U+, SRv6, QUIC, etc.)

-	how to support a dynamic N3/N9 traffic routing

-	how to minimize the impact due to endpoint change over N3 and N9 (e.g. change of PSA UPF, change of I-UPF)

NOTE: Study on 6G UP protocols needs to coordinate with CT4.

1.5	Small data packet transmission

1.5.1	Architectural Motivation

Small data use case 1: AI traffic

Generative AI is a key use case of artificial intelligence. It can generate new and creative content based on input user’s prompts, rather than simply analysing or categorizing data, by learning patterns and regularities from vast amounts of text, images, audio, etc.

Generally, to alleviate the user’s anxiety, Generative AI employs streaming response. The streaming response displays the response in a very fine granularity, e.g. word by word (similar to a typewriter). The streaming response makes the DL data traffic from application server has a very small size. For example, to send a sentence "How are you today", the sentence may be divided into 3 packets, wherein first packet contains only "How", and second packet contains only "are" and the final packet contains "you today".

With exponentially increased applications that support Generative AI, it is foreseen that 6G will face massive AI data packet transmission over UP.

Small data use case 2: Heartbeat packet

the UE needs to send heartbeat packet to keep the connection with application server. The heartbeat packet may be used to keep TCP connections alive, or keep application-level connections alive.

Small data use case 3: IoT

Sometimes, the data packets transmitted by an IoT device are even lower than the overhead of establishing and maintaining a PDU session. In 5G, before transmitting small data packet, the device needs to enter the connected state and activate a PDU session, which incurs overhead of signalling compared to the data packet itself. Furthermore, sometimes IoT device transmissions are best effort. After waking up to transmit data, the IoT device may be powered down immediately, removing all context related to the transmission.

Besides, for IoT scenario, it often faces the case that a large number of devices with low differentiation access the network. These IoT devices may have similar data transmission behaviour. For example, some IoT devices sometimes awake and sends small data packet, and same type IoT devices have similar traffic patten (e.g. transmitting picture for camera sensors). Managing these IoT device at the PDU session granularity on the N3 tunnel is not efficient.

Observation 6: In 6G, both eMBB and IoT scenario have a requirement on small data packet transmission.

1.5.2	Gap Analysis

In 5G, there are some related optimisation for small data packet transmission:

a)	user plane CIoT 5GS optimisation (UP-CIoT):

Summary: NG-RAN maintains the N3 tunnel endpoint information while a UE is in CM-IDLE with Suspend. The user plane CIoT 5GS optimisation requires that UE, RAN, and AMF negotiate the support of user plane CIoT 5GS optimisation. Specific suspend procedure and resume procedure are defined for a quick resume from IDLE to CONNECTED state. The data packet is hence sent via N3 tunnel.

Drawbacks: Dedicated suspend procedure and resume procedure is defined. At most 2 DRBs over the air.

b)	control plane CIoT 5GS optimisation (CP-CIoT):

Summary: The control plane CIoT 5GS optimisation is used to exchange user data between the UE and the SMF as payload of a NAS message in both uplink and downlink directions, avoiding the establishment of a user plane connection for the PDU Session. It requires that UE and AMF negotiate the support of control plane CIoT 5GS optimisation. The data packet was sent to AMF and forwarded to UPF by SMF.

Drawbacks: A frequent transmission of small data packet may lead to control plane signalling overload, especially frequent small packet for AI traffic.

c)	small data transmission (SDT):

Summary: The data packet is included in msg3. The SDT has no CN impact as in SDT it is assumed that the UE is in CM-CONNECTED state, and hence the N3 tunnel is always connected from RAN perspective.

Drawbacks: A very limited data size as the data packet is included in msg3. Besides, the UE will not always stay in RRC-INACTIVE. Besides, a frequent transmission of small data packet can easily lead to a congestion for resource scheduling in RAN, especially frequent small packet for AI traffic.

d)	control plane early data transmission (CP-EDT):

Summary: The data packet is included in msg3. For control plane EDT, the data packet was sent from RAN to AMF and forwarded to UPF by SMF. The UE does not need to send a NAS message.

Drawbacks: A very limited data size as the data packet is included in msg3. If the data volume exceeds the size supported by UP-EDT or the threshold configured by RAN, the transmission will fall back and increases the overall overhead and latency. Besides, a frequent transmission of small data packet may lead to control plane signalling overload, especially frequent small packet for AI traffic.

e)	user plane early data transmission (UP-EDT):

Summary: The data packet is included in msg3. The data packet was sent from RAN to UPF and the UL data was sent before the N3 tunnel is resumed. The UE does not need to send a NAS message. The UE is able to receive DL data during the resume of N3 connection.

Drawbacks: The UE shall stay in RRC-IDLE or RRC-INACTIVE. If the data volume exceeds the size supported by UP-EDT or the threshold configured by RAN, the transmission will fall back and increases the overall overhead and latency. Besides, a frequent transmission of small data packet may lead to congestion for resource scheduling in RAN, especially frequent small packet for AI traffic.

The existing 5G N3 tunnel do not adequately account for the characteristics of small data packet transmission.

1.5.3	Technical Impact

This aspect has the following impact:

-	study on small data packet transmission over user plane, including:

-	UL and DL user plane optimisation for small data packet transmission

-	how to minimize the N3/N9 management overload considering similar traffic patten of massive devices

2	Technical Impact Summary

This WT/KI has impact on N3 and N9 reference points, may have impact on N4 reference point.

Interaction between network functions/services is not in scope of this WT/KI.

Interworking or migration from 5G to 6G is not in scope of this WT/KI.

This WT/KI has the following impact:

-	Study on QoS monitoring over UP, including:

-	UE and UPF interaction for QoS monitoring, including:

-	what parameters can be supported by QoS monitoring between UE and UPF (e.g., RTT measurement).

-	procedure to support UE and UPF interactions.

-	study on 6G user plane architecture, including:

-	potential architecture of 6G user plane, including N3 and N9 reference point

-	how to dynamically insert PSA UPF while maintaining the continuity of IP address

-	how to support single PDU session with multiple PDU session anchors in 6G (e.g. ULCL+, BP+)

-	study on 6G user plane protocols, including:

-	candidate protocols for 6G user plane (e.g. GTP-U+, SRv6, QUIC, etc.)

-	how to support a dynamic N3/N9 traffic routing

-	how to minimize the impact due to endpoint change over N3 and N9 (e.g. change of PSA UPF, change of I-UPF)

NOTE: Study on 6G UP protocols needs to coordinate with CT4.

-	study on small data packet transmission over user plane, including:

-	UL and DL user plane optimisation for small data packet transmission

-	how to minimize the N3/N9 management overload considering similar traffic patten of massive devices

**** First Change (all text new)****

Annex A.X. WT#X Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

This WT for User Plane Architecture for 6GS includes the following aspects:

-	Study on QoS monitoring over UP, including:

-	UE and UPF interaction for QoS monitoring, including:

-	what parameters can be supported by QoS monitoring between UE and UPF (e.g., RTT measurement).

-	procedure to support UE and UPF interactions.

-	study on 6G user plane architecture, including:

-	potential architecture of 6G user plane, including N3 and N9 reference point

-	how to dynamically insert PSA UPF while maintaining the continuity of IP address

-	how to support single PDU session with multiple PDU session anchors in 6G (e.g. ULCL+, BP+)

-	study on 6G user plane protocols, including:

-	candidate protocols for 6G user plane (e.g. GTP-U+, SRv6, QUIC, etc.)

-	how to support a dynamic N3/N9 traffic routing

-	how to minimize the impact due to endpoint change over N3 and N9 (e.g. change of PSA UPF, change of I-UPF)

NOTE: Study on 6G UP protocols needs to coordinate with CT4.

-	study on small data packet transmission over user plane, including:

-	UL and DL user plane optimisation for small data packet transmission

-	how to minimize the N3/N9 management overload considering similar traffic patten of massive devices.

**** Second Change ****

5.X. Key Issue #X: User Plane Architecture for 6GS

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

The 6GC shall support an efficient and flexible design of user plane architecture. To simplify the implementation of both the UE and the network, this key issue includes the following aspects:

-	Study on QoS monitoring over UP, including:

-	UE and UPF interaction for QoS monitoring, including:

-	what parameters can be supported by QoS monitoring between UE and UPF (e.g., RTT measurement)

-	procedures and protocol to support UE and UPF interactions

-	study on 6G user plane architecture, including:

-	potential architecture of 6G user plane, including N3 and N9 reference point.

-	how to dynamically insert PSA UPF while maintaining the continuity of IP address.

-	how to support single PDU session with multiple PDU session anchors in 6G (e.g. ULCL plus, BP plus).

-	study on 6G user plane protocols, including:

-	candidate protocols for 6G user plane (e.g. GTP-U+, SRv6, QUIC, etc.)

-	how to support a dynamic N3/N9 traffic routing.

-	how to minimize the impact due to endpoint change over N3 and N9 (e.g. change of PSA UPF, change of I-UPF).

NOTE: Study on 6G UP protocols needs to coordinate with CT4.

-	study on small data packet transmission over user plane, including:

-	UL and DL user plane optimisation for small data packet transmission.

-	how to minimize the N3/N9 management overload considering similar traffic patten of massive devices.

**** End of Changes ****