SA WG2 Meeting #170	S2-2506640

25 – 29 Aug, 2025, Goteborg, SE

Source:	vivo

Title:	[WT#1.2, Network exposure] Support for network exposure to UE in 6G network

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes to study the WT and KI for network exposure to UE for 6G network.

Introduction:

The approved SA2 6G SID includes the following work task:

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

…..

1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

……

This paper aims to focus on network exposure to UE in 6G under WT#1.2.

1. Justifications

1.1 Architectural Motivation on network exposure to the UE

The network exposure enables the network to expose capability and information to the third party service providers. However, for the UE side, the network information is also needed from the following aspects:

- UE application client needs the network status information for the optimization of the UL data transmission, e.g., adjustment of the codec algorithms according to the network status information. The information exposed to the UE should be real-time. Since the network status may be changed frequently, so the UE application needs the latest network status information for the UL traffic optimization.

- Improve end-user QoE experience: When the network transmission performance is getting worse, the UE can identify that the problem is due to the communication network and provides the solution from the UE's perspective to improve the service experience

- The UE is a whole system including OS, modem, application clients, etc. The network information exposure to the UE could benefit other layers within the UE, not only the application clients. The benefits include the following aspects:

  - Improvement of the UE’s internal control efficiency;

  - Multi-layer optimization within the UE;

  - Support for real-time optimization of service flows at the UE.

- In 6G, the UE may be enabled with AI capabilities, e.g., QoS evaluation and QoS prediction capability. The information (e.g. per UE or per flow QoE, network resource status) exposed to the UE may help the UE to train the AI capability for QoS prediction. This AI capability enables the UE to get the information on whether the QoS requirements for certain applications can be satisfied in the near future.

  - For example, when the user is on the subway during the morning rush hour, the network is congested and the service experience is bad; however, when the user is at the office, the network status is good and service experience will be much better than on the subway. So the network status information for different time and location varies a lot and is beneficial for the AI model training for QoS prediction within the UE.

1.2 Gap Analysis on network exposure

In 5G network, network capability and information exposure to AF has been specified. However, it is not supported for the 5G network to expose the information to the UE directly. The UE may get the exposed information from the AS/AF via application layer; however, this aspect is not fully specified due to the fact that the application layer between the UE and AS/AF is out of 3GPP SA2 scope. The current mechanisms for the network information exposure to UE via AF/AS has the following issues:

- It relies on the deployment of AF at the application server side. And third party AFs are not commonly deployed in the real business.

The 3rd party application providers need to deploy AF for the interaction with the mobile network. Since the users of the application may belong to different operators, the 3rd party AF needs to interact with multiple operators to get certain network information and apply this information for its applications.

Furthermore, the development and deployment of AF in the 3rd party may require extra capabilities to understand and realize APIs defined in the 3GPP standards. Furthermore, different operators may support different APIs towards the AF. So this may not be an easy work for the 3rd party to support the AF.

Figure 1: Network exposure to the AF

- The information exposure to UE from AF/AS may introduce extra delay.

Currently, the network exposes the information to AF via NEF/PCF/UPF. Then the application server sends the information to the UE via application layer. To the delay sensitive applications, this information may not be useful if the information can’t be received by the UE in time.

- The information exposure to the UE via AF/AS can only be exposed to the UE’s App Client. However, as the motivation stated, the UE’s OS or modem may also need the information. If the UE’s App Client doesn’t expose the info to the UE’s OS or modem, these parts within the UE can’t get this information.

1.3 Technical Impact

- The exposure architecture, including whether the new 6G entities or 5G NEF enhancements are needed for the management of the network exposure to the UE?

- Network information exposure from the core network to the UE (e.g., via control plane, or via user plane);

- Network information request from the UE and the authentication of the UE request for network information exposure;

- It may not be needed for the interworking or migration from 5G to 6G.

For the Overlap with other WTs, e.g., data plane, sensing:

- Other WTs, e.g., data plane, sensing, may also consider the information exposure. It is proposed to solve the information exposure issue, if any, in the specific features first, and then, consider how to merging the related conclusion, if any, into the network exposure framework.

**** First Change ****

Annex A.X. WT#X Scope on network exposure for 6G

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

This WT will study:

- How to support efficient network exposure to both the AF and UE in 6G.

**** Second Change ****

5.X. Key Issue #X: System architecture design to support network exposure for 6G

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This Key Issue will study:

- How to support the network information exposure to the UE;

  - How to support the UE request for network information and how to authorize the UE request.

  - What information can be exposed to the UE, e.g., latency, congestion information, available bitrate, etc.;

  - How to expose the information to the UE, e.g., via user plane, via control plane.

**** End of Changes ****