SA WG2 Meeting #170	S2-2506641

25 – 29 Aug, 2025, Goteborg, SE

Source:	vivo

Title:	[WT#1.2, Network slicing] Simplified Network Slicing

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper focus on promoting network slice in 6G that how to efficiently, simply and with low complexity implement the benefits of network slice.

1. Justifications

1.1 Architectural Motivation& Gap Analysis on simplifying the network slicing:

Network Slice is one of the core innovations in 5G communication technology. From the technical perspective of network slice, it can effectively satisfy the diverse business requirements, achieve efficient sharing and isolation of network resources, and accelerate the deployment and innovation of new services. The specific advantages can be reflected by the following aspects:

-	Precise guarantee of quality of service (QoS);

-	Optimized resource utilization rate and cost;

-	Flexibility and scalability;

-	Conducive to business model innovation;

-	Enhanced security and reliability;

However, after multiple release of study and evolution as the following table, network slicing has become extremely bloated, greatly increasing the processing logic and complexity of networks and UE.

| Release | Mainly study contents | Introduced NSSAI |
| --- | --- | --- |
| R15-NS (network slice) | Basic procedure and usage (e.g., request, allow, reject slice during registration procedure) | Requested NSSAI, Allowed NSSAI, rejected NSSAI, configured NSSAI… |
| R16-eNS | - network slicing interworking support from EPC to 5GC； - NSSAA | Pending NSSAI |
| R17-eNS_Ph2 , NRslice | - network slice access control - enhancement of RAN slicing for NR - network slice AS group information. (RAN alignments) - UE is only allowed to be registered with compatible Network Slices (GSMA alignments) | NSAG information, NSSRG information |
| R18-eNS_Ph3 | - Alternative NSSAI； - Slice-based SoR mechanism；(i.e., slice-based network selection, cancelled) - Partial NSSAI； - network slice validity； - Network controlled slice behavior | Replaced NSSAI, partially allowed NSSAI, partially rejected NSSAI, on-demand NSSAI |

On the other hand, in business, due to the insufficient support of UE for URSP, network slice has not been deployed and commercialized on a large scale.

Therefore, how to efficiently, simply and with low complexity implement the benefits of network slice requires 6G research.

1.2. Technical Impact

To efficiently, simply and with low complexity implement the benefits of network slice in 6G for UE, there may be the following directions:

- Direction1: The UE is not aware of the slice, and the processing logic of the slice is executed by the network side.

- Direction2: The UE is aware of the slice, but some processing logics of the slice in UE is simplified, e.g., the RSD validation from slice perspective is implemented by PCF, and ensure the RSD is validate to UE and avoid the UE implementing it.

For the Direction1, since UE is not aware of the network slice, then the following issues needs to be considered:

-	For a given service, how the network side accurately determine the slices that to be applied;

-	For a given service, how the network decides to establish PDU session and how to establish PDU session.

For the Direction2, since the RSD validation from slice perspective is implemented by CN (i.e. PCF), then the RSD validation by UE based on the Allowed NSSAI is not needed, thus the following aspects may be impacted:

-	How the network ensures the RSD is valid from the network slice perspective to reduce the processing logic for UE;

-	If the network ensures the RSD is valid, whether Allowed NSSAI needs to be configured to UE, then whether and how to determine the Requested NSSAI.

**** First Change ****

Annex A.X. WT#1.2 Scope of Network slicing

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT1.2- Study 6G Network slicing, in order to efficiently, simply and with low complexity implement the network slice in UE side.

**** Second Change ****

5.X. Key Issue #X: Simplify the Network Slicing

-	Study how to simplify the network slicing in UE side:

-	Option#1: study how to support that UE is not aware of the Network slicing, including:

-	Study how to determine the network slices to be used for a given service;

-	Study how to establish PDU Session for a given service;

-	Option#2: study how to simplify the Network slicing handling logic in UE side, including.

-	study how the network ensures the RSD is validate from the network slice aspect to UE.

-	study whether and how to determine the Requested NSSAI.

Editor's Note: it is assumed only one option is considered in the normative phase, it is required evaluate the above options during the conclusion phase and make final decision.

**** End of Changes ****