SA WG2 Meeting #170	S2-2506642

25 – 29 Aug, 2025, Goteborg, SE

Source:	vivo

Title:	[WT#2, migration and interworking] migration and interworking

Document for:	Agreement

Agenda Item:	20.6.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: this contribution introduces the key issues for migration and interworking.

1. Justifications

1.1 Architectural Motivation:

Migration: SA and NSA

In 5GS, in the view of CN, 2 options are specified to support migration from 4G to 5G:

Option 1: Standalone Architecture (SA)

In this option, the 5G Core(5GC) is deployed, and the 5G UE can access to 5GC via NG RAN,

The 5G UE supports 5G NAS and can use the services provided by 5GC.

Option 2: Non-Standalone Architecture (NSA)

In this option, the 5G Core is not deployed, the UE supports only EPC based Dual Connectivity with secondary RAT NR.

The UE doesn’t support 5G NAS and cannot use the services provided by 5GC.

Figure 1: Option 1                                           Figure 2: Option 2

In the perspective of 6G, the above SA and NSA options are also taken into account for migration. The

6GS targets to support both communication evolution and beyond communication services.

- The communication evolution includes WT#1.1 CP enhancement and WT#4 data framework and possible WT1.2, etc.

- The new beyond communication services include WT#2 AI, WT#5 Computing, etc.

It can be foreseen the communication evolution probably brings in new NAS between 6G UE and CN and new interface within the CN. Those new features require standalone architecture.

In order to support new beyond communication services, the standalone architecture is more suitable, which has no legacy limitation and easily reach native design (AI native design, data native design).

if NSA method is selected, the UE cannot use the new 6G services (e.g., AI, data framework, computing), because 5GC does not support such 6G services.

Also, in 5GS deployments, most non-standalone architecture is only used in a short period and quickly move to standalone architecture. The non-standalone architecture has consumed substantial investment, yet it features a short lifecycle and low cost-effectiveness. From the perspective of the UE, the UE needs to connect to both NG RAN and E-UTRAN simultaneously, which leads to high terminal power consumption and short standby time, thus affecting the user experience.

Proposal 1: it suggests to select Standalone Architecture (SA) to perform migration from 5GS to 6GS.

Interworking:

The interworking with legacy system is specific in clause 5.2 of 6G SA1 TR 22.870, as following:

5.2 Interworking with legacy systems Subject to operator’s policy, the 6G system shall support mobility between the core network of the 6G system and a 5G core network with minimum impact to the user experience (e.g. QoS, QoE). Editor’s note: other inter-system requirements (e.g. between 6G and 4G), or exclusions, are FFS.

Proposal 2: Interworking with 5GS should be studied, whether to interworking with EPS is FFS.

In the SA2 SID, the WT#2 is descripted as following:

WT#2: Study migration and interworking, including

-	How to support migration to 6GS

-	How to support interworking with 5GS

-	Whether and how to support interworking with EPS

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

To support WT#2, the following scenarios for interworking need to be considered:

- Scenario-1:	From 6GC to 5GC.

- Optionally, the UE can further transfer from 5GC to EPC after transferring from 6GC to 5GC.

- Scenario-2:	From 5GC to 6GC.

- Scenario-3:	From 6GS to 4G/5G NTN/satellite access that use EPS/5GS.

- Scenario-4:	From 4G/5G NTN/satellite access that use EPS/5GS to 6GS.

Proposal 3: the scenario of interworking between 6GS and 5GS should be studied, and if the UE transfers from 6GS to 5GC, the UE can further transfer to EPC.

Proposal 4: the scenario of interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS should be studied.

1.2 Technical Impact:

In 5GS, architecture for interworking between 5GS and EPC/E-UTRAN is specified as figure 3:

Figure 3: Non-roaming architecture for interworking between 5GS and EPC/E-UTRAN

The key features of interworking between 5GS and EPC/E-UTRAN as blow:

- A N26 interface between 5G AMF and MME is specified to transfer UE context during interworking.

- The combined NF, i.e. PGW-C+SMF and UPF+PGW-U, dedicated for interworking between 5GS and EPC are specified to guarantee IP continuity during interworking.

- Multiple 5G QoS flows can be mapped to one EPS bearer, and the EPS bearer ID(EBI) for the QoS flow is allocated by 5G AMF when the QoS flow is created.

For interworking between 6GS and 5GS, the same key features may also need to be considered:

- Whether and how to specified a N26 like interface between 5GC and 6GC

To support interworking between 6G and 5G, the UE contexts may also need to be transferred between 6G AMF and 5G AMF, so the interface is needed.

But whether the interface is a N26 like interface or service-based interface needs to be studied considering both 5GS and 6GS support SBA.

Proposal 5: Study whether the interface between 6G AMF and 5G AMF is needed, and if needed define interface is a 5G N26 like interface or service-based interface..

- Whether and how to introduce combined NF

For User Plane (UP), to support IP continuity from 6GS to 5GS for UE, and considering the UE may need to further transfer from 5GS to EPS, the 6G UPF may need to combined with 5G UPF+PGW-U i.e., the combined 6G UPF+5G UPF+PGW-U needs to be introduced.

For Control Plane(CP), two options exist：

- Option 1: introduce a combined NF: 6G SMF+5G SMF+PGW-C

- Option 2: separate 6G SMF and 5G SMF+PGW-C,

both 6G SMF and 5G SMF+PGW-C can control 6G UPF+5G UPF+PGW-U

Option 1 may lead to the combined CP NF complex and not easy to support new 6G features.

Option 2 may need to specify the interface between 6G SMF and 5G SMF+PGW-C, but much easier to support new 6G features.

Proposal 6: Whether the 6G SMF needs also to be combined into 5G SMF+PGW-C needs to be studied.

- What is the mapping relationship between 6G QoS flow and 5G QoS flow

6GS QoS flow is depending to the 6GS QoS framework. It is unclear whether the scenario of multiple 6G QoS flows mapped to one 5G QoS flow exist or not. But it needs to study how the 6G QoS flow mapped to 5G QoS flow

Whether multiple 6G QoS flows are mapped to one 5G QoS flow or one 6G QoS flow is mapped to multiple 5G QoS flows need to be studied.
Proposal 7: study how the 6G QoS flow mapped to 5G QoS flow (e.g. one to one or multiple to one) and vice versa.

In addition to interworking for communication, there is also interwork requirement for new services, e.g. sensing. However, the basic logic design for sensing is prioritized, the interworking requirement is in low priority in the study phase or should be considered together in sensing study.

Proposal 8: the interworking of new services, e.g., sensing, should be considered but with low priority.

**** First Change ****

5.X. Key Issue #X: Interworking and Migration

To support migration and interworking, this key issue studies the following issues:

- System enhancements to support migration to 6GS, including making decision between NSA and SA.

- Architecture enhancements to support interworking with 5GS

- Study whether the interface between 6G AMF and 5G AMF is needed, and if needed define interface is a 5G N26 like interface or service-based interface.

- Study whether combined NFs are needed, such as whether to combine 6G SMF with 5G SMF+PGW-C, whether to combine 6G UPF with 5G UPF+PGW-U.

-    Study when the UE transfers from 6GS to 5GS, whether and how the UE can further transfer to EPS.

- Study the relationship between 6G QoS flow and 5G QoS flow and how the 6G QoS flow can be mapped to 5G QoS flow.

- Study whether and how to support interworking between 6GS and EPS

- Study how to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

- The interworking of new services, e.g., sensing, should be considered but with low priority.

**** End of Changes ****